SA WG2 Meeting #S2-155	S2-2303802

# Athens, Greece, Feb 20 – 24, 2023                                              revision of S2-2301473

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3792 | rev | 6 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF support of 5GS Packet Delay Variation monitoring based on QoS monitoring mechanism and exposed to AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Tencent, Samsung, Nokia Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the conclusion of 8.7 in TR 23.700-60 for Key Issue #7 policy enhancements to minimize jitter has been concluded, 5GS should support to monitor the 5GS jitter value i.e. Packet Delay Variation based on AF request with utilizing the QoS monitoring method. Revision 5: Measurements of PDV alone are not meaningful. The AF would also need the actual packet delay measurements and take actions |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GS support to monitor the 5GS Packet Delay Variation value based on AF request with utilizing the QoS monitoring method. Revision 5: The AF shall request PDV measurement together with packet delay measurements, i.e. using the same set of QoS monitoring requirements (measurement period, measurement type) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF cannot get the Packet Delay Variation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

5.37.X		 5GS Packet Delay Variation monitoring and reporting

5GS Packet Delay Variation means the variation of packet delay measurements between UE and PSA UPF. AF may send the requirement for Packet Delay Variation monitoring to 5GS together with the requirement for packet delay measurement, as described in TS 23.503[12] clause 6.1.3.xx. Based on the QoS monitoring mechanism described in clause 5.33.3, the 5GS Packet Delay Variation is derived by PCF and then reported to AF.

Upon AF request for Packet Delay Variation monitoring, the PCF triggers the QoS monitoring procedure, and gets the UL, DL or RT QoS Monitoring result from SMF. PCF derives the 5GS Packet Delay Variation based on QoS Monitoring results and then reports to the AF/NEF both, packet delay measurements and Packet Delay Variation.

NOTE: Derivation of 5GS Packet Delay Variation by PCF, based on QoS Monitoring, is implementation dependent. The Packet Delay Variation calculation method needs to be the same within the PLMN, based on operator policy.

Editor’s Note: Whether all QoS monitoring needs can be addressed with the limitation of one QoS Monitoring control information per PCC rule is FFS.

<<< End of changes >>>