3GPP TSG-SA WG2 Meeting #155	S2-2303563

Athens, Greece, Feb 20 – 24, 2023

Title:	Reply LS on the data and analytics exchange between two NWDAFs in different PLMNs

Response to:	S2-2302179 (S3-223020)

Release:	Rel-18

Work Item:	FS_eNA_Ph3, eNA_Ph3

Source:	SA2

To:	SA3

Cc:	GSMA

Contact Person:

Name:	Xiaoyan Duan

E-mail Address:	duanxiaoyan@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks SA3 for their Reply LS S3-223020 on data and analytics exchange between two NWDAFs in different PLMNs.

SA2 has discussed the questions from SA3 and would like to provide the following answers:

Question 1: SA3 is interested in knowing what type of UE-specific data and/or analytics is foreseen to be exchanged between VPLMN and HPLMN in the context of eNA.

Answer to Question 1:

According to the conclusions in clause 8.3 of TR 23.700-81, the following analytics and input data for these analytics can be exchanged between VPLMN and HPLMN (for the purpose of network slice selection, NF selection, QoS control, URSP provisioning, etc.): Slice load level analytics, NF load analytics and Service experience analytics.

Among these analytics identified for exchange in roaming case so far:

- The Slice load level analytics is not UE-specific and does not use UE-specific data as input for analytics, see clause 6.3.2A of TS 23.288.

- The NF load analytics is not UE-specific, and may or may not use UE-specific data (i.e. UE speed, UE Orientation, Per UE attribute (Destination, Route, Average Speed, Time of arrival)) as input for analytics, see clause 6.5.2 of TS 23.288.

- The Service experience analytics can be non UE-specific (i.e. Service Experience for a Network Slice or an application) or UE-specific (i.e. Service Experience for a UE), and may or may not use UE-specific data (i.e. QoE metrics, UE location, Performance Data) as input for analytics, see clause 6.4.2 of TS 23.288.

SA2 would also like to inform SA3 that SA2 agrees to send a LS on UE-specific data and/or analytics exchanges between VPLMN and HPLMN. Based on GSMA feedback, whether and what type of UE-specific data and/or analytics exchanges between VPLMN and HPLMN will be finally decided.

Question 2: SA3 is also interested in understanding whether the UE-specific data and/or analytics to be exchanged is essentially required to provide the service.

Answer to Question 2:

SA2 is waiting for the feedback from GSMA on the exchange of UE-specific data/analytics between HPLMN and VPLMN to finally decide the need of UE-specific data and/or analytics to be exchanged between VPLMN and HPLMN, See the LS on UE specific data and analytics exchanges between HPLMN and VPLMN.

From SA2 point of view, the UE-specific data and/or analytics to be exchanged between HPLMN and VPLMN are not essential for the delivery of services to the end users; but depending on operator policy, the UE-specific data and/or analytics may be used by the NFs to optimize service provisioning.

2. Actions:

To SA3:

ACTION: SA2 kindly asks SA3 to take the above information into account.

3. Date of Next SA2 Meetings:

SA2#156e	17th - 21st April 2023 		E-Meeting

SA2#157	22nd – 26th May 2023 		Berlin, DE