3GPP TSG SA WG2 Meeting #157	S2-2306375

# Berlin, Germany, May 22 - 26, 2023 											(revision of S2-2306054)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1278 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on IMS Data Channel Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR handles the following Editors Notes: Editor’s Note: The exact nature of the Data Channel Service profile, and how it is used is FFS. Editor's note: It is FFS whether the Data Channel service profile is downloaded to the S-CSCF during initial IMS Registration or from the IMS AS via N71/Sh Changes in this revision: This new revision of the CR is correct implementation of the approved CR at the last meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indicate that the Data channel services extend the current service profile, and is conveyed to the S-CSCF and IMS AS during third party Registration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# AC.3	IMS Data Channel Service Subscription

Service Subscription for IMS Data Channel shall be an extension to the MMTEL service profile in HSS. The Data Channel subscription information shall be used by the IMS AS to authorize subscribers to use the DC service.

IMS AS Service specific data is enhanced with DC specific service data, optionally stored in HSS (e.g as repository data) and retrieved by IMS AS using N71 interface.

DCSF service specific data used by DCSF for data channel management, may be stored and retrieved from  HSS using the N72 interface. DCSF service specific data are out of scope of 3GPP.

**** End of Changes ****