3GPP TSG-SA WG2 Meeting #157	S2-2306384

# Berlin, Germany, 22 – 26 May, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4568 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving Editor’s Note for one PIN serving multiple PDU Sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is one EN related to the support of multiple PDU Sessions in PEGC for a PIN. Editor's note: One PIN served by more than one PDU sessions in PEGC is FFS. The restriction on limiting one PIN serving only one PDU Session in one PEGC is impractical use case for PIN. The existing mechanism can be enhanced to enable the support of multiple PDU Sessions in PEGC for a PIN. The related analysis and proposals can refer to S2-2306383. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EN is removed by introducing the following: (option 1 in S2-2306383): - Specify that one PDU Session is associated to one 5G VN Group identified by External Group ID for a specific PIN service in a PIN. - Existing mechanism for 5G VN Group can be enhanced for PIN by enabling multiple 5G VN Groups for corresponding PIN services in a PIN. (option 2 in S2-2306383): - Specify that one PDU Session is associated to one 5G PIN Service Group identified by PIN Service Group ID for a specific PIN service in a PIN. - Existing mechanism for 5G VN Group can be associated to a PIN Group and extended for PIN by enabling multiple 5G PIN Service Groups for corresponding PIN services in a PIN (as a 5G VN Group). Option 1 or Option 2 is up to discussion in the meeting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PIN is incomplete which results in inpractical restritictions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of changes*******

#### 5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC.

One PIN may be served by more than one PDU sessions in the PEGC. (Option 1) One PDU Session is associated to one 5G VN Group identified by External Group ID for a specific PIN service in a PIN. One PIN may contain multiple 5G VN Groups for corresponding PIN services. (Option 2) One PDU Session is associated to one 5G PIN Service Group identified by PIN Service Group ID for a specific PIN service in a PIN. One PIN may contain multiple 5G PIN Service Groups for corresponding PIN services.

Editor's note:	How and whether to handle the case where PINs share a PDU session and local switching is FFS.

Editor's note:	The handling of the PEMC in 5GC in relation with PIN is FFS.

*******End of changes*******