3GPP TSG-SA WG2 Meeting #157	S2-2306664

# May 22 – 26, 2023, Berlin, Germany										 (revision of S2-2304191)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4233 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Paging Early Indication with emergency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, [T-Mobile USA, AT&T,] Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | LS R2-2302302 indicates that gNB will apply PEI (Paging Early Indication) even when UE has emergency PDU session if UE-ID based grouping function is used, since RAN does not know if UE has emergency PDU session in case of CN triggered Paging for UE in CM-IDLE Mode. For the RAN triggered Paging case for UE in RRC_INACTIVE state, RAN realizes the emergency PDU session based ARP value and restrict the use of PEI. NOTE: Current TS 24.501 clause 5.3.25 already specifies that negotiated PEIPS information shall not be used both at UE and AMF side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the AMF handling related to PEI when there is emergency PDU session for the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Paging for UE with emergency PDU session is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1, 5.4.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START of 1st CHANGE ***************

### 5.4.3	Paging strategy handling

#### 5.4.3.1	General

Alternative 1:

Based on operator configuration, the 5GS supports the AMF and NG-RAN to apply different paging strategies for different types of traffic.

In the case of UE in CM-IDLE state, the AMF performs paging and determines the paging strategy based on e.g. local configuration, whether the UE has an emergency PDU session, what NF triggered the paging and information available in the request that triggered the paging. If NWDAF is deployed, the AMF may also use analytics (i.e. statistics or predictions) on the UE's mobility as provided by NWDAF (see TS 23.288 [86]).

When the UE has an emergency PDU Session, the AMF should page promptly in all gNBs of the UE’s registration area for Network Triggered Service Request. In the Paging Message sent to NG-RAN, the AMF shall include an indication that the UE has an emergency PDU Session.

In the case of UE in CM-CONNECTED with RRC_INACTIVE state, the NG-RAN performs paging and determines the paging strategy based on e.g. local configuration, and information received from AMF as described in clause 5.4.6.3 and SMF as described in clause 5.4.3.2.

In the case of Network Triggered Service Request from SMF, the SMF determines the 5QI and ARP based on the downlink packet (if the SMF performs buffering) or the Downlink Data Report received from UPF (if the UPF performs buffering). The SMF includes the 5QI and ARP corresponding to the QoS Flow of the received downlink PDU in the request sent to the AMF. If the UE is in CM IDLE, the AMF uses e.g. the 5QI and ARP to derive different paging strategies as described in clause 4.2.3.3 of TS 23.502 [3].

NOTE:	The 5QI is used by AMF to determine suitable paging strategies.

****************** START of 2nd CHANGE ***************

#### 5.4.12.2	Core Network Assistance for PEIPS

To support the Paging Early Indication with Paging Subgrouping (PEIPS), Paging Subgrouping Support Indication and the PEIPS Assistance Information is used by the AMF and NG-RAN to help determine whether PEIPS applies to the UE and which paging subgroup used when paging the UE (see TS 38.300 [27]).

In the Registration Request message, the Paging Subgrouping Support Indication indicates whether the UE supports PEIPS with AMF PEIPS Assistance Information. If the UE includes Paging Subgrouping Support Indication, the UE may also include the paging probability information to assist the AMF. If the AMF supports PEIPS assistance and if the UE provided Paging Subgrouping Support Indication, the AMF stores the indication in the UE context in AMF. The AMF may use local configuration, the UE's paging probability information if provided, information provided by the RAN (e.g. any of the Information On Recommended Cells And RAN nodes For Paging), and/or previous statistical information for the UE to determine the AMF PEIPS Assistance Information. The AMF PEIPS Assistance Information includes the Paging Subgroup ID.

NOTE 1:	To minimise MT voice call setup latency, the AMF could allocate Paging Subgroup IDs taking into account whether or not the UE is likely to receive IMS voice over PS session calls.

NOTE 2:	To avoid MT traffic for more mobile UEs causing more stationary UEs to be woken up, the AMF could allocate Paging Subgroup IDs taking into account the UE's mobility pattern.

If the AMF has determined AMF PEIPS Assistance Information for the UE, the AMF stores it in the UE context in AMF and provides it to the UE in every Registration Accept message.

If the AMF has determined AMF PEIPS Assistance Information, the AMF shall provide it to NG RAN when paging the UE. In addition, in order to support PEIPS for UEs in RRC_INACTIVE mode, the AMF shall provide the AMF PEIPS Assistance Information to NG-RAN as part of the RRC Inactive Assistance Information.

The NG-RAN chooses on a per-cell basis whether to use PEIPS and which paging subgrouping mechanism to use. When using AMF allocated subgroups, both the UE and NG-RAN use the AMF PEIPS Assistance Information to determine the paging subgroup to apply as defined in TS 38.300 [27].

The AMF may use the UE Configuration Update procedure (as described in clause 4.2.4 of TS 23.502 [3]) and N2 UE Context Modification procedure (as described in clause 8.3.4 of TS 38.413 [34]) to update the AMF PEIPS Assistance Information in the UE and NG-RAN.

When the UE has an active emergency PDU Session:

Alternative 1:

-	The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message. The AMF shall not signal AMF PEIPS Assistance Information in the Registration Accept message.

-	As specified in TS 23.501 [47] clause 5.3.25, the UE and AMF shall not use the negotiated PEIPS Assistance Information.

-	For CN triggered Paging Request in CM-IDLE state, the AMF shall not signal PEIPS Assistance Information to NG-RAN and shall provide an indication that UE has emergency PDU Session as specified in clause 5.4.31 to prohibited use of PEI if paging subgrouping based on the UE’s temporary ID is deployed in the network.

-	For NG-RAN triggered Paging Request in RRC_INACTIVE state, the NG-RAN does not apply the PEI function when there is emergency PDU Session in the UE context in NG-RAN.

Alternative 2:

-	The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message. The AMF shall not signal PEIPS Assistance Information in Registration Accept message.

-	If there is negotiated PEIPS Assistance Information, the UE and AMF shall not use the negotiated PEIPS Assistance Information as specified in TS 24.501 [47] clause 5.3.25.

****************** END CHANGES ***************