3GPP TSG-SA WG2 Meeting #157	S2-2306852

# May 22 – 26, 2023, Berlin, Germany										 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4626 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolution related to MBSR configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the following answers in the SA3 LS (S2-2303971/S3-231400) and the SA5 LS (S2-2306325/S5-233546), the following EN can be removed. SA3 assumes offline methods are sufficient, as there is no security issue identified. SA5 assumes offline methods are sufficient, as the current DU configuration can be used to implement the MBSR configuration. Editor's note: How the connection to OAM server, including in the roaming case, is established securely is to be decided by interaction with SA WG3 and SA WG5 In addition, the following text and the EN described in §5.35A.4 are about MBSR configuration rather than MBSR authorization and the similar text exists in §5.35A.2. So, these can be removed from §5.35A.4. The MBSR(IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN for the MBSR operation. Editor's note: Configuration mechanism will be further described based on development of clause 5.35A.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.35A.2 - Remove the EN. §5.35A.4 - Remove the text and the related EN about MBSR configuration mentioned in the Reason for change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved EN exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.2, 5.35A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.35A.2		Configuration of the MBSR

In order for an MBSR to operate as a mobile IAB node, it receives configuration from the OAM system of the serving PLMN as specified in TS 38.401 [42]. The MBSR (IAB-UE) establishes a secure and trusted connection to the OAM server only if it is authorized to operate as MBSR in the serving PLMN.

In addition, the MBSR (IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMN lists by the HPLMN to perform PLMN selection as specified in TS 23.122 [17].

When a PDU session is used for the MBSR to access the OAM server, the MBSR establishes a dedicated PDU session for the OAM traffic. The serving PLMN provides an Allowed NSSAI and the MBSR establishes the PDU session for the OAM server access, considering the S-NSSAI and DNN requested by MBSR and/or the default values in subscription data. The MBSR can be (pre-)configured with UE policy or provisioned using existing UE Policy mechanism as defined in TS 23.503 [45] including the OAM access PDU session parameters for the authorized PLMNs.

* * * * Start of Next Change * * * *

5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods.

Editor's note:	The subscription information for the MBSR will be further specified.

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place, and the 5GC can make use of it for authorization of MBSR in VPLMN. MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into VPLMN to provide service.

.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to NG-RAN. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation.

Editor's note:	It is FFS whether the MBSR indication is a new IE or part of UE 5G MM capabilities.

Editor's note:	Whether existing IAB-Operation allowed indication and IAB authorized indication can be reused for MBSR will be determined.

NOTE 1:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.1.

The AMF of the MBSR can indicate to the MBSR IAB-UE that it is not allowed to act as an MBSR IAB node as part of registration procedure, and in this case the AMF does not include MBSR authorization indication to donor-gNB. The AMF may provide the indication either in a Registration Accept (if the PLMN allows the MBSR IAB-UE to be registered in the PLMM) or in a Registration Reject (if the PLMN does not allow the MBSR IAB-UE to be registered in the PLMN).

Editor's note:	In what conditions and how (e.g. UCU or deregistration with re-registration) an MBSR is informed when there is a change in the MBSR authorization information is FFS.

NOTE 2:	The mechanism applies to both roaming and non-roaming MBSR operations.

* * * * End of Changes * * * *