3GPP TSG-SA WG2 Meeting #157 	S2-2306887

MAY 22 – 26, 2023, Berlin, Germany	(was)

Source:	vivo

Title:	TS 23.586: Updates to QoS handling

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to update the QoS handling.

1. Introduction/Discussion

This paper tries to resolve the following ENs in the discovery:

| #1 | 4.3.2/4.3.3 | Editor's note: It is FFS what the Ranging/SL positioning QoS parameters are, whether the Ranging/SL positioning QoS parameters are needed and how to use the Ranging/SL positioning QoS parameters. |
| --- | --- | --- |

For the above EN#1, it has agreed that the RSPP is transported over the PC5-U and the RSPP transport QoS is also added to the NG-RAN for resource management. For the Ranging/SL Positioning QoS, it still needs the RAN feedback. This aspect FFS can be still kept.

In addition, if the application layer has provided the Ranging/SL Positioning QoS requirement, then the Ranging/SL Positioning layer maps the Ranging/SL Positioning QoS requirement to the Ranging/SL Positioning QoS parameters and provides the Ranging/SL Positioning QoS parameters to the AS layer, instead of using the configuration QoS parameters.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

4.3.2	PCF

In addition to the functions defined in TS 23.501 [2] and TS 23.503 [4], the PCF includes functions to provision the UE with necessary policies and parameters to use Ranging/SL Positioning services, as part of the UE Ranging/SL Positioning Policy information as defined in TS 23.503 [4] clause 4.2.2:

-	Authorization policy and parameters for Ranging/SL Positioning over PC5;

-	Authorization policy and parameters for Network assisted SL Positioning;

-	Authorization policy and Ranging/SL Positioning service exposure.

The PCF may update the Ranging/SL Positioning to the UE under certain conditions.

When receiving the Ranging/SL Positioning in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters used by NG-RAN for RSPP transport over PC5 corresponding to a UE as defined in clause 5.4.2 of TS 23.287 [6].

Editor's note:	Whether Ranging/SL positioning QoS parameters as defined in clause 5.7.2 are also provided to NG-RAN is FFS, and will be decided by RAN WGs.

# * * * * Next change * * * *

4.3.3	AMF

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the AMF performs the following functions:

-	Select a PCF supporting Ranging/SL positioning Policy/Parameter provisioning based on indication of Ranging/SL positioning Capability as part of the 5G ProSe Capability/V2X capability in the "5GMM capability" in the Registration Request.

-	Store the Ranging/SL positioning Capability.

-	Select the LMF supporting Ranging/SL positioning and forward the Ranging/SL positioning Capability of Target UE to LMF.

-	Select a GMLC supporting Ranging/SL positioning and request the GMLC for Ranging/SL Positioning result.

-	Forward the Ranging/SL positioning Capability to PCF.

-	Obtain from UDM the subscription information related to Ranging/SL positioning and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about Ranging/SL Positioning over PC5 (i.e. as Ranging/SL positioning-enabled UE).

-	Provision the NG-RAN with PC5 QoS parameters related to RSPP transport over PC5.

Editor's note:	Whether Ranging/SL positioning QoS parameters as defined in clause 5.7.2 are also provided to NG-RAN is FFS, and will be decided by RAN WGs.

# * * * * Next change * * * *

## 5.7	QoS Handling

### 5.7.1	General

QoS of Ranging/SL Positioning includes 2 aspects:

-	Ranging/SL Positioning QoS, which refers to the quality of the Ranging/SL Positioning result

-	RSPP transport QoS, which refers to the quality of the PC5-U communication for transmitting RSPP traffic.

### 5.7.2	Handling of Ranging/SL Positioning QoS

Ranging/SL Positioning QoS requirement may be provided in the Ranging/SL Positioning service request, which is generated at the application layer and is provided from the application layer to the Ranging/SL Positioning layer, provided by the SL Positioning Client UE or provided by LMF.

The Ranging/SL Positioning layer maps the Ranging/SL Positioning QoS requirement to the Ranging/SL Positioning QoS parameters and provides the Ranging/SL Positioning QoS parameters to the AS layer. If there is not received Ranging/SL Positioning QoS requirement from the application layer, the Ranging/SL Positioning layer determines the Ranging/SL Positioning QoS parameters based on the Ranging/SL positioning Policy/parameters as configured in the clause of 5.1.

Ranging/SL Positioning QoS information contains attributes defined in clause 4.1b of TS 23.273 [8] with the following additions:

-	The accuracy attribute also includes the accuracy of horizontal/vertical distance and the accuracy of horizontal/vertical direction;

-	Range, which indicates the applicability of the QoS attributes in the Ranging/SL Positioning operation over PC5.

Editor's note:	Other QoS attributes are FFS, and will be aligned with RAN WGs.

5.7.3	Handling of RSPP transport QoS

The V2X/ProSe layer handles the RSPP traffic as the V2X/ProSe application data packets for the QoS treatment. QoS handling mechanism of V2X as defined in clause 5.4 of TS 23.287 [6] or QoS handling mechanism of 5G ProSe as defined in clause 5.6.1 of TS 23.304 [7] is reused.

Editor's note:	PQI values for RSPP transport is FFS.

# * * * * End of changes * * * *