3GPP TSG-WG SA2 Meeting #157	S2-2307005

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2304713)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0122 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 EACI provisioning from VPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | On the following EN: Editor’s note: it is FFS whether a new API is used for V-AF to provide ECAI information for roamers or existing NEF services can be extended to do so. Current TS is to reuse external parameter privisioning procedure. However, a V-UDM is needed to reuse the procedure, which is too heavy and will impact the roaming architecture. Thus this contribution proposes to add a new procedure and corresponding new services to forward the EACI in the VPLMN to the V-SMF via the V-NEF, similar with EDI and PFD provisioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new clauses 6.5.2.6.X and 6.5.2.6.Y to support ECS Address Configuration Information provisioning from VPLMN in roaming case. - Some editoral changes to align with the proposed procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF deployed in the VPLMN provides the VPLMN EACI to the V-SMF is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.6.1, 6.5.2.6.X (new), 6.5.2.6.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4062 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes in SA2#157: Reason for change: - For ECS address provisioning in roaming case, the V-NEF should also map External Group ID to Internal Group ID as described in S2-2307003. Summary of new change: - Add clarifications on V-NEF retrieves Internal Group Identifier from the UDM in the HPLMN of the UE(s) in ECS address provisioning in roaming case. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.5.2.6	ECS Address Provisioning in Roaming

6.5.2.6.1	General

For both LBO and HR case, the subscription data of the ECS Address Configuration Information in UDM or UDR is stored per PLMN ID.

For the LBO case, an AF in the visited PLMN may provide the EACI via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502 [3] to UDM via H-NEF. This ECS Address Configuration Information is further provided to SMF as part of subscription information.

For the HR case when access to EHE in VPLMN is allowed:

-	The HPLMN has the knowledge of EACI in the VPLMN: For this scenario, the AF is able to interact with NEF in HPLMN. The AF may provide the VPLMN EACI to H-NEF via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502 [3], and further to the UDM. During the HR PDU session establishment procedure, the H-SMF sends the VPLMN EACI to V-SMF and then to UE. The V-SMF does not modify, but just delivers the EACI provided by the H-SMF.

-	HPLMN does not have the knowledge of EACI in VPLMN: For this scenario, the AF can't interact with NEF in HPLMN. As defined in clause 6.5.2.6.X and 6.5.2.6.Y, V-NEF stores the VPLMN EACI received from AF deployed in the VPLMN in V-UDR, and V-SMF subscribes from V-NEF to retrieve the VPLMN EACI. During the HR PDU Session establishment, the V-SMF sends the VPLMN EACI obtained from V-NEF to the H-SMF, and H-SMF decides the VPLMN ECS Address Configuration Information sent to V-SMF and then to UE according to the PLMN ID additionally.

NOTE:	It depends on the PLMN operation that the HPLMN can decide the ECAI if both VPLMN and UDM provides the ECAI.

# * * * * Second change, all new text * * * *

6.5.2.6.X	ECS Address Configuration Information Provision from AF via NEF in VPLMN

The AF provides non-PDU Session specific ECS Address Configuration Information via NEF in VPLMN to 5GC is defined in this clause.

UDR

NEF

AF

1.Nnef_ECSAddress_Create /Update/Delete Request

2.NEF Handling

3.Nudr_DM_Create/Update/Delete Request

4.Nudr_DM_Create/Update/Delete Response

5.Nnef_ECSAddress_Create /Update/Delete Response

Figure 6.5.2.6.X-1 ECS Address Configuration Information provisioning to UDR via NEF in VPLMN

1.	The AF invokes the Nnef_ECSAddress_Create /Update/Delete service operation to provide ECS Address Configuration Information to the NEF in VPLMN.

2.	NEF checks whether the AF is authorized to perform the request based on the operator policies.

If External Group Identifier is provided, the NEF determines the HPLMN of the UE(s) (e.g. based on the Realm in the identifier) and invokes Nnef_UEId_Get service to indicate NEF in HPLMN to retrieve Internal Group Identifier from UDM by invoking Nudm_SDM_Get service.

3.	The NEF invokes the Nudr_DM_Create/Update/Delete to the UDR in VPLMN if it is authorized.

4.	The UDR stores/updates/removes the corresponding information (and responds a Nudr_DM_Create/Update/Delete Response to the NEF).

5.	The NEF sends Nnef_ECSAddress_Create/Update/Delete Response to the AF.

6.5.2.6.Y	ECS Address Configuration Information Provision to the SMF in VPLMN

V-SMF supporting HR-SBO may receive the ECS Address Configuration Information from NEF in VPLMN via Subscribe/Notify procedure is defined in this clause.

SMF

NEF

3.Nnef_ ECSAddress _Notify Request

4.Nnef_ ECSAddress _Notify Response

1.Nnef_ ECSAddress _Subscribe Request

2.Nnef_ ECSAddress _Subscribe Response

Figure 6.5.2.6.Y-1: ECS Address Configuration Information provisioning to SMF in VPLMN

1-2.	As pre-requisite condition, the SMF subscribes to ECS Address Configuration Information Change Notification from the NEF by sending Nnef_ECSAddress_Subscribe message. The SMF may indicate that the current status of ECS Address Configuration Information shall be notified immediately (if available). The SMF may indicate for which (list of) DNN and/or S-NSSAI and/or Internal Group Identifier (if available) it subscribes. NEF may further subscribe to ECS Address Configuration Information Change Notification from the UDR using Nudr_DM_Subscribe.

3-4.	The NEF invokes Nnef_ECSAddress_Notify (ECS Address Configuration Information) to the SMF if the ECS Address Configuration Information is updated. If there is ECS Address Configuration Information available and immediate report is required, the NEF notifies the SMF(s) with such information immediately. NEF may retrieve the ECS Address Configuration Information from UDR using Nudr_DM_Query/Notify.

# * * * * End of changes * * * *