3GPP TSG-SA2 Meeting #157	S2-2307054

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0765 | rev | 5 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service operations update to general procedure for Federated Learning between NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Intel, ETRI, Ericsson, Nokia, Nokia Shanghai Bell, Vivo, LG Electronics, China Mobile], Lenovo Future Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current procedure for FL among multiple NWDAFs has the following shortcomings: i) The Server NWDAF periodically reports to the conusmer (AnLF) (step 6a) the updated trainign status. This is an unnecessary step. Instead, the consumer (AnLF) in step 0 provides to the Server NWDAF the desired status in terms of the accuracy level and the total training time. The Server NWDAF can use this information in step 5 after aggregating the ML model to check if the desired accuracy level and the total training time are fulfilled and stop the process if that is the case. The consumer (AnLF) has not reason to repeatetly update this subscription at that point. ii) Step 3 shall be optional since steps 7-9 in 6.2.C.2.1 may also check the data availability related to the selection of Client NWDAFs. iii) The Client NWDAF (in step 4) computes the local model metric and provides the local training status to the Server NWDAF. The Server NWDAF needs to perform this process for the global model metric without being clear how to use the local model metric. That makes the use of the local model metric questionable. iv) The Client NWDAF (in step 4) provides information regarding the characteristics of the local training data without explaining how this information can be useful to the Server NWDAF. v) The FL Server NWDAF (in step 4a) may instruct FL Clients NWDAF that have not yet provided an ML model update to report the current ML model updates to take advantage of the effort spend so far for FL training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution introduces the following changes as highlighted in the text: i) The periodic report produced from Server NWDAF towards the conusmer (AnLF) (step 6a) is removed. Instead, this contribution proposes to use the consumer (AnLF) input at step 0 related to the accuracy level and the total training time to enable the Server NWDAF to check in step 5 if the desired accuracy level and the total training time are fulfilled and stop the process if that is the case. If the process is stopped, in step 6 the Server NWDAF can then provide the updated aggregate ML model to the consumer, otherwise step 6 is skiped and a new iteration continues with step 7. ii) Mark step 3 as optional and elaborate when it is needed. iii) Remove in step 4 the text related to the Client NWDAF computed local model metric and provision of the local training status to the Server NWDAF. iv) Remove in step 4 the text related to the Client NWDAF provision of information regarding the characteristics of the local training data to the Server NWDAF. v) Add an alternative option (step 4a) that enables the FL Server NWDAF to optionally send to the FL Client NWDAF that have not yet provided an ML model update report a request instructing it to cease the ML model training and to report back the current ML model updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF or NWDAF containing MTLF) sends a subscription request to NWDAF containing MTLF to retrieve/train an ML model, using Nnwdaf_MLModelProvision service (if consumer is NWDAF containing AnLF) as defined in clause 7.5 or Nnwdaf_MLModelTraining_Subscribe Service (if consumer is NWDAF containing MTLF) as defined in clause 7.10 or Nnwdaf_MLModelTrainingInfo_Request(if consumer is NWDAF containing MTLF) as defined in 7.X, including Analytics ID, ML model metric (e.g., ML model Accuracy/Precision/Recall, Accuracy reporting intervalpre-determined status (ML model Accuracy threshold, Total training time).

NOTE 1:	The ML model Accuracy threshold can be used to indicate the target ML Model Accuracy of the training process, and the FL server NWDAF may stop the training process when the ML model Accuracy threshold is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF) provides the time when the ML model  is needed, the FL Server NWDAF can take this information into account to decide the maximum response time for its FL Client NWDAFs.

1. FL Server NWDAF containing MTLF selects NWDAF(s) containing MTLF (FL Client NWDAF) as described in clause 6.2C.2.1.

2.	FL Server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or  Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF containing MTLF(FL Client NWDAF) that participates in the Federated learning to perform the local model training based on the input parameter in the request from FL Server NWDAF, including model metric. The request also includes the maximum response time before which the FL Client NWDAF has to report the interim local ML model information to the FL Server NWDAF.

3.	[Optional] Each FL Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5] if the Client NWDAF has not local data available already.

4.	During Federated Learning training procedure, each FL Client NWDAF further trains the ML model provided by the FL Server NWDAF based on its own data, and reports the interim local ML model information to the Server NWDAFFL Server NWDAF in Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Response.

The ML model, which is sent from the FL Client NWDAF(s) to the FL Server NWDAF during the FL training process, is the information needed by the FL Server NWDAF to build the aggregated model based on the locally trained ML model(s).

If the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the FL Server NWDAF, the FL Client NWDAF shall  notify the FL Server NWDAF on the delay event with proper cause information (e.g. local ML model training failure, more time necessary for local ML model training) and the expected time to complete the training if available before the maximum response time elapses.

4a.	[Optional]If FL Server NWDAF receives notification that the FL Client NWDAF is not able to complete the training within the maximum response time, the FL Server NWDAF may send to the FL Client NWDAF an extended maximum response time in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request, before which the FL Client NWDAF has to report the interim local ML model information to the FL Server NWDAF. Alternatively, the FL Server NWDAF may send to the FL Client NWDAF that have not yet provided an ML model update report a request instructing it to cease the ML model training and to report back the current ML model updates. Otherwise, the FL Server NWDAF may indicate FL Client NWDAF to skip reporting for this iteration. FL Server NWDAF includes the current iteration round ID in the message to indicate that the request is to modify the training parameters of the current interation round.

5.	The FL Server NWDAF aggregates all the local ML model information retrieved at step 3, to update the global ML model. The FL Server NWDAF may update the global ML model each time a FL Client NWDAF provides updated local ML model information as part of FL or the FL Server NWDAF may decide to wait for local ML model information from all FL Client NWDAF before updating the global ML model.

If the FL Server NWDAF provides the maximum response time for the FL Client NWDAFs to provide the interim local ML model information in step 1, or the extended maximum response time in step 4a, the FL Server NWDAF decides either to wait for the FL Client NWDAFs which have not yet provided their interim local ML model within the (extended) maximum response time or aggregates only the retrieved local ML model information instances to update global ML model. The FL Server NWDAF makes this decision, considering the notification/response from the FL Client NWDAF or, if the notification is not received, based on local configuration.

The FL Server NWDAF checks if the requested ML model metric from the consumer (NWDAF containing AnLF) regarding (e.g., accuracy level and/or total training time) are fulfilled or not. If the requested accuracy level is fulfilled, then the process is completed and/or if the total training time is reached then the process shall be ceased, and step 6 follows to update the consumer (NWDAF containing AnLF) and break the loop, i.e., steps 7 and 8 are skipped. Otherwise, a new iteration is needed, step 6 is it skipped, i.e., not performed on this round, and the process continuous with a new iteration starting in step 7.

6.	[Optional] Based on the consumer request in step 0, the FL Server NWDAF sends a Nnwdaf_MLModelProvision_Notify message or Nnwdaf_MLModelTraining_Notify Service or Nnwdaf_MLModelTrainingInfo_Response to update the global model metric when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).

7.	If the FL procedure continues, FL Server NWDAF determines FL Client NWDAF as described in clause 6.2C.2.2 and sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request that includes the aggregated ML model information to selected FL Client NWDAF(s) for next round of Federated Training.

8.	Each FL Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the FL Server NWDAF at step 7.

NOTE 3:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

When the Federated Training procedure is complete, the FL Server NWDAF requests the FL client NWDAF(s) to terminate the FL procedure by invoking Nnwdaf_MLModelTraining_Unsubscribe service with a cause code that the FL process has finished and optionally with the final aggregated ML model information. Then the FL client NWDAF(s) terminate the local model training, and if the final aggregated ML model information is received from the FL server NWDAF, the FL client NWDAF(s) can store it for further use.

After the training procedure is complete, the FL Server NWDAF may send Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelTraining_Notify Service or Nnwdaf_MLModelTrainingInfo_Response that includes the globally optimal ML model information to the consumer.

* * * End of Changes * * *