3GPP TSG-SA2 Meeting #157	S2-2307080

# Berlin, Germany, May 22 – 26, 2023													    (was S2-2305486)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3959 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Forwarding of UE reporting of URSP rule enforcement between SM-PCF and UE-PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Deutsche Telekom, Samsung, NEC, OPPO,] Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#155, the support of UE reporting of URSP rule enforcement for deployments where the PCF serving the PDU session is not the same as the PCF serving the UE was left FFS. Furthermore, concerns were raised regarding using parameter provisioning, with some views expressed that reusing the procedure already present for application detection would be preferred. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to reuse the existing procedure in 4.16.14.2.1 for forwarding the information between SM-PCF and UE-PCF. For clarity reasons, it was decided to write a new clause. Adding the required information to the existing clause or writing a new clause based on describing changes to the existing clause would result in convoluted text that would not be clear to the reader, worst case breaking existing text. Rev 2: Procedure for URSP rule enforcement information is replaced by using the new service operation. Definition for the new service is added in clause 5.2.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality in deployments where the PCF serving the PDU session is not the same as the PCF serving the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.X(New) , 5.2.5.1, 5.2.5.X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.503 CR 0947 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R01: - Revision of S2-2304069r13 - +Step 9: removing “and PDU session information” - +Editor's note: whether “and PDU session information” should be mentioned in step 9 as part of Npcf_PolicyAuthorization_Notify is FFS Rev 2: - Subscription procedure is replaced with new service provided by PCF for the UE, to simpify the interaction of CC reporting and URSP retrieval as discussed in S2-2307079 - Definition of the new service |  |  |  |  |  |  |  |  |

* * * Start of Change (all new text)* * *

4.16.X	Awareness of URSP Rule Enforcement

4.16.X.1	General

Awareness of URSP rule enforcement is specified in clause 6.6.2.4 of TS 23.503 [20].

The content of this clause describes the PCF procedures necessary to realize this functionality.

If PCF for the UE and PCF for the PDU Session are the same PCF, then step 5, 6 and 8 in Figure 4.16.x.2-1 are not performed.

If the PCF for the UE and the PCF for the PDU Session are different PCFs, then the PCF for the UE is informed when the PCF for the PDU Session receives URSP rule enforcement.

4.16.X.1	Forwarding of URSP Rule Enforcement Information

This procedure applies when the PCF serving the PDU session receives URSP rule enforcement information from the SMF and forwards this information to the PCF serving the UE.

Figure 4.16.X.1-1: Procedure of URSP Rule Enforcement Awareness

1.	The UE registers to 5GC, as described in clause 4.2.2.2.2.

2.	The PCF for the UE registers its information to the BSF by invoking Nbsf_Management_Register service operation.

3.	The UE initiates PDU Session Establishment/Modification procedure based on URSP matching result. If the Traffic Descriptor of the matched URSP rule contains Connection Capabilities and the UE is capable of reporting URSP rule enforcement to the network, the UE includes the corresponding Connection Capabilites within the PDU Session Establishmen or Modification request message towards the SMF.

4. The SMF initiates SM Policy Association Establishment or Modification procedure as defined in clasue 4.16.4 or 4.16.5.1 respectively. The PDU Session parameters and the UE reported Connection Capabilities is included the request message, if available.

5. Based on the received Connection Capabilities or local policies, the PCF for the PDU Session may determine to contact with the PCF for the UE to inform the URSP rule enforcement information.

If the PCF ID of the PCF for the UE is not available yet, it invokes Nbsf_Management_Discovery to the BSF to retrieve the PCF ID of the PCF for the UE. Alternatively, if the PCF for the PDU Session receives the request sent by the PCF for the UE in step 1 via AMF and SMF, then step 5 is skipped and step 6B will be performed.

6. The PCF for the PDU Session invokes Npcf_PolicyCombination_Create or Npcf_PolicyCombination_Update request to the PCF for the UE, by including URSP rule enforcement information in the request message, which may contain Connection Capabilities reported from the UE if available, detected application events if available and the PDU Session parameters, e.g. DNN, S-NSSAI, SSC mode, etc.

Based on the received URSP rule enforcement inforamtion, the PCF for the UE checks the the PDU Session parameters is consistent with the list of URSP rules sent to the UE, and provides one or more traffic descriptor corresponds to the URSP rule enforcement information to the PCF for the PDU Session as input for PCC rule generation.

7a. If the PCF for the UE determines to adjust the URSP rules based on the request message, it triggers UE Policy Association modification procedure as defined in clause 4.16.12.2, and updates the URSP rules in the UE.

7b. Based on the traffic descriptor in the response message, the PCF for the PDU Session generates PCC rules as defined in clause 6.1.6.1 of TS 23.503[20].

8. When the PCF for the PDU Session receives updated Connection Capabilities from the UE, or it receives newly detected application event, it invokes the Npcf_PolicyCombination_Update request to the PCF for the UE.

When the PDU Session is released, then the PCF for the PDU Session invokes the Npcf_PolicyCombination_Delete request to the PCF for the UE.

# * * * * Next change * * * *

5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF, TSCTSF |
| (NOTE 1) | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Subscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Unsubscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, V-PCF |
|  | Update | Request/Response | AMF, V-PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Npcf_AMPolicyAuthorization | Create | Request/Response | AF, NEF, TSCTSF |
|  | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, 5G DDNMF |
|  | Subscribe |  | AF, NEF, 5G DDNMF |
|  | Unsubscribe |  | AF, NEF, 5G DDNMF |
| Npcf_PDTQPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_PolicyCombiniation (NOTE 1) | Create | Request/Response | PCF |
|  | Update | Request/Response | PCF |
|  | Delete | Request/Response | PCF |
| NOTE: In the Npcf_PolicyAuthorization and Npcf_PolicyCombination operations, PCF is a consumer when the PCF for the UE and the PCF for the PDU session are different. |  |  |  |

# * * * * Next change * * * *

5.2.5.X	Npcf_PolicyCombination Service

5.2.5.X.1	General

Service description: This service allows the PCF for the PDU Session interact with the PCF for the UE for combined policy decision scenario when the PCF for the UE and PCF for the PDU Session are separately deployed, which includes the following functionalities:

-	triggers the check for UE reported Connection Capabilities, and retrieves the traffic descriptor of the traffic allowed to occur.

5.2.5.X.2	Npcf_PolicyCombination_Create service operation

Service operation name: Npcf_ PolicyCombination _Create

Description: This service is to create the combined policy association in the PCF for the UE.

Inputs, Required: SUPI.

Inputs, Optional: Connection Capabilities, DNN, S-NSSAI, SSC mode, PDU Session Type, Acceees Type, RSN and/or PDU Session Pair ID.

Outputs, Required: Combined Policy Association ID.

Outputs, Optional: One or more traffic descriptor of the traffic corresponding to the UE reported Connection Capabilities as described in TS 23.503[20].

5.2.5.X.3	Npcf_ PolicyCombination _Update service operation

Service operation name: Npcf_ PolicyCombination _Update

Description: This service is to update the combined policy association in the PCF for the UE.

Inputs, Required: Combined Policy Association ID.

Inputs, Optional: Connection Capabilities, DNN, S-NSSAI, SSC mode, PDU Session Type, Acceees Type, RSN and/or PDU Session Pair ID.

Outputs, Required: None

Outputs, Optional: One or more traffic descriptor of the traffic corresponding to the UE reported Connection Capabilities as described in TS 23.503[20].

5.2.5.X.4	Npcf_ PolicyCombination _Delete service operation

Service operation name: Npcf_ PolicyCombination _Notify

Description: This service is to terminate the combined policy association in the PCF for the UE.

Inputs, Required: Combined Policy Association ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

* * * End of Change* * *