SA WG2 Meeting #S2-157	S2-2306272

22 - 26 May, 2023, Berlin, Germany

SA WG2 Meeting #S2-156E	S2-2303925

17 - 21 April, 2023, Electronic

|  | 5GMRR Doc 41_39r2 LS to 3GPP (SA1, SA2, SA3) on Roaming Value Added Services requirements |
| --- | --- |
| Meeting Information |  |
| Meeting Name and Number | 5GMRR #41 |
| Meeting Date | 2023-03-02 |
| Meeting Location | London |
| Document Information |  |
| Document Author | Gery Verwimp, Bics |
| Additional Contributors | [List of contributors] |
| Document Creation Date | [Document Creation Date] |
| Document Status | [This document is for] |
| Security Classification | Non-confidential |
| Document Summary |  |
| [Summary] |  |

| Version History |  |  |
| --- | --- | --- |
| Date | Version | Author / Comments |
| 2 March 2023 | 1 | Gery Verwimp, Bics Draft LS as part of a bundle of LS agreed at 5GMRR#41 requesting refinements to the 5G SA roaming architecture |
| 20 March 2023 | 2 | Pieter Veenstra (Titan.ium Platform) 5GMRR Chair Approved LS after 5GMRR email review based r6 |

© GSMA © 2023. The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice. This document has been classified according to the GSMA Document Confidentiality Policy. GSMA meetings are conducted in full compliance with the GSMA Antitrust Policy.

# Liaison Statement

| Liaison Statement Title: | LS Roaming Value Added Service Requirements |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| 5GMRR #41 | 03/02/2023 | London - GB |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| 5GMRR Doc 41_39r2 | 6 March 2023 | Gery Verwimp |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| NG/NRG/5GMRR | Fill in Date if any | gsmaliaisons@gsma.com |
| Security Classification | Non-confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | NRG, DESS, WSOLU |
| External Recipients: | To: 3GPP SA1, SA2, SA3 CC: 3GPP SA, CT4 |

# Overall Description

This LS focusses on one of the three roles in the roaming eco-system, namely roaming value-added service (RVAS) providers.

# Background

RVAS have been the subject of dynamic innovation in an industry that is constantly looking to improve roaming revenues or reduce roaming costs. In previous generations than 5G, RVAS are typically embedded in standard architectures and network functions and are based on a combination of techniques and solutions available in the market.

In 5G, the integration via the NEF API appears to be straight-forward for some RVAS, for others this is not the case.

One notable RVAS is sponsored roaming, as described in GSMA PRD BA.23. This service enables network operators (MVNOs or non-public networks) to fast launch international roaming for their subscribers. Technically speaking, these network operators (called client MNO in the following) make use of roaming agreements of an established MNO, while using their own network or relations when available. Such an established MNO is called “sponsor” or “donor” MNO. An RVAS provider may bundle multiple sponsor MNOs in its outbound roaming service offering.

In such a context it is not possible to establish an end-to-end relation between VPLMN (roaming partner of sponsor MNO) and the actual HPLMN of the subscriber (client MNO): the sponsor MNO needs to play a pivotal role. While the relation between VPLMN and sponsor MNO fits within the 5G SA roaming architecture, an adequate solution for the relation between sponsor MNO, outbound roaming service (RVAS) provider and client MNO (with or without an official PLMN-ID) remains undefined.

In the sense of 5G SA architecture, the only roaming relations that exist are between the VPLMN and the sponsor MNO (as HPLMN). Other relation types remain undefined:

- Relation between sponsor MNO and outbound roaming service (RVAS) provider; the latter can have relations with multiple sponsor MNOs.

- Relation between RVAS provider and client MNO. There may be multiple client MNOs per sponsor MNO.

While the relation between sponsor MNO and RVAS provider is private and most likely requires a bespoke service-based integration, a more scalable and standard solution is needed for the relation between RVAS provider and client MNO. GSMA 5GMRR suggest using N32 for this purpose, where N32 connections per sponsor-client pair are conceivable, and where VPLMN-ID remains visible to the client MNO.

It is also noteworthy that sponsored roaming is currently facilitated by using dual-IMSI profiles, i.e. single SIM profile (single secret key) with 2 IMSI: a sponsor IMSI and a client IMSI. The client IMSI is used in the client’s own network and in any roaming partner the client managed to contract directly. In all other visited networks, the sponsor IMSI is used. Translated to a 5G context, this may mean:

- SUCI deconcealment would happen at the RVAS provider, before the actual client MNO can be identified (client MNO is identified based on assigned IMSI subranges, managed by RVAS provider).

- SUPI is mapped from sponsor to client SUPI and vice versa, at the RVAS provider. Client MNO SUPI is never exposed to VPLMN (there is no roaming relation between them) and sponsor SUPI is not exposed to the client MNO (this would require multi-IMSI/multi-SUPI HSS/UDM with different mcc/mnc which is usually not supported in a client’s network).

5GMRR proposes the following list of requirements to consider for enhancement of the 5G SA roaming eco-system.

Requirements

- Allow for sponsored 5G roaming relations to be handled by RVAS providers, notably to enable sponsored 5GS roaming for new entrants or private network operators

- Define N32 security context between RVAS provider and client, notably in the context of sponsored roaming

# Actions

GSMA NRG 5GMRR kindly requests 3GPP to consider the attached service requirements from RVAS providers.

GSMA 5GMRR kindly asks 3GPP to determine how the 5G specifications can be adapted to support these requirements.

GSMA 5GMRR asks 3GPP to take note of the current sponsored roaming facilitation using dual or multi-IMSI profiles on the SIM, and advise on any concerns with respect to authentication or concealment algorithms.

SA1, SA2, and SA3 are kindly asked to take into account the above requirements of GSMA 5GMRR to enable RVAS providers.

# Next meetings

5GMRR#42:	3-4 April 2023

5GMRR#43:	TBA June 2023