3GPP TSG-SA2 Meeting #157	S2-2306393

May 22-26, 2023, Berlin, DE							                                      (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4569 | Rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Asynchronous Type Communication for CN based MT communication handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT, NR_RedCap_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the discussion in S2-2306343, it is proposed to clarify that the Asynchronous Type Communictaion (ATC) can also be used in the following cases: - when Control Plane CIoT 5GS Optimisation are used with UE configured with eDRX for CM_IDLE state - when the CN handles mobile terminated (MT) communication for the UE configured with eDRX for RRC_INACTIVE state |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.23 (ATC): added two additional cases for ATC as in the reason for Change. Clause 5.31.7.2.2.3 (CM_IDLE): added text on AMF behaviour similar to the existing text in clause 5.31.7.2.4 (RRC_INACTIVE). Clause 5.31.7.2.4 (RRC_INACTIVE): clarified that in case UE is unreachable, the AMF stores the relevant information and later forwards the N2 message when the AMF determines that the UE has become reachable again. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether ATC can be used in case of Control Plabe CIoT 5GS Optimiastions with UE configured with eDRX cycle for CM_IDLE, or in case of CN based MT communication handling for UE in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.23; 5.31.7.2.2.3; 5.31.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.23	Supporting for Asynchronous Type Communication

Asynchronous type communication (ATC) enables 5GC to delay synchronizing UE context with the UE, so as to achieve an efficient signalling overhead and increase system capacity. The support of ATC is optional for the AMF.

5GC supports asynchronous type communication with the following functionality:

-	Capability to store the UE context based on the received message, and synchronize the UE context with the involved network functions or UE later;

For network function (e.g. PCF, UDM, etc.) triggered signalling procedure (e.g. network triggered Service Request procedure, network triggered PDU Session Modification procedure, etc.), if the UE CM state in the AMF is CM-IDLE state and the requesting network function indicates to the AMF the ATC is allowed for the signalling, if the AMF supports the ATC feature, the AMF may update and store the UE context based on the received message without paging UE immediately. When the UE CM state in the AMF enters CM-CONNECTED state, the AMF forwards N1 and N2 message to synchronize the UE context with the (R)AN and/or the UE.

ATC is also used in the following cases:

- when Control Plane CIoT 5GS Optimisation are used with UE configured with eDRX for CM_IDLE state (refer to clause 5.31.7.2.2.3). Upon receiving a request for MT communication from an NF, if the AMF determines that the UE is not reachable based on UE’s eDRX cycle for CM_IDLE, the AMF stores the information received in the NF request and proceeds with sending the N2 message only when the AMF determines that the UE has become reachable again, as described in clause 5.31.7.2.2.3.

- when the CN handles mobile terminated (MT) communication for the UE configured with eDRX for RRC_INACTIVE state (refer to clause 5.31.7.2.4). Upon receiving a request for MT communication from an NF, if the AMF determines that the UE is not reachable based on UE’s eDRX cycle for RRC_INACTIVE, the AMF stores the information received in the NF request and proceeds with sending the N2 message only when the AMF determines that the UE has become reachable again, as described in clause 5.31.7.2.4.

If the originating NF does not require immediate delivery, it may indicate that the AMF is allowed to use ATC.

NOTE:	Pre-Rel-17 AMF implementation cannot decode the indication that ATC is allowed.

NEXT CHANGE

5.31.7.2.2.3	AMF paging and paging retransmission strategy

NOTE:	This clause applies for extended DRX cycle lengths of 10.24s or longer.

When MT data or signalling arrives for a UE in CM_IDLE state, the other NFs communicate with the AMF for delivery of MT data or signalling. The AMF calculates the UE reachability based on the eDRX cycle value for CM_IDLE state provided by NG-RAN and triggers NG-RAN paging via an N2 message if the UE is considered reachable as specified in clause 4.24.2 of TS 23.502 [3]. If the UE is considered unreachable, the AMF stores the information received in the NF request and replies to the requesting NF to apply high latency communication functions as specified in clause 5.31.8 based on eDRX cycle for CM_IDLE (e.g. an Estimated Maximum Wait Time is calculated based on eDRX cycle value for CM_IDLE). When the AMF determines that the UE is about to become reachable, the AMF uses the stored information to send an N2 message just before the next paging occasion.

The AMF determines the Paging Time Window length and a paging retransmission strategy, and executes the retransmission scheme.

For extended DRX length of 10.24s, in the paging request message the AMF sends the Paging Time Window to the ng-eNB but does not send the Paging Time Window to the gNB.

NOTE:	Pre-Rel-17 AMF implementation cannot decode the indication that ATC is allowed.

NEXT CHANGE

##### 5.31.7.2.4	Paging for extended DRX for RRC_INACTIVE in NR connected to 5GC

For NR, the NG-RAN may request the CN to handle mobile terminated (MT) communication for the UE configured with eDRX for RRC_INACTIVE state by means of the Connection Inactive procedure with CN based MT communication handling Procedure (see clause 4.8.1.1a of TS 23.502 [3]). This allows the CN to apply high latency communication functions as specified in clause 5.31.8. The NG-RAN provides the eDRX cycle value for RRC_INACTIVE to AMF (i.e. >10.24s). Based on the request from NG-RAN, the AMF responds to NG-RAN and informs other NFs (e.g. SMF and UPF) involved in downlink data or signalling handling and trigger the data buffering as specified in clause 4.8.1.1a of TS 23.502 [3].

When MT data or signalling arrives for a UE in RRC_INACTIVE state, the other NFs communicate with the AMF for delivery of MT data or signalling. The AMF calculates the UE reachability based on the eDRX cycle value for RRC_INACTIVE state provided by NG-RAN and triggers NG-RAN paging via an N2 message if the UE is considered reachable as specified in clause 4.8.2.2b of TS 23.502 [3]. If the UE is considered unreachable, the AMF stores the information received in the NF request and replies to the requesting NF to apply high latency communication functions as specified in clause 5.31.8 based on eDRX cycle for RRC_INACTIVE (e.g. an Estimated Maximum Wait Time is calculated based on eDRX cycle value for RRC_INACTIVE). When the AMF determines that the UE has become reachable, the AMF uses the stored information to send an N2 message.

When the UE enters RRC_CONNECTED state if the NG-RAN had sent the indication for the CN to handle mobile terminated (MT) communication, NG-RAN informs the AMF that the UE is now in RRC_CONNECTED state as specified in clause 4.8.2.2 of TS 23.502 [3]. The AMF then informs other NFs that the UE is now reachable using the high latency communication functions as specified in clause 5.31.8 and MT data and signalling can be delivered to the UE.

END CHANGE