SA WG2 Meeting #157	S2-2306399

Berlin, Germany, May 22 – 26, 2023	(revision of S2-2304686)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4376 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update supporting Edge Computing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_ph2 |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the conclusion in TR 23.700-48, some of the new features are supported, for example supporting roaming, common EAS/DNAI determination and etc. So, in section 5.13, some of the newly supported features should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add supporting edge computing in roaming and LBO Scenarios. Add supporting definition of finer granular sets of UEs. Add supporting common EAS/DNAI determination for UE collections. Also, the AF can select the common EAS/DNAI. Add supporting to enable the discovery of an EAS deployed by the serving Operator's OP on cloud resources provided by another PLMN or partner. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The newly added features in edge computing may not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

5.13	Support for Edge Computing

Edge computing enables operator and 3rd party services to be hosted close to the UE's access point of attachment, so as to achieve an efficient service delivery through the reduced end-to-end latency and load on the transport network. Edge Computing support by 5GC is specified in this specification and in TS 23.548 [130].

Edge Computing applies to non-roaming, LBO roaming and HR romaing scenarios.

The 5G Core Network selects a UPF close to the UE and forwards traffic to enable the local access to the DN via a N6 interface according to the provided traffic steering rules to the UPF. This may be based on the UE's subscription data, UE location, the information from Application Function (AF) as defined in clause 5.6.7, the EAS information reported from EASDF (as defined in TS 23.548 [130]), policy or other related traffic rules.

Due to user or Application Function mobility, the service or session continuity may be required based on the requirements of the service or the 5G network.

The 5G Core Network may expose network information and capabilities to an Edge Computing Application Function.

NOTE:	Depending on the operator deployment, certain Application Functions can be allowed to interact directly with the Control Plane Network Functions with which they need to interact, while the other Application Functions need to use the external exposure framework via the NEF (see clause 6.2.10 for details).

Edge computing can be supported by one or a combination of the following enablers:

-	User plane (re)selection: the 5G Core Network (re)selects UPF to route the user traffic to the local part of the DN as described in clause 6.3.3;

-	Local Routing and Traffic Steering: the 5G Core Network selects the traffic to be routed to the applications in the local part of the DN;

-	this includes the use of a single PDU Session with multiple PDU Session Anchor(s) (UL CL / IP v6 multi-homing) as described in clause 5.6.4 and the use of a PDU Session with Distributed Anchor Point using SSC mode 2/3.

-	Session and service continuity to enable UE and application mobility as described in clause 5.6.9;

-	An Application Function may influence UPF (re)selection and traffic routing via PCF or NEF as described in clause 5.6.7 in both non-roaming and roaming cases;

-	Network capability exposure: 5G Core Network and Application Function to provide information to each other via NEF as described in clause 5.20 or directly as described in clause 4.15 of TS 23.502 [3] or from the UPF as described in clause 6.4 of TS 23.548 [130];

-	QoS and Charging: PCF provides rules for QoS Control and Charging for the traffic routed to the local part of the DN;

-	Support of Local Area Data Network: 5G Core Network provides support to connect to the LADN in a certain area where the applications are deployed as described in clause 5.6.5.

-	Discovery and re-discovery of Edge Applications Servers as described in TS 23.548 [130]. Support discovery of an EAS deployed by the serving Operator's OP on cloud resources provided by another PLMN or partner (i.e. the Edge Node Sharing Scenario as described in GSMA OPG.02 [5]).

-	Support of Edge Relocation as described in TS 23.548 [130] and the case of involving AF change as described in clauses 4.3.6.2, 4.3.6.3 and 4.3.6.4 of TS 23.502 [3]. Support Edge relocation within the same hosting PLMN's EHE and different PLMNs' EHE.

-	Support of (I-)SMF (re)selection based on DNAI as described in clauses 4.3.5.1, 4.3.5.2 and 4.23.5.1 of TS 23.502 [3].

-	Support definition of finer granular sets of UEs. The 5GC and the AF may agree on a Target Category information that is an abstract value and that may represent a combination of a list of External group Identifiers or any UE. The NEF may, based on local policies, determine a combination of a list of group Identifiers or any UE, that is to be associated with the Target Category.

-	Support definition of UE collections, and common EAS discovery and common DNAI determination from 5GC or AF for UE collections.

-	Support storage of mapping information between EAS IP/IP range and DNAI in NEF/UDR, and support to obtain the mapping information from NEF/UDR.

* * * End of Change * * *