3GPP TSG-SA2 Meeting #157	S2-2306427

May 22 – 26, 2023,  Berlin, Germany

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4579 | Rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 23.501 update for support of network slice usage control on deregistration inactivity timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the LS from CT1 on UE implementing the de-registration inactivity timer, CT1 proposes keep network control only in order to avoid the misalignment between the AMF side and UE side. CT1 notes the AMF has been required to maintain the deregistration inactivity timer per on-demand S-NSSAI to achieve the network slice usage control. Namely, the AMF is aware of the timer and the case for which the on-demand S-NSSAI needs to be de-registered. If the network slice needs to be de-registered, the AMF removes it from the allowed NSSAI and informs the UE about the new allowed NSSAI. In addition, the timer and the status of the on-demand S-NSSAI on the AMF side will always be regarded as the baseline regardless of the misalignment on the UE side. Based on the above, CT1 considers whether a solution with network control only can be possible in which a de-registration inactivity timer on the UE side would be avoided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Keep the network-based per UE Network Slice usage behaviour control solution only, the de-registration inactivity timer only runs at the AMF side to avoid the misalignment of de-registration inactivity timer on the UE side |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned in different specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.15.15.2	UE Configuration of network-controlled Slice Usage Policy

The UE during the Registration procedure may indicate in UE MM Core Network Capability that it supports UE configuration of network-controlled Slice Usage Policy. If so, the AMF determines Slice Usage Policy for a Network Slice for the UE and may configure the UE with this information together with Configured NSSAI to control the usage of this Network Slice. The AMF may be locally configured with network Slice Usage Policy, or receive the policy from the (AM-)PCF, or per the information received from UDM for AF managed timer values (see clause 5.15.15.3 for more details).

The network-controlled Slice Usage Policy is provided to the UE in the Registration Accept or the UE Configuration Update Command and may include:

-	An indication, for one or more of S-NSSAI(s) of the HPLMN in the Configured NSSAI, whether the UE only registers with the Network Slice with the network when applications in the UE require data transmission in the Network Slice (i.e. the UE can only register the Network Slice only on demand and consider the Network Slice as on demand S-NSSAI).

NOTE:	All Other Network Slices in the Configured NSSAI are handled by the UE using UE specific policies (e.g. they may be registered irrespective of applications need).

END OF CHANGE