3GPP TSG SA2 Meeting #157 	S2-2306464

Berlin, Germany, 22 – 26 May, 2023	(revision of S2-230xxxx)

# Source:	China Telecom

# Title:	New SID on Proximity-based Services in 5GS Phase 3

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Proximity-based Services in 5GS Phase 3

Acronym:	FS_5G_ProSe_Ph3

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| SMARTER | SA1 | 720005 | New Services and Markets Technology Enablers |
| REFEC | SA1 | 840048 | Enhanced Relays for Energy Efficiency and Extensive Coverage |
| MultiRelay | SA1 | 970044 | Multi-path relay |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 900007 | Proximity based Services in 5GS | Rel-17 Stage 2 5G_ProSe work item. |
| 980012 | Proximity-based Services in 5GS Phase 2 | Rel-18 Stage 2 5G_ProSe work item. |
| 930008 | Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS) | Rel-17 SA3 work item on security aspects. |
| 320022 | AIPN-SAE | Requirements for evolution of the 3GPP system architecture |
| 911005 | NR Sidelink Relay | Rel-17 RAN work item on NR Sidelink Relay. |
| 941002 | NR sidelink relay enhancements | Rel-18 RAN work item on NR sidelink relay enhancements |

Dependency on non-3GPP (draft) specification: TBD

3	Justification

The 5G System has been enhanced to support the following Proximity Services features in Release 18 as specified in TS 23.304, based on the service requirements defined in TS 22.278, TS 22.261 and TS 22.115:

-	Support of single NR PC5 hop UE-to-UE Relay for unicast.

-	Enhance UE-to-Network Relay functionality to support:

-	service continuity when switching between two indirect network communication paths for UE-to-Network Relay;

-	service continuity when switching between direct network communication path and indirect network communication path for 5G ProSe Layer-2 UE-to-Network Relay, including inter-gNB indirect-to-direct and inter-gNB direct-to-indirect path switching;

-	multi-path transmission using only one direct network communication path and only one indirect network communication path e.g. for improved reliability or data rates;

-	Emergency Services for Remote UE over UE-to-Network Relay.

-	Public Warning Notification Relaying by UE-to-Network Relay

-	Support of path switching between direct NR Uu communication path and direct NR PC5 communication path.

However, due to lack of time the following objectives were de-prioritized from Rel-18 FS_5G_ProSe_Ph2 study item at the SA#94e meeting:

-	Enhancement of UE-to-Network Relay for support of multiple NR PC5 hops

-	Whether and how to support of non-3GPP RAT (e.g. Wi-Fi or Bluetooth) over PC5 reference point for Layer-3 UE-to-Network Relay.

There are also some other requirements about Proximity Services captured in TS 22.115 and TS 22.261, e.g. relaying traffic of Remote UE via different indirect network connection paths, supporting all types of traffic for indirect network connection mode, supporting use of a relay UE that supports multiple access types, identifying the relayed traffic of an UE-to-network Relay, etc., which imposes architecture enhancements that need to be addressed by Rel-19 study item.

In Rel-17 and Rel-18, NWDAF has been able to provide network data analytics to assist policy determination, e.g. RFSP, URSP, to enhance several services, e.g. AI/ML-based services, LCS services. However, whether and how Proximity Services can be benefit from NWDAF and related use cases have not been studied. It is worthy to make Proximity Services more intelligent.

All these above items not standardized within Rel-18 worth to be further investigated in the Rel-19 study item, in order to increasingly fulfil the service requirements defined in TS 22.278, TS 22.261 and TS 22.115.

4	Objective

The study item aims at further investigating 5G System enhancements to support Proximity Services, based on what has been specified in Rel-17 and Rel-18, and based on the services requirements defined in TS 22.278, TS 22.261 and TS 22.115.

The detailed objectives are to investigate potential 5GS enhancements in order to support the followings:

Objective A.	5G System enhancements to support Rel-18 5G ProSe leftovers

WT#1.	Enhancement of UE-to-Network Relay for support of multiple NR PC5 hops.

WT#2.	Whether and how to support of non-3GPP RAT (e.g. Wi-Fi or Bluetooth) over PC5 reference point for Layer-3 UE-to-Network Relay.

Objective B.	5G System enhancements to support new Proximity Services requirements

WT#3.	Support of multi-path transmission using different UE-to-Network Relay.

WT#4.	Support of Layer-3 UE-to-Network Relay over non-3GPP access.

WT#5.	Support of IMS service for Remote UEs via UE-to-Network Relay.

WT#6.	Support of VN service for Remote UEs via UE-to-Network Relay.

WT#7.	Support of identifying the access via Layer-3 ProSe UE-to-Network Relay with N3IWF.

WT#8.	Whether and how the Proximity Services can benefit from NWDAF reporting.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |

Total TU estimates for the study phase:  TBD

Total TU estimates for the normative phase:  TBD

Total TU estimates: TBD

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.XXX | Study on Proximity-based Services in 5GS Phase 3 | TBD | TBD | LI, Jiahui, China Telecom, lijh36@chinatelecom.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

LI, Jiahui, China Telecom, lijh36@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

Coordination is expected with SA3 to support security, privacy integrity.

Potentially SA5.

9	Supporting Individual Members

Supporting IM name

China Telecom