3GPP TSG-WG SA2 Meeting #157	S2-2306599

# Berlin, Germany, May 22-26, 2023   		(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0825 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on input data collection of QoS sustainability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS23.288 clause 6.9 QoS sustainability analytics enables to output finer granularity geographical area QoS sustainability instead of Area of Interest. However, the UE location information collected from LCS system as shown in Table 6.9.2-2 is better to be modified as GMLC instead of LCS. Besides, collecting UE ID from LCS system is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Group the collected input data from LCS system. Modify source “LCS” to “GMLC” Delete LCS as source of UE ID. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Information collects wrongly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## ****First Changes****

### 6.9.2	Input data

To derive the QoS Sustainability analytics for a path of interest or for an area of interest with coarse granularity (i.e. TAIs or Cell IDs), the input data is listed in Table 6.9.2-1.

Table 6.9.2-1: Data collection for "QoS Sustainability" analytics

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [10] | Average UE bitrate in the cell (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [10] | Number of abnormally released QoS flows during the time the QoS Flows were used per timeslot, per cell, per 5QI and per S-NSSAI. |
| Delay in RAN | OAM TS 28.554 [10] | Average Uplink and downlink packet transmission delay through RAN part to the UE, per timeslot, per cell, per 5QI and per S-NSSAI. |

NOTE 1:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

To derive the QoS Sustainability analytics for an area of interest with fine granularity (i.e. geographical area that can be smaller than a cell), additional input data is listed in Table 6.9.2-2.

Table 6.9.2-2: UE level data collection for "QoS Sustainability" analytics with fine granularity

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | (list of) SUPI(s) |
| Finer granularity UE location | GMLC | UE location information |
| >Timestamp |  | A time stamp associated with the collected information. |
| >UE location |  | Geodetic Location, Local Location including Coordinate ID, Civic Location as defined in TS 23.273 [39] |
| >speed |  | UE speed |
| SMF info | AMF | SMF address for the SMF serving the UE |
| UPF info | SMF | Address for the UPF serving the UE. |
| 5QI | SMF | 5G QoS Identifier. |
| PEI | UDM | Permanent Equipment Identifier, as described in clause 5.9.3 of TS 23.501 [2]. |
| Equipment type | GSMA IMEI database or OAM | Equipment Type such as smartphone, tablet, dongle, etc. as described in GSMA TS.06 [38]. |

NOTE 2:	How to use the input data from GSMA for QoS Sustainability analytics is up to NWDAF implementation.

Additionally, the NWDAF collects the following input (Table 6.9.2-3) according to measurements defined in clause 5.33.3 QoS Monitoring to Assist URLLC Service of TS 23.501 [2] and IP-layer clause capacity and IP-layer available clause capacity definition from ITU-T Y.1540 [40] between UE, NG-RAN and UPF at GTP level.

Table 6.9.2-3: Data collection for QoS Sustainability analytics at GTP level

| Information | Source | Description |
| --- | --- | --- |
| UL/DL packet delay GTP | OAM TS 28.552 [8] (NOTE) | UL/DL packet delay measurement round trip on GTP path on N3 for non-GBR traffic. |
| UL/DL capacity GTP between UPF and NG-RAN | OAM TS 28.552 [8] | UL/DL capacity measurement from UPF to NG-RAN based on GTP path. The capacity measurement corresponds to the IP-layer section capacity definition from ITU-T Y.1540 [40]. |
| UL/DL capacity GTP between UPF and UE | OAM TS 28.552 [8] | UL/DL capacity measurement from UE to UPF based on GTP path. The capacity measurement corresponds to the IP-layer section capacity definition from ITU-T Y.1540 [40]. |
| UL/DL available capacity GTP between UPF and NG-RAN | OAM TS 28.552 [8] | UL/DL available capacity measurement from UPF to NG-RAN based on GTP path. The available capacity measurement corresponds to the IP-layer available section capacity definition from ITU-T Y.1540 [40]. |
| UL/DL available capacity GTP between UPF and UE | OAM TS 28.552 [8] | UL/DL available capacity measurement from UE to UPF based on GTP path. The available capacity measurement corresponds to the IP-layer available section capacity definition from ITU-T Y.1540 [40]. |
| NOTE: Refer to clause 5.1 of TS 28.552 [8] for the performance measurement in NG-RAN and clause 5.4 of TS 28.552 [8] for the performance measurement in UPF. In addition, Annex A of TS 28.552 [8] describes various performance measurements, especially, clause A.61 "Monitoring of one way delay between PSA UPF and NG-RAN" indicates that the measurements on the one way DL and UL delay between PSA UPF and NG-RAN can be used to evaluate and optimize the DL and UL user plane delay performance between 5GC and NG-RAN. |  |  |

## ****Second Changes****

## End of Changes