3GPP TSG-SA2 Meeting #157	S2-2306641

Berlin, Germany, May 22nd-26th, 2023	(revision of S2-2305444)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3953 | rev | 2 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys - QoS filtering criteria corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Fixing incorrect or incomplete information in the QoS filter criteria section for UE member selection for AIML operation. In particular, this CR focuses on providing real-time QoS filter criteria UE member selection. Additionally, the Specific procedure for the 5GC assistance to member selection based on the UE’s current location, historical location and direction got implemented in the middle of the Specific procedure for QoS UE member filtering criteria, mixing both procedures. Rev 2 addresses two ENs: EN#1 in clause 4.15.13.2: Editor's note: The list of UE Member filtering criteria and their corresponding actions are FFS. EN #2 in clause 4.15.13.3.3: Editor's note: For QoS Flows other than QoS Flow associated to the default QoS rule, whether and how QoS monitoring is performed is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed references to time windows - Added QoS filter criteria parameters - Clarified that Application ID must be included - Moved former clause 4.15.13.3.2 to a new clause 4.15.13.X. - The list of QoS monitoring parameters of the QoS filtering criteria is a growing list, so it is replaced with a reference to 23.501 clause 5.45. - The flow in 4.15.13.3.3 now includes the UPF. Event Exposure notifications are sent from UPF directly to NEF. Rev 2 addresses these two ENs: EN#1 is now considered complete and can be deleted EN#2 has been addressed in UPEAS, tdocs S2-2306257 and S2-2306258, and the procedure allows to subscribe to the default or non-default QoS whenever the subscription is done via SMF, requiring Application ID (Flow Descriptiors are not accepted). The procedure in 4.15.13.3.3 just considers the subscription to QoS monitoring via SMF, therefore is aligned with the agreed tdocs. As a consequence, the EN is removed. Additional text is adding clarifying the cases where the QoS Flows are bound to a default QoS rule or associated to an application. The subscription requires App ID for identifying the flows to be monitored. References to Flow Descriptions have been removed. Since the AF needs to provide the App ID for the QoS filtering criteria, the S-NSSAI/DNN provided by the AF are optional parameters rather than alternative to App ID. It is clarified that the AF ID is used by the NEF for authorization purposes. The NEF southbound interface may include a request for monitoring the default QoS flows or not. The App ID is therefore, an optional parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect or incomplement information leads to inability to provide a QoS filtering criteria assistance to UE member selection and confuses ther reader with the procedure for member selection base don UE’s current location, historical location anddirecton. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.2, 4.15.13.3.1, 4.15.13.3.2, 4.15.13.3.3, 4.15.13.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.15.13.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

Table 4.15.13.2-1 provides a summary of the UE member filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria | Service operation: Nsmf_EventExposure or Nudm_EvenExposure, Filter: target=SUPI, Event=QoS Monitoring, Application ID | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events, and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time | Clause 6.18 of TS 23.288 [50]. |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) | 4.15.13.X |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period | 4.15.13.X |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction | 4.15.13.X |

* * * Next Change * * *

#### 4.15.13.3	Specific procedure for QoS UE member filtering criteria

##### 4.15.13.3.1	General

An AF may invoke Nnef_UEMemberSelectionAssistance_Subscribe service operation with a QoS filtering criteria for receiving a list of UEs that match or exceed such criteria.

In addition to the mandatory parameters, the AF also includes in the request:

-	QoS filtering criteria.

-	Optionally, an Area of Interest: location area of the candidate UEs.

The QoS filtering criteria includes:

- Application ID.

And one or more of the QoS parameters subject to QoS monitoring (see list in TS 23.501 [2] clause 5.45).

The current list of UEs that match or exceed the QoS filtering criteria and are optionally located in the AoI in real time, for the duration of the subscription, is further described in clause 4.15.13.3.3.

* * * Next Change * * *

##### 4.15.13.3.2	VoidSpecific procedure for the 5GC assistance to member selection based on the UE's current location, historical location and direction.

Figure 4.15.13.3.2-1: 5GC assistane to Member selection based on the UE's travel history, direction and separation distance

1.	AF requests 5GS assistance to support the UE selection by considering the UE's historical location, UE's current location and direction. AF includes the UE lists and the following criteria as part of the FL UE selection request:

-	UE historical location: The Target AoI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

-	UE current location: The current AoI which is the coverage area of the FL training server where the selected UEs located in to participate in the FL operation.

-	Direction: Select the UE with the different direction in the FL coverage Area.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the filtering criteria into the corresponding UE filtering information.

3-4. The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, the current AoI and event ID = Location report. AMF will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the current AoI.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility, AOI = Target AOI) with UE list received from step 4.

6-7. The NEF invokes NWDAF Analytics Info request (UE list received from step 4, Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period NWDAF and also provide a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility and the UE's direction.

8-9. Based on the information provided by the AMF and NWDAF, the NEF can determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI to keep track the movement of the UE(s) in/out of the AOI.

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify AF of the given UE which is moving out the coverage.

* * * Next Change * * *

##### 4.15.13.3.3	UE Member Assistance with QoS filtering criteria for real-time QoS Monitoring

At the reception of Nnef_UEMemberSelectionAsistance_Subscribe request of step 1, NEF determines to do real-time QoS monitoring for determining the list of UEs that match or exceeds the QoS filtering criteria in real time. Unless the QoS flow to be monitored is associated with a default QoS flow, QoS Monitoring must have been activated in SMF for that QoS Flow before (e.g. by AF Session with required QoS) (see clause 4.15.4.5.1 for details).

Figure 4.15.13.3.3-1: 5GC assistance to UE member selection for real-time QoS monitoring

- AF subscribes the member selection assistance functionality by sending Nnef_UEMemberSelectionAssistance_subscribe request including its AF ID, the Application ID, optionally  S-NSSAI/DNN, and the QoS filtering criteria.

2.	NEF uses the AF ID to verify the authorization of the AF request and identifies which information needs to be collected and executed based on the QoS filtering criteria provided by the AF.

3.	If an S-NSSAI/DNN are not included in the request from the AF, the NEF derives the S-NSSAI and DNN to which this Application ID have access. NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination.

4.	For each target SMF, NEF sends an Nsmf_EventExposure_Subscribe request (Event: 'QoS monitoring', target UEs, optional Application ID, optional indication for default QoS flows monitoring, , S-NSSAI, DNN, Notification Target Address set to NEF, etc.). Alternatively, NEF may send an Nudm_EventExposure_Subscribe request to the UDM (not shown in Figure 4.15.13.3.3-1).

5.	The SMF may need to send an N4 Session Modification request to UPF for requesting the QoS monitoring for certain flows.

6.	UPF sends an Nupf_EventExposure_Notify request to NEF including an Event Exposure notification, according to the subscription received from SMF.

7.	Based on the Event Exposure reports received from the UPFs, NEF consolidates the received results and derives the list(s) of candidate UE(s) and additional information which fulfil the QoS filtering criteria requested by the AF request.

8.	NEF sends a Nnef_ UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * * Next Change * * *

##### 4.15.13.X	Specific procedure for the 5GC assistance to member selection based on the UE's current location, historical location and direction.

Figure 4.15.13.X-1: 5GC assistane to Member selection based on the UE's travel history, direction and separation distance

1.	AF requests 5GS assistance to support the UE selection by considering the UE's historical location, UE's current location and direction. AF includes the UE lists and the following criteria as part of the FL UE selection request:

-	UE historical location: The Target AoI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

-	UE current location: The current AoI which is the coverage area of the FL training server where the selected UEs located in to participate in the FL operation.

-	Direction: Select the UE with the different direction in the FL coverage Area.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the filtering criteria into the corresponding UE filtering information.

3-4. The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, the current AoI and event ID = Location report. AMF will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the current AoI.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility, AOI = Target AOI) with UE list received from step 4.

6-7. The NEF invokes NWDAF Analytics Info request (UE list received from step 4, Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period NWDAF and also provide a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility and the UE's direction.

8-9. Based on the information provided by the AMF and NWDAF, the NEF can determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI to keep track the movement of the UE(s) in/out of the AOI.

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify AF of the given UE which is moving out the coverage.

* * * End of Changes * * *