3GPP TSG SA WG-2 Meeting #157 	S2-2306694

Berlin, Germany, May 22 – 26, 2023

# Source:	Qualcomm Incorporated

# Title:	New SID on upper layer traffic steering, switching and split over dual 3GPP access

# Document for:	Approval

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on upper layer traffic steering, switching and split over dual 3GPP access

Acronym:	FS_DualSteer

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  | X |
| Don't know |  |  | X |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 960018 | Study on upper layer traffic steering, switching and split over dual 3GPP access | Stage 1 study for DualSteer in Rel-19 |

# 3	Justification

The Rel-16 to Rel-18 Access Traffic Steering, Switch and Splitting feature enables simultaneous communication between UE and UPF over multiple paths. By leveraging the simultaneous communication over multiple paths, the 5G system provides services with improved user experience, distributes the traffic across multiple accesses in a policy-based fashion, provides new high-data-rate services, etc.

However currently the feature requires a Multi-Access PDU Session to have one 3GPP access path and one non-3GPP access path, with the possibility to temporarily leverage two non-3GPP access paths in case of non-3GPP access path switch. As identified in the SA1 study on DualSteer (see TR 22.841), there are certain use cases in which it is necessary to apply traffic steering, switching and splitting between dual 3GPP access paths connected to the same or to different 3GPP networks. Use cases cover examples of diverse combinations of 3GPP access networks using the same or different RATs, including terrestrial NR plus NR or NR plus E-UTRA (e.g. using a combined EPC and 5GC), mix of terrestrial plus non-terrestrial NR, as well as dual NR non-terrestrial access (e.g. using same or different NTN orbits, e.g., GEO/MEO/LEO).

# 4	Objective

Based on the above justification, the objective of this study item is to study how to enhance the 5G system architecture to address how an MA PDU Session can simultaneously support two 3GPP access paths. The two 3GPP access paths can be in the same PLMN or in different 3GPP networks. In case of different 3GPP networks, the MA PDU Session has one 3GPP access path via a certain PLMN and the other 3GPP access path via another PLMN or SNPN, and the UE registers with the first PLMN and with the other PLMN/SNPN using the same 3GPP subscription credentials.

The two 3GPP access paths may use the same or different RATs, including the following cases:

-	terrestrial NR plus terrestrial NR;

-	terrestrial NR plus terrestrial E-UTRA (e.g. using a combined EPC and 5GC);

-	terrestrial NR plus non-terrestrial NR;

-	dual non-terrestrial NR (e.g. using same or different NTN orbits, e.g., GEO/MEO/LEO).

More in detail, in order to support traffic steering, switching and splitting for the above-mentioned type of MA PDU Session, the work tasks of this study are:

1)	How to select the second PLMN or SNPN in case of different 3GPP networks after an initial PLMN was selected for the UE.

2)	How to enhance registration and session management procedures.

3)	How to support mobility between a PDU Session with single 3GPP access and a MA PDU Session with dual 3GPP access as well as between MA PDU Sessions with dual 3GPP accesses.

4)	How to route the traffic of the MA PDU Session towards the PSA UPF.

5)	How to extend policies, steering functionalities and steering modes.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 |  |  |  |  |
| WT#2 |  |  |  |  |
| WT#3 |  |  |  |  |

Total TU estimates for the study phase:

Total TU estimates for the normative phase:

Total TU estimates: N + M =

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on upper layer traffic steering, switching and split over dual 3GPP access | TSG#XX Dec. 2024 | TSG#XX Mar. 2024 | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

-

# 9	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated

CableLabs

Lockheed Martin

Futurewei

Thales

Novamint

NEC

SyncTechno Inc.