TSG SA2 Meeting #157	S2-2306821

22 - 26 May 2023, Berlin, Germany

# Source:	CATT

# Title:	New SID on Study on Proximity-based Services in 5GS Phase 3

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Proximity-based Services in 5GS Phase 3

Acronym:	FS_5G_ProSe_Ph3

Unique identifier:	xxxxxx

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X | X | X |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| AIPN-SAE | SA1 | 320022 | Requirements for evolution of the 3GPP system architecture |
| SMARTER | SA1 | 720005 | New Services and Markets Technology Enablers |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 900007 | Proximity based Services in 5GS | Rel-17 Stage 2 5G_ProSe work item. |
| 970016 | Proximity-based Services in 5GS Phase 2 | Rel-18 Stage 2 5G_ProSe_Ph2 work item. |
| 941002 | NR sidelink relay enhancements | Rel-18 RAN work item on NR sidelink relay enhancements |
| 950024 | Study on Security Aspects of Proximity Based Services in 5GS Phase 2 | Rel-18 SA3 study item on security aspects. |

3	Justification

The 5G System has been enhanced to support Proximity Services in Release 17 and Release 18 as specified in TS 23.304, based on the service requirements defined in TS 22.278, TS 22.261 and TS 22.115. The following Proximity Services features are standardized within the Rel-17 and Rel-18 timeframe respectively.

Proximity Services features standardized within the Rel-17 timeframe:

-	5G System architecture reference models for Proximity Services.

-	Support of PC5 Direct Discovery (including in-coverage and out-of-coverage cases).

-	Support of PC5 unicast, groupcast and broadcast modes communication (including in-coverage and out-of-coverage cases).

-	Support of PC5 Service Authorization and Policy/Parameter Provisioning.

-	Support of direct communication path selection between PC5 and Uu.

-	Support of Charging for PC5.

-	Support of Layer-3 and Layer-2 based UE-to-Network Relay (including QoS and service continuity aspects).

Proximity Services features standardized within the Rel-18 timeframe:

-	Support single NR PC5 hop UE-to-UE Relay for unicast.

-	Enhance UE-to-Network Relay functionality to support:

-	Service continuity when switching between two indirect network communication paths for UE-to-Network Relay.

-	Service continuity when switching between direct network communication path and indirect network communication path for 5G ProSe Layer-2 UE-to-Network Relay, including inter-gNB indirect-to-direct and inter-gNB direct-to-indirect path switching.

-	Multi-path transmission using only one direct network communication path and only one indirect network communication path e.g. for improved reliability or data rates.

-	Emergency Services for Remote UE over UE-to-Network Relay.

-	Support path switching between direct NR Uu communication path and direct NR PC5 communication path.

However, due to lack of time the following objectives were de-prioritized from Rel-18 FS_5G_ProSe_Ph2 study item at the SA#94-e meeting:

-	Support of MBS traffic to Remote UE by UE-to-Network Relay.

-	Enhancement of UE-to-Network Relay for support of multiple NR PC5 hops.

-	Support of multiple NR PC5 hops UE-to-UE Relay.

-	Whether and how to support of non-3GPP RAT (e.g. Wi-Fi or Bluetooth) over PC5 reference point for Layer-3 UE-to-Network Relay.

There are also some requirements about Proximity Services captured in TS 22.278 and TS 22.261 not supported yet, e.g. multi-path transmission over multiple indirect paths, etc., which imposes architecture enhancements that need to be addressed by Rel-19 study item.

All these above items not standardized within Rel-17 and Rel-18 worth to be further investigated in the Rel-19 study item, in order to increasingly fulfil the service requirements defined in TS 22.278 and TS 22.261.

4	Objective

The study item aims at further investigating 5G System enhancements to support Proximity Services, based on what has been specified in Rel-17 and Rel-18, and based on the services requirements defined in TS 22.278, TS 22.261 and TS 22.115.

The detailed objectives are to investigate potential 5GS enhancements in order to support the followings:

WT#1	Enhance UE-to-Network Relay functionality to support:

WT#1.1	MBS traffic to Remote UE by UE-to-Network Relay.

WT#1.2	Multiple NR PC5 hops.

WT#1.3	Non-3GPP RAT (e.g. Wi-Fi or Bluetooth) over PC5 reference point for Layer-3 UE-to-Network Relay.

WT#1.4	Multi-path transmission over multiple indirect network communication paths.

WT#2	Enhance UE-to-UE Relay functionality to support:

WT#2.1	Multiple NR PC5 hops.

NOTE 1:	The UE-to-Network Relay and UE-to-UE Relay in the above WTs include both Layer-3 and Layer-2 Relays unless explicitly stated otherwise.

NOTE 2:	Whether and how objectives with RAN dependencies (as illustrated in the table below) will be supported need to be confirmed with RAN.

Architectural implications for RAN will be coordinated with RAN WGs.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |

Total TU estimates for the study phase:  TBD

Total TU estimates for the normative phase:  TBD

Total TU estimates: TBD

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on Proximity-based Services in 5GS Phase 3 | TBD | TBD | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA2.

8	Aspects that involve other WGs

SA3 for the Security aspects, SA5 for the Charging aspects, RAN for the RAN related issues.

9	Supporting Individual Members

Supporting IM name