3GPP TSG-WG SA2 Meeting #157	S2-2306870

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2306188)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4364 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Data Rate Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [CATT,] Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The data rate for a QoS Flow can be different types. One type of data rate is the measured traffic data rate of the QoS Flow, if there is no user traffic, there is no data rate (i.e. the measured traffic data rate = 0). Another type of data rate is the measured available data rate of the QoS Flow. Even if there is no user traffic, there is still available radio resource that can be used by the QoS Flow to transport the PDUs, in such case, the measured traffic data rate =0 but the measured available data rate can be used by the AF adaptively to change the XR stream codec below the measured available data rate. The NG-RAN and PSA UPF can measure the measured traffic data rate. But only the NG-RAN can measure the measured available data rate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the data rate into measured traffic UL and/or DL data rate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measured UL and/or DL data rate can not be used by the AF to perform the bitrate changing for the XRM service data flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | A different wording is suggested to avoid that parameter names become too long. It is also clarified over which time window the data rate is measured (following the approach used for the Session-AMBR). Further details of the NG-RAN based measurement are added. |  |  |  |  |  |  |  |  |

## FIRST CHANGE

### 5.45.4	Data rate monitoring

The data rate is the measured UL and/or DL data rate of the traffic transferred by a Non-GBR or GBR QoS Flow. The data rate is measured over a data rate averaging window which is a standardized value.

Either the NG-RAN or the UPF  can perform the measurement of the UL and/or DL data rate.

According to the QoS Monitoring request for UL and/or DL data rate, the SMF decides whether the UPF or the NG-RAN is required to perform data rate measurement for the QoS Flow and instructs the UPF or the NG-RAN accordingly based on the requirements of the QoS Monitoring request. If the NG-RAN is selected to perform data rate measurement for the QoS Flow, the SMF shall also instruct the UPF to detect the monitoring results in the UL packets of that QoS Flow and to forward them accordingly.

If the NG-RAN is instructed to perform data rate measurement, the NG-RAN sends the monitoring results in the GTP-U header of the UL packets of the QoS Flow or in a dummy UL packet (in case there is no UL packet available when the monitoring result is to be sent).

Editor's note:	It is pending for RAN WGs to confirm whether providing measured data rate information for QoS Flow to the CN can be included or not in Release-18.

## END of CHANGES