3GPP TSG-SA2 Meeting #157	S2-2307024

# Berlin, Germany, May 22 - 26, 2023												 (revision of S2-2305594)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0356 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Location Service Continuity between EPS and 5GS for Immediate Location Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Three issues are discussed and the corresponding changes are proposed in this CR: - How to support the location service continuity for immediate location request in the interworking architecture without N26 interface. - How to support service continuity for MO-LR procedure. Issue#1: How to support the location service continuity for immediate location request in the interworking architecture without N26 interface location service continuity for LIR in the interworking architecture without N26 interface To support location service continuity between EPS and 5GS for Immediate Location Request, the clause 6.19.1 and clause 6.19.2 are added in TS 23.273: - Based on the step 3 in the procedure in clause 6.19.1, the 5GS to EPS handover in clause 4.11.1.2.1 in TS 23.502 is referred. - Based on the step 2 in the procedure in clause 6.19.2, the EPS to 5GS handover in Fig 4.11.1.2.2.2-1 in TS 23.502 is referred. Because the procedures referred above only apply to the N26 based interworking architecture. So it is not clear how to support the location service continuity for LIR in the interworking architecture without N26 interface. In the interworking architecture without N26 interface, the current interworking mechanism is as follows: - 5GS to EPS mobility: Based on the step 10 of the procedure in clause 4.11.2.2, the MME sends an Update Location Request to the HSS+UDM indicating that registration of an AMF at the HSS+UDM, if any, shall not be cancelled. The HSS+UDM does not send Nudm_UECM_DeregistrationNotification to the old AMF. The UE does not maintain registration in 5GC, upon reachability time-out, the AMF implicitly detach the UE. - EPS to 5GS mobility: Based on the step 5 of the procedure in clause 4.11.2.3, the AMF sends an Nudm_UECM_Registration to the HSS+UDM indicating that registration of an MME at the HSS+UDM, if any, shall not be cancelled. The HSS+UDM does not send cancel location to the old MME. The UE does not maintain registration in EPC, upon reachability time-out, the MME implicitly detach the UE. Based on the current interworking mechanism above, if there is an LIR on-going during the interworking procedure, before the UE detaches/de-registers from the source side, the LIR is already failed because time-out of the location request. To support the location service continuity for LIR in the interworking architecture without N26 interface, it is proposed that the GMLC subscribes the CN Type change event notifiy from HSS+UDM, if the event report is received, the GMLC tries to query the HSS+UDM for the new MM NF address when the LIR is failed in the source side. If a new address is received, the GMLC restarts the LIR to the MM NF. Issue#2: How to support service continuity for MO-LR procedure. The MO-LR procedure supports three functions, i.e. UE requests it’s location, UE requests assistance data, UE requests to send it’s location to LCS Client/AF. For the first two functions, the GMLC is not involved in the MO-LR procedure, so there is no need for AMF/MME to notify the HO event to 5GC-GMLC/EPC-GMLC as specified in clause 6.19.1 and clause 6.19.2. Considering the MO-LR is triggered by UE and the UE knows the interworking is on-going, so it is proposed that the UE re-starts the MO-LR after the interworking procedures if needed. Reason for change for revision 2: Revision 2 is proposed to solve two issues as discussed in S2-230xxxx. Issue#1: description confliction in TS 23.273 The description of the procedure in clause 6.19.1.y conflicts with the descriptio in clause 4.2a. To solve the confliction, it is proposed to update the procedure above to align with the description in clause 4.2a. Issue#2: consistent location service continuity procedure for immediate location request To reduce implementation complexity, it is proposed to use a consistent procedure to support the location service continuity procedure for immediate location request, no matter whether N26 interface is deployed by operator or not. Because the proceudres in clause 6.19.1.x and clause 6.19.1.y can apply to different deployment scenarios, it is proposed to only keep the procedures in TS 23.273. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the location service continuity mechanism for immediate location request in the interworking architecture without N26 interface. - Clarify the UE re-starts the MO-LR after the interworking procedure. For revision 2: update the procedure in clause 6.19.1.y to align with the description in clause 4.2a and unify the location service continuity procedure for immediate location request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.19.1, 6.19.1.1, 6.19.1.2, 6.19.1.x (new), 6.19.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.19.1	Location Service Continuity for Immediate Location Request

6.19.1.x	Location Service Continuity from 5GS to EPS

Figure 6.19.1.x-1: Location Service Continuity from 5GS to EPS

1.	Target UE positioning is started in 5GS, e.g. steps 1-12 of 5GC-MT-LR procedure in clause 6.1.2, or steps 1-5 of 5GC-MO-LR procedure in clause 6.2.

2.	[Conditional] If the 5GC-GMLC sends the message in step 4 or step 5 in Figure 6.1.2-1 in TS 23.273, 5GC-GMLC invokes the Nudm_EventExposure_Subscribe to HSS+UDM to subscribe the CN Type change event report.

3.	LMF handles the ongoing LCS session.

4.	5GS to EPS mobility happens successfully as described in Figure 4.11.2.2-1 of TS 23.502 [19].

5.	[Conditional] If step 2 is performed, the UDM detects that the CN Type of UE changes, the UDM notifies the CN Type of UE to 5GC-GMLC by invoking the Nudm_EventExposure_Notify service operation.

6a.	The LMF invokes Nlmf_Location_DetermineLocation Response including failure.

6b.	The AMF invokes Namf_Location_ProvidePositioningInfo Response including failure.

7.	The steps 8~ 11 in the location service continuity from 5GS to EPS with N26 interface for immediate location request in clause 6.19.1.1 in TS 23.273 are performed in the case of MT-LR. In the case of MO-LR, the UE restarts the EPC-MO-LR procedure.

NOTE:	The 5GC-MT-LR in 5GS may be failed because of timeout of the location request. The 5GC-GMLC can trigger the step 6 when the LIR in 5GS is failed.

8.	[Conditional] If step 2 is performed, 5GC-GMLC invokes the Nudm_EventExposure_UnSubscribe to HSS+UDM to cancel the subscription of the CN Type change event report.

6.19.1.y	Location Service Continuity from EPS to 5GS

Figure 6.19.1.y-1: Location Service Continuity from EPS to 5GS

1.	Target UE is registered in EPC, target UE triggers EPC-MO-LR procedure (i.e. steps 1-3 of EPC-MO-LR procedure in clause 9.2.6 in TS 23.271[4] are performed) or LCS Client/AF requests target UE location (i.e. steps 1-3 of 5GC-MT-LR procedure in clause 6.1.2 are performed).

2.	[Conditional] If LCS Client/AF requests target UE location, the 5GC-GMLC sends LCS Service Request to EPC-GMLC when 5GC-GMLC obtains the MME address from HSS+UDM. The 5GC-GMLC derives the address of EPC-GMLC by using the mechanisms described in step 2 in clause 6.13.1.

3.	[Conditional] If the step 2 is performed, Target UE positioning is started in EPS, e.g. steps 1-6 of EPC-MT-LR procedure in clause 9.1.15 in TS 23.271[4].

4.	[Conditional] If the step 2 is performed, the 5GC-GMLC invokes the Nudm_EventExposure_Subscribe to HSS+UDM to subscribe the CN Type change event report.

5.	E-SMLC handles the ongoing LCS session.

6.	EPS to 5GS mobility happens successfully as described in clause 4.11.2.3 of TS 23.502 [19].

7.	[Conditional] If step 4 is performed, the UDM detects that the CN Type of UE changes, the UDM notifies the CN Type of UE to 5GC-GMLC by invoking the Nudm_EventExposure_Notify service operation.

8.	In the case of MT-LR, 5GC-GMLC triggers LCS Session over 5GS, i.e. steps 3-24 of 5GC-MT-LR procedure in clause 6.1.2 are performed. In the case of MO-LR, the UE triggers LCS Session over 5GS, i.e. steps 1-13 of 5GC-MO-LR procedure in clause 6.2 are performed.

NOTE:	The EPC-MT-LR in EPS is failed because of timeout of the location request. The EPC-GMLC sends LCS Service Response with failure indication to 5GC-GMLC.

9.	[Conditional] If step 4 is performed, 5GC-GMLC invokes the Nudm_EventExposure_UnSubscribe to HSS+UDM to cancel the subscription of the CN Type change event report.

* * * End of Change * * *