3GPP TSG-WG SA2 Meeting #157	S2-2307101

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4652 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice Replacement in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is proposed to update clause 5.15.19 by adding some descriptions of Network Slice Replacement in roaming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Network Slice Replacement in roaming |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of Network Slice Replacement in roaming is not captured in the TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4219 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.15.19	Support of Network Slice Replacement

The Network Slice Replacement feature is used to replace an S-NSSAI with an Alternative S-NSSAI when an S-NSSAI becomes unavailable or congested. The Network Slice Replacement may be triggered in the following cases:

-	If the NSSF detects that an S-NSSAI becomes unavailable or congested (e.g. based on OAM or NWDAF analytics output), it sends network slice availability notification for the S-NSSAI to the AMF. The notification may include an Alternative S-NSSAI which can be used by the AMF to replace the S-NSSAI. The NSSF notifies the AMF when the S-NSSAI is available again.

-	If the PCF detects that an S-NSSAI becomes unavailable or congested for a UE (e.g. based on OAM or NWDAF analytics output), it sends access and mobility related policy notification to the AMF. The notification may include an Alternative S-NSSAI which can be used by the AMF to replace the S-NSSAI. The PCF notifies the AMF when the S-NSSAI is available again for the UE.

-	The OAM sends notification to AMF when an S-NSSAI becomes unavailable or congested (and also when this S-NSSAI becomes available again) and provides the Alternative S-NSSAI to AMF.

Based on the notification above from NSSF or PCF or OAM, the AMF may determine that an S-NSSAI is to be replaced with Alternative S-NSSAI. For roaming case, the AMF may receive network slice availability notification of the HPLMN S-NSSAI from NSSF in the HPLMN via NSSF in VPLMN, to trigger the Network Slice Replacement of the HPLMN S-NSSAI as described in clause 5.15.6.

NOTE 1:	It is recommended that, the operator configures to use only one mechanism when triggering the Network Slice Replacement for S-NSSAI.

The AMF determines the Alternative S-NSSAI for a UE registered with the S-NSSAI based on the notification from NSSF or PCF, or based on local configuration if the NSSF or PCF do not provide an alternative S-NSSAI. The Alternative S-NSSAI shall be supported in the UE Registration Area. If AMF cannot determine the Alternative S-NSSAI for the S-NSSAI, e.g. PCF or NSSF doesn't provide Alternative S-NSSAI, the AMF may further interact with the PCF to determine the Alternative S-NSSAI. The event trigger in AMF for interacting with PCF is described in clause 6.1.2.5 of TS 23.503 [45].

The UE indicates the support of Network Slice Replacement feature during the UE Registration procedure. For supporting UE in CM-CONNECTED mode, and if there is a PDU Sessions in the UE context associated with the S-NSSAI that needs to be replaced, the AMF provides the Alternative S-NSSAI for this S-NSSAI in the Allowed NSSAI and in the Configured NSSAI, if not included yet, and the mapping between S-NSSAI(s) to Alternative S-NSSAI(s) to the UE in UE Configuration Update message as follows:

-	for non-roaming UEs, the AMF provides the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE.

-	for roaming UEs when the VPLMN S-NSSAI has to be replaced by a VPLMN Alternative S-NSSAI, the AMF provides the mapping of the VPLMN S-NSSAI to the Alternative VPLMN S-NSSAI to the UE.

-	for roaming UEs when the HPLMN S-NSSAI has to be replaced by an Alternative HPLMN S-NSSAI, the AMF provides the mapping of the HPLMN S-NSSAI to the Alternative HPLMN S-NSSAI to the UE.

NOTE 2:	The Alternative S-NSSAI or Alternative HPLMN S-NSSAI may be part or not part of the Subscribed S-NSSAI.

For the supporting UE in CM-IDLE state, when the UE establishes a NAS signalling connection, e.g. through a Service Request procedure or through a UE registration procedure, if the AMF determines that the S-NSSAI is to be replaced and there is a PDU Session associated with the S-NSSAI in the UE context, the AMF sends the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE in the UE Configuration Update message or in the Registration Accept message.

Editor's note:	Whether to send the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE when there is no PDU session associated with the S-NSSAI is FFS.

In case of non-roaming scenario, during a new PDU Session establishment procedure towards an S-NSSAI,

-	if the UE is provided with the mapping of the S-NSSAI to an Alternative S-NSSAI, the UE provides both the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message. When the AMF receives the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message, or when the AMF receives only the S-NSSAI in PDU Session Establishment message the AMF determines that the S-NSSAI is to be replaced with the Alternative S-NSSAI, the AMF includes both the Alternative S-NSSAI and the S-NSSAI to the SMF.

-	if the UE is not provided with the mapping of the S-NSSAI to the Alternative S-NSSAI, the UE provides the S-NSSAI in the PDU Session Establishment message. When the AMF determines that the requested S-NSSAI is to be replaced with the Alternative S-NSSAI and if the UE supports Network Slice Replacement, the AMF performs UE Configuration Update procedure to reconfigure the UE with the Alternative S-NSSAI. The AMF continues the PDU Session establishment procedure with the Alternative S-NSSAI and provides both the Alternative S-NSSAI and the S-NSSAI to the SMF.

In case of non-roaming scenario, the SMF retrieves the Session Management Subscription data using the S-NSSAI to be replaced from UDM and proceeds with the PDU Session establishment using the Alternative S-NSSAI (e.g. UPF selection). In addition, the SMF registers with the UDM using the S-NSSAI to be replaced. The SMF sends the Alternative S-NSSAI to NG-RAN in N2 SM information and to UE in PDU Session Establishment Accept message.

In case of non-roaming scenario, for existing PDU Session associated with an S-NSSAI that is replaced with the Alternative S-NSSAI, after the AMF sends mapping of the S-NSSAI to the Alternative S-NSSAI to the supporting UE in UE Configuration Update message, the AMF sends updates to the SMF of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to Alternative S-NSSAI and includes the Alternative S-NSSAI as follows (see details in clause 4.3.5.x of TS 23.502 [3]):

-	If the SMF determines that the PDU Session needs to be retained (e.g. if the anchor UPF can be reused with the alternative S-NSSAI and SSC mode 1), the SMF sends the Alternative S-NSSAI to the UPF in the N4 message, to the NG-RAN in N2 message and to the supporting UE in PDU Session Modification Command message.

-	If the SMF determines that the PDU Session needs to be re-established, the SMF sends the Alternative S-NSSAI to the supporting UE either in PDU Session Modification Command if the PDU Session is of SSC mode 3, or in PDU Session Release if the PDU Session is of SSC mode 2 or SSC mode 1, to trigger the re-establishment of the PDU Session. The UE includes both, the S-NSSAI and the Alternative S-NSSAI in the PDU Session Establishment message.

Editor's note:	How to use the existing NG-RAN resource of the replaced S-NSSAI is for FFS.

In case of roaming scenario, during a new PDU Session establishment procedure towards an S-NSSAI,

-	When the VPLMN S-NSSAI has to be replaced by a VPLMN Alternative S-NSSAI, the following applies:

-	if the UE is provided with the mapping of the VPLMN S-NSSAI to a VPLMN Alternative S-NSSAI, the UE provides both the VPLMN Alternative S-NSSAI and the VPLMN S-NSSAI in the PDU Session Establishment message. When the AMF receives the VPLMN Alternative S-NSSAI and the VPLMN S-NSSAI in the PDU Session Establishment message, or when the AMF receives only the VPLMN S-NSSAI in PDU Session Establishment message the AMF determines that the VPLMN S-NSSAI is to be replaced with the VPLMN Alternative S-NSSAI, the AMF includes both the VPLMN Alternative S-NSSAI and the VPLMN S-NSSAI to the SMF in the VPLMN.

-	if the UE is not provided with the mapping of the VPLMN S-NSSAI to the VPLMN Alternative S-NSSAI, the UE provides the VPLMN S-NSSAI in the PDU Session Establishment message. When the AMF determines that the requested VPLMN S-NSSAI is to be replaced with the VPLMN Alternative S-NSSAI and if the UE supports Network Slice Replacement, the AMF performs UE Configuration Update procedure to reconfigure the UE with the VPLMN Alternative S-NSSAI. The AMF continues the PDU Session establishment procedure with the VPLMN Alternative S-NSSAI and provides both the VPLMN Alternative S-NSSAI and the VPLMN S-NSSAI to the SMF in the VPLMN.

-	Then the SMF in the VPLMN retrieves the Session Management Subscription data using the S-NSSAI to be replaced from UDM and proceeds with the PDU Session establishment using the Alternative S-NSSAI (e.g. UPF selection). In addition, the SMF registers with the UDM using the S-NSSAI to be replaced. The SMF in the VPLMN proceeds with the PDU Session establishment using the VPLMN Alternative S-NSSAI. The SMF in the VPLMN sends the VPLMN Alternative S-NSSAI to NG-RAN in N2 SM information and to UE in PDU Session Establishment Accept message.

-	When the HPLMN S-NSSAI has to be replaced by a HPLMN Alternative S-NSSAI, the following applies:

-	if the UE is provided with the mapping of the HPLMN S-NSSAI to a HPLMN Alternative S-NSSAI, the UE provides both the HPLMN Alternative S-NSSAI and the HPLMN S-NSSAI in the PDU Session Establishment message. When the AMF receives the HPLMN Alternative S-NSSAI and the HPLMN S-NSSAI in the PDU Session Establishment message, or when the AMF receives only the HPLMN S-NSSAI in PDU Session Establishment message the AMF determines that the HPLMN S-NSSAI is to be replaced with the HPLMN Alternative S-NSSAI, the AMF includes both the HPLMN Alternative S-NSSAI and the HPLMN S-NSSAI to the SMF in the VPLMN.

-	if the UE is not provided with the mapping of the HPLMN S-NSSAI to the HPLMN Alternative S-NSSAI, the UE provides the HPLMN S-NSSAI in the PDU Session Establishment message. When the AMF determines that the requested HPLMN S-NSSAI is to be replaced with the HPLMN Alternative S-NSSAI and if the UE supports Network Slice Replacement, the AMF performs UE Configuration Update procedure to reconfigure the UE with the HPLMN Alternative S-NSSAI. The AMF continues the PDU Session establishment procedure with the HPLMN Alternative S-NSSAI and provides both the HPLMN Alternative S-NSSAI and the HPLMN S-NSSAI to the SMF in the VPLMN.

-	In case of LBO, the SMF in the VPLMN retrieves the Session Management Subscription data using the S-NSSAI to be replaced from UDM and proceeds with the PDU Session establishment using the Alternative S-NSSAI (e.g. UPF selection). In addition, the SMF registers with the UDM using the S-NSSAI to be replaced. The SMF in the VPLMN proceeds with the PDU Session establishment using the VPLMN Alternative S-NSSAI. The SMF in the VPLMN sends the VPLMN Alternative S-NSSAI to NG-RAN in N2 SM information and to UE in PDU Session Establishment Accept message.

-	 In case of HR, the SMF in the VPLMN sends HPLMN S-NSSAI and HPLMN Alternative S-NSSAI to the SMF in the HPLMN. The SMF in the HPLMN proceeds with the PDU Session establishment using the HPLMN Alternative S-NSSAI. The SMF in the VPLMN sends HPLMN Alternative S-NSSAI to UE in PDU Session Establishment Accept message.

In case of roaming scenario, for existing PDU Session associated with an S-NSSAI that is replaced with the Alternative S-NSSAI, after the AMF sends mapping of the S-NSSAI to the Alternative S-NSSAI to the supporting UE in UE Configuration Update message.

-	When the VPLMN S-NSSAI has to be replaced by a VPLMN Alternative S-NSSAI, the AMF sends updates to the SMF in the VPLMN of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to VPLMN Alternative S-NSSAI and includes the VPLMN Alternative S-NSSAI:

-	In case of LBO, the SMF in the VPLMN determines whether the PDU Session needs to be retained. If yes, the SMF in the VPLMN sends the VPLMN Alternative S-NSSAI to the UPF in the VPLMN in the N4 message, to the NG-RAN in N2 message and to the supporting UE in PDU Session Modification Command message.

-	When the HPLMN S-NSSAI has to be replaced by a HPLMN Alternative S-NSSAI, the AMF sends updates to the SMF in the VPLMN of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to HPLMN Alternative S-NSSAI and includes the HPLMN Alternative S-NSSAI:

-	In case of LBO, the SMF in the VPLMN sends the HPLMN Alternative S-NSSAI to UE in PDU Session Modification Command message.

-	 In case of HR, the SMF in the VPLMN sends HPLMN Alternative S-NSSAI to the SMF in the HPLMN. The SMF in the HPLMN determines whether the PDU Session needs to be retained. If yes, the SMF in the HPLMN sends the HPLMN Alternative S-NSSAI to the UPF in the HPLMN in the N4 message. The SMF in the VPLMN sends the HPLMN Alternative S-NSSAI to UE in PDU Session Modification Command message.

When the AMF is notified that the S-NSSAI is available again or the congestion of the S-NSSAI has been mitigated, if the AMF has configured the supporting UE with the Alternative S-NSSAI, and the AMF determines for the UE to use the S-NSSAI again, the AMF reconfigures the supporting UE (e.g. by using UE Configuration Update message) to use the S-NSSAI. If there is an existing PDU Session associated with the Alternative S-NSSAI, the AMF sends updates to the SMF of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to the S-NSSAI.

Editor's note:	How to perform Network Slice Replacement during handover is FFS.

# * * * * End of changes * * * *