3GPP TSG-WG SA2 Meeting #157 	S2-2307109

May 22 – 26, 2023, Berlin	(revision of S2-230xxxx)

Source:	Sony

Title:	Update Network based SL Positioning for UE with NAS connection.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: Based on the CR for TS 23.273 it is clear what UE_ID that is provided to the LMF. This paper addresses this part of a remaining Editor’s note.

# 1. Introduction/Discussion

Based on the CR for TS 23.273 it is clear what UE_ID that is provided to the LMF. This paper addresses this part of a remaining Editor’s note.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change* * * *

### 5.5.2	Network based SL Positioning for UE with NAS connection

A UE with a NAS connection is in RM-REGISTERED state. The UE can enter CM-Connected state by performing UE triggered Service Request for 5GC-MO-LR/SL-MO-LR or performing Network triggered Service Request for 5GC-NI-LR or 5GC-MT-LR/SL-MT-LR. As the Target UE can establish a NAS signalling connection with the AMF, the functionality specified in TS 23.273 [8] for location services can be reused including e.g. 5GC-MO-LR, 5GC-MT-LR and 5GC-NI-LR with the additional functionality captured in this clause.

NOTE 1:	Target UE can establish a NAS signalling connection directly or indirectly via a ProSe L2 UE-to-Network Relay.

-	The Location Service is triggered via the AMF serving the Target UE. The location request comes either from an AF/external client via the GMLC, a 5G NF, or the Target UE.

-	The LTE positioning protocol (LPP) as specified in TS 37.355 [9] is used between the Target UE and the LMF. When LCS procedures are used to estimate the location of the Located UE, LPP is used between Located UE and LMF.

Editor's note:	RAN WGs will determine whether and what enhancements and the subset functionalities of LPP are needed to support Network based SL positioning including an 5GC-MT-LR, 5GC-MO-LR and 5GC-NI-LR.

-	Either the Target UE or LMF determines if UE Positioning using SL positioning will be applied, if 5GC-MO-LR/5GC-MT-LR service is triggered. When LMF determines that UE Positioning using SL positioning is used, LMF may trigger the Target UE to perform the discovery of Located UE(s).

-	The Target UE discovers Located UE(s) for UE Positioning using SL positioning. Optionally, to assist the Target UE the LMF may provide to the Target UE a list of candidate Located UE(s). In this case the LMF maintains the candidate list of Located UE(s) including e.g. the capability and location information of the Located UE(s). The Target UE selects the Located UE(s) and reports the UE identity of the selected Located UE(s) to LMF.

-	The Target UE and Located UE(s) perform ranging/SL positioning. The Target UE includes the UE identity i.e, the application layer 2 ID and GPSI of the Located UE(s) to the LMF together with the Ranging measurement data or result. The LMF may interact with GMLC to get the location of Located UE.

Editor's note:	 The security impact will be coordinated with SA WG3.

-	The LMF uses the location of Located UE(s) together with the ranging/SL positioning measurement data or results reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE.

-	In case the Located UE is a non-stationary UE, the LMF interacts with GMLC a required QoS to get the location of Located UE. The ranging/SL positioning and the positioning of the Located UE(s) can be scheduled with the same time using the scheduled location time, as specified in TS 23.273 [8], to improve the Target UE positioning accuracy.

NOTE 2:	The LMF may need to compensate the time difference of the positioning of Target UE and Located UE with additional information, e.g. velocity.

-	The LMF provides the UE location result to the Target UE over LPP, or to the AMF, and AMF forwards the UE location to the GMLC, or to the 5GC NF according to legacy functionality.

# * * * * End of changes* * * *