3GPP TSG-WG SA2 Meeting #157 	S2-2307110

May 22 – 26, 2023, Berlin	(revision of S2-230xxxx)

Source:	Sony

Title:	Update Network based SL positioning for UE without NAS connection.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: One of the sub-bullets includes two sperate functions. This paper proposes to split them in two and remove one remaining Editor’s note.

# 1. Introduction/Discussion

One of the sub-bullets includes two sperate functions. This paper proposes to split them in two and remove one remaining Editor’s note.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change* * * *

### 5.5.3	Network based SL positioning for UE without NAS connection

When Target UE cannot establish the NAS connection with AMF due to the Target UE being out of coverage and at least one of the discovered Located UEs can establish NAS connection for 5GC-MO-LR, the following principles are applied:

NOTE:	5GC-MT-LR is not applicable in this release of the specification.

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE and Located UE(s) perform ranging/SL positioning.

-	The Target UE selects at least one of the Located UEs that can use 5GC-MO-LR service for SL Positioning.

-	The Target UE may transmit its ranging measurements and/orresults to the Located UE(s).

-	Each Located UE that can establish the NAS connection may report the ranging/SL positioning measurement or result to its LMF via the serving AMF of the Located UE. This may include ranging measurements and/or results received from the Target UE. The endpoints for LPP messages are the LMF and the Located UE(s).

-	The LMF may use the received information (i.e., ranging/SL positioning measurement or result) to calculate the location of the Target UE, and provide the resulting location via the Located UE to the Target UE.

-	The in case the LMF has the Target UE context it may use the received information (i.e. ranging/SL positioning measurement and/or result and deferred routing identifier) to calculate the location of the Target UE, and provide the resulting location via the GMLC to the LCS Client or AF as specified in TS 23.273 [8] clause 6.3. In case the UE context was deleted, then this functionality cannot be performed.

-	If a LMF is aware of multiple Located UEs associated with the same Target UE, it may perform the calculation using all information from these Located UEs.

Editor's note:	It is FFS if an enhanced 5GC-MO-LR message is sent by the Located UE.

Editor's note:	Security aspects require confirmation from SA WG3.

# * * * * End of changes* * * *