3GPP SA WG2-Meeting #157	S2-2307115

Berlin, Germany, May 22-26, 2023	(revision of S2-230xxxx)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New WID on 5G support for Femto (HgNB)

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on 5G support for Femto (HgNB)

Acronym:	FS_5GSF

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| tbd |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830042 | Vertical_LAN | CAG introduction |
| 900015 | Enhanced support of Non-Public Networks | CAG enhancements |

# 3	Justification

SA1 has defined normative requirements for Premises Radio Access Station (PRAS) as part of the PIRates WID (see TS 22.261, clause 6.38). This study item defines the overall architecture and required functional and procedural impacts for supporting deployments of 5G Femto with focus on enabling access control for “5G Femto” use cases. 5G Femto or Home gNB or HgNB is analogous to the LTE concept for Home eNodeB(s) with e.g., NR capable base stations deployed at home or at enterprise premises. Following are the justifications for the study objectives:

-	Concept of a 5G “HgNB” similar to “H(e)NB” from 3G/LTE times is currently not explicitly specified in 3GPP standards.

-	5G Femto offers a cost-effective way to improve 5G indoor coverage, offload macro gNB network traffic, enable better voice quality, and better support for Enterprise mobility.

-	5G Femto extends also coverage using higher frequency bands, allowing end customers to deploy HgNB on their own. This leads to efficient and effective usage of higher frequency spectrum.

-	High number of mobile sessions are indoor and inside coverage with 5G mid and high-bands is limited. Need for a solution that enables simple end user plug and play and allowing for customized access control (by specifying access control using CAG concept).

-	High bandwidth and throughput with 5G are required at home and at campus locations to enable new immersive applications such as AR/VR/MR gaming, e-sports, UHD 8K video, telepresence, etc.

Access control for 5G Femto can leverage the CAG concept defined for PNI-NPN. Close cooperation with RAN groups is required.

# 4	Objective

The following aspects will be studied:

1.	Study how to define the overall architecture and required functional and procedural impacts for supporting 5G Femto deployments.

2.	Study how to define the 5G Femto access control mechanism based on the existing CAG concept.

NOTE 1:	RAN internal architecture aspects need to be studied in RAN WGs. Cooperation with RAN WGs is required, e.g., regarding potential enhancements to the RAN-Core interface.

3.	Study how to enable provisioning of subscribers allowed to access 5G Femto cells (such as using a function like CSG Subscriber Server (CSS) specified for EPS) and how to manage 5G Femto access control by the CAG owner or an authorized administrator.

NOTE 2:	Possible enhancements to the existing CAG concept may require cooperation with RAN WGs.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or it depends on the completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 |  |  | Yes | self-contained |
| WT#2 |  |  | Maybe | Depends on outcome of WT#1 |
| WT#3 |  |  | Maybe | Depends on outcome of WT#1 |

Total TU estimates for the study phase:

Total TU estimates for the normative phase:

Total TU estimates:

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | TR 23.xxx | Study on support for 5G Femto (HgNB) | SA#xx | SA#yy | Nokia |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Rainer Liebhart, Nokia, rainer dot liebhart at nokia dot com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

RAN WGs (RAN2, RAN3) for RAN aspects.

SA3 for security aspects.

SA5 for OAM and charging aspects.

CT WGs for stage 3.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Verizon

Casa Systems

Oracle

BT

MATRIXX Software

SK Telecom

AT&T

Samsung