3GPP TSG-SA2 Meeting #157	 S2-2307140

Berlin Germany, May 22-26, 20223 											(revision of S2-2306046)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4236 | Rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving open issues for Network Slices with Network Slice Area of Service not matching deployed TAs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson, Apple],Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following are the open issues as per the Editor’s notes: Editor's note: It is FFS whether to enable a per UE differentiation of Network Slice availability. As there is a need to finalize stage 2 in Q2 2023, it is proposed to not enable a per UE differentiation in Rel-18. There are missing requirements on per UE NS-AoS. Editor's note: How to steer the UE to the area of the Location information is FFS. If the S-NSSAI is registered, then the NG-RAN gets the S-NSSAI in some information e.g. Allowed NSSAI or Partially Allowed NSSAI (dependent on KI#5 progress) and also the RFSP can be set to a suitable value. RRM can consider the case of target cells declaring that zero resources allocated for the cell over Xn. In idle mode the slice-based cell re-selection can consider the S-NSSAI of Partially Allowed NSSAI as already agreed. If the S-NSSAI is not yet registered or the RFSP of Allowed NSSAI has not been considering the S-NSSAI, then AMF can create a Target NSSAI including the S-NSSAI as to enable the NG-RAN to try and steer the UE to cells supporting the S-NSSAI and having resources allocated for the S-NSSAI. Editor's note: Whether the AMF or SMF rejecting the PDU Session establishment is FFS. If AMF enforces the NS-AoS the AMF can also reject the UE request in a similar way as done for S-NSSAI based congestion control i.e. in which case the AMF provides a NAS Transport Error message for the NAS Transport message carrying the SM message, and in the NAS Transport Error message it includes a back-off timer. Editor's note: It is FFS whether it is suitable to remove the S-NSSAI from the Configured NSSAI/Allowed NSSAI or whether to release the PDU Session when the non-supporting UE is outside the NS-AoS. For non-supporting UEs, the logic should enable the UE to not use the S-NSSAI for user data outside the NS-AoS while enable the UE to use the S-NSSAI for user data inside the NS-AoS. It is proposed to leave it to AMF implementation or configuration whether to trigger release of the PDU Session or also remove the S-NSSAI from Allowed NSSAI/Configured NSSAI. Rev2 provides updates to solve the handling of Allowed NSSAI and the handling of connected mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the Editor’s notes, Editor's note: The need for Registration level control is FFS e.g. requiring S-NSSAI to be deregistered when the UE move out of the Location. Editor's note: Logic for when the S-NSSAI is in the Allowed NSSAI is FFS. And provide handling of allowed NSSAI and how to handle the connected mode UEs whether supporting the feature or not.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification includes unresolved open issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.18.2, 5.15.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.15.18	Support for Network Slices with Network Slice Area of Service not matching deployed Tracking Areas

#### 5.15.18.1	General

The network support for a Network Slice is defined on a per Tracking Area granularity. It may be beneficial to deploy some Network Slices such that the Network Slice have a limited geographical availability that is not matching existing Tracking Area boundaries.

The operator can in this case decide to change the topology of the Tracking Areas so they match the boundaries of the Network Slice, or the operator may configure resources for the Network Slices in the cells of TAs where the Network Slices are to be available, and in areas of the TAs where the network slice is defined to be not available the cells are configured with zero resources.

The AMF receives from the OAM the information on availability of a network slice when the granularity is smaller than TA, i.e. if the NS-AoS includes TAs where the network slice is not available in some cells of the TA.

In order to optimize the end-to-end behaviour, the AMF can, based on NS-AoS information received from OAM, configure supporting UEs with S-NSSAI location availability information, and the network may need to monitor the S-NSSAI usage and take decisions on whether to release, deactivate, or reactivate PDU sessions to enforce the NS-AoS in CM-CONNECTED mode including in the case the UE does not support the S-NSSAI location availability information.

#### 5.15.18.2	S-NSSAI location availability information

S-NSSAI location availability information defines additional restrictions to the usage of an S-NSSAI in TAs where the Network Slice availability does not match the TA boundaries. The AMF is configured per S-NSSAI whether to send the S-NSSAI location availability information to supporting UEs.

The S-NSSAI location availability information sent to the UE includes, for each applicable S-NSSAI of the Configured NSSAI, Location information indicating the cells of TAs in the RA where the related S-NSSAI is available if the S-NSSAI is not available in all the cells of the TA.

If the UE has indicated that the UE supports S-NSSAI location availability information in the 5GMM Core Network Capability (see clause 5.4.4a), the AMF may, based on OAM configuration, configure the UE with S-NSSAI location availability information for one or more S-NSSAIs when the AMF allocates an RA where the Network Slice availability does not match whole TAs, by including the S-NSSAI location availability information in the Registration Accept message or the UE Configuration Command message. A UE that receives S-NSSAI location availability information applies the information as follows.

1.	If the S-NSSAI is rejected in the RA or rejected partially in the RA or rejected with a cause code that allows attempting to register the S-NSSAI again, the UE can request the S-NSSAI only if the S-NSSAI location availability information indicates that the S-NSSAI is available at the cell where the UE is camping.

2.	If the S-NSSAI is in the Partially Allowed NSSAI, the UE shall not activate User Plane for any already established PDU Session with that S-NSSAI if the UE is in a cell within the RA but outside the Location information of the S-NSSAI.

3.	If the S-NSSAI is in the Partially Allowed NSSAI, and the UE moves to a cell inside the RA but outside the Location information of the S-NSSAI, the UE needs not perform a Mobility Registration Update.

4.	If the S-NSSAI is in the Allowed NSSAI, and the UE moves to a cell inside the RA but outside the Location information of the S-NSSAI, the UE shall perform a Mobility Registration update and, if the UE has an established PDU Session with that S-NSSAI, the PDU Session is dropped locally.

#### 5.15.18.3	Network based monitoring and enforcement of Network Slice Area of Service not matching deployed Tracking Areas

OAM may configure RRM policies for S-NSSAIs on a per cell basis as defined in TS 28.541 [149], i.e. cells outside the Network Slice Area of Service while in a TA supporting the S-NSSAI are allocated no Radio resources for the S-NSSAI.

The network handles the NS-AoS for an S-NSSAI while the US is in CM-CONNECTED mode as follows:

1.	The network monitors the validity of the S-NSSAI for UE in CM-CONNECTED state, i.e. as per the location inside a TA which support the S-NSSAI but the S-NSSAI is not available in some cells of the S-NSSAI. The NG-RAN can detect when the UE moves to cell where a network slice is not available and releases the related PDU sessions resources while keeping the PDU session context for network slices which have S-NSSAI in the Partially Allowed NSSAI or in the Allowed NSSAI. The NG-RAN reports the updated PDU session resources to the AMF in N2 messages. 

 When the User Plane Resources are activated for a PDU Session of an S-NSSAI part of the Partially Allowed NSSAI or the Allowed NSSAI in the UE context in the NG-RAN and the UE moves to a cell outside the NS-AoS, the User Plane Resources for the PDU Session shall be deactivated as per clause 4.3.7 of TS 23.502[3], but the PDU Session context in UE, NG-RAN and SMF is not released unless requested by the AMF. The User Plane Resources for the PDU Session shall not be activated as long as the UE is located in a cell which is not part of the NS-AoS associated with the S-NSSAI of the Partially Allowed NSSAI or the Allowed NSSAI.

When the User Plane Resources are deactivated for a PDU Session of an S-NSSAI in the Partially Allowed NSSAI or the Allowed NSSAI in the UE context in the NG-RAN, and the UE moves to a cell which is part of the NS-AoS associated with the S-NSSAI, the NG-RAN shall re-activate the User Plane Resources for the PDU Session and request the AMF to reactivate the PDU session.

Editor's note:	The mechanism  described above needs to be confirmed by RAN3

2.	If a non-supporting UE makes a PDU Session establishment request with an S-NSSAI that is not valid as per the S-NSSAI location availability information, the AMF may reject the NAS Transport message with a back-off timer using S-NSSAI based congestion control as described in clause 5.19.7.4.

3.	If the AMF determines that a non supporting UE in CM-CONNECTED has moved outside the NS-AoS, when it receives indication from the RAN that the PDU session resources have been released in the RAN, the AMF performs the following logic:

a)	If the non-supporting UE has other S-NSSAI(s) in the Allowed NSSAI, then the AMF may update the UE with a UE Configuration Update by removing the S-NSSAI from the Allowed NSSAI (which causes the UE to locally release the PDU Sessions) and optionally removing the S-NSSAI from the Configured NSSAI, and then, the AMF requests the SMF to locally release in the network the related PDU Sessions for the S-NSSAI as per step 1f in clause 4.2.3.4 in TS 23.502 [3]. Alternatively, the AMF requests the SMF to release PDU Sessions associated with the S-NSSAI.

b)	If the non-supporting UE does not have any other S-NSSAI in the Allowed NSSAI, then the AMF may update the UE with a UE Configuration Update by removing the S-NSSAI from the Allowed NSSAI (which causes the UE to locally release the PDU Sessions) and optionally removing the S-NSSAI from the Configured NSSAI, and adding a default S-NSSAI to the Allowed NSSAI, and then the AMF requests the SMF to locally release in the network the related PDU Sessionsfor the S-NSSAI as per step 1f in clause 4.2.3.4 in TS 23.502 [3]. Alternatively, the AMF requests the SMF to release PDU Sessions associated with the S-NSSAI.

NOTE:	Whether the AMF removes the S-NSSAI from Allowed NSSAI and Configured NSSAI or only releases the associated PDU Sessions when the AMF enforces the NS-AoS is up to AMF configuration e.g.based on operator policy which could be per S-NSSAI. Removing a S-NSSAI for the Configured NSSAI may be problematic if the UE enters CM-IDLE mode as it may move to area of availability, but the NG-RAN may not be able to trigger the restoration of the S-NSSAI in the configured NSSAI, hence the UE applications that need the S-NSSAI to operate may not be able to request the network slice. Unless the UE becomes CM connected for other reasons, the AMF has also no means to restore the S-NSSAI in the Configured NSSAI based on the AMF detecting the UE is in an area of availability (see bullet 4 below).

c)	For a non-supporting UE that does not have any other S-NSSAI in the Allowed NSSAI nor in the Configured NSSAI, then the AMF indicates to the SMF to release the PDU Session.

4.	For a non supporting UE, if the AMF determines that the S-NSSAI becomes valid e.g. the UE has moved in CM-CONNECTED mode into the NS-AoS, the AMF may update the UE with a UCU e.g. including the S-NSSAI in the Configured NSSAI, if the AMF had previously removed the S-NSSAI from the Configured NSSAI. 	For A supporting UE, if the PDU sessions of the S-NSSAI were deactivated, the network reactivates the PDU sessions.

END CHANGE