3GPP TSG-SA WG2 Meeting #157	S2-2307143

# Berlin , DE, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR |  | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Paging Early Indication(PEI) handling with emergency service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2023-04-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is a mirror coresponding to a change in Rel 17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description that AMF provides indication RAN to indicate RAN to disable/re-enable UE’s temporary ID based PEI based on whether UE has an emergency PDU Session; UE disable/re-enable UE’s temporary ID based PEI based on whether UE has an emergency PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Paging for UE with emergency PDU session may be delayed or, owing to UE missing paging in case the PDU session is locally released in UE side, fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1, 5.4.6.3, 5.4.12.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

### 5.4.3	Paging strategy handling

#### 5.4.3.1	General

Based on operator configuration, the 5GS supports the AMF and NG-RAN to apply different paging strategies for different types of traffic.

In the case of UE in CM-IDLE state, the AMF performs paging and determines the paging strategy based on e.g. local configuration, what NF triggered the paging and information available in the request that triggered the paging, whether the UE has an emergency PDU Session. If NWDAF is deployed, the AMF may also use analytics (i.e. statistics or predictions) on the UE's mobility as provided by NWDAF (see TS 23.288 [86]).

When the UE has an emergency PDU Session or the emergency PDU Session has been released but the PDU Session status has not been synchronized with UE, the AMF shall set a “Disable UE’s temporary ID based PEI” indication to indicate the RAN to disable the UE’s temporary ID based PEI as specified in clause 5.4.12. After the emergency PDU Session released and the PDU Session status synchronized with UE, the AMF shall set a “Re-enable UE’s temporary ID based PEI” indication to indicate the RAN to re-enable the UE’s temporary ID based PEI.

In the case of UE in CM-CONNECTED with RRC_INACTIVE state, the NG-RAN performs paging and determines the paging strategy based on e.g. local configuration, and information received from AMF as described in clause 5.4.6.3 and SMF as described in clause 5.4.3.2.

In the case of Network Triggered Service Request from SMF, the SMF determines the 5QI and ARP based on the downlink packet (if the SMF performs buffering) or the Downlink Data Report received from UPF (if the UPF performs buffering). The SMF includes the 5QI and ARP corresponding to the QoS Flow of the received downlink PDU in the request sent to the AMF. If the UE is in CM IDLE, the AMF uses e.g. the 5QI and ARP to derive different paging strategies as described in clause 4.2.3.3 of TS 23.502 [3].

NOTE:	The 5QI is used by AMF to determine suitable paging strategies.

#### ****************** next CHANGE ***************

#### 5.4.6.3	Core Network assisted RAN paging information

Core Network assisted RAN paging information aids the RAN to formulate a RAN paging policy and strategy in RRC Inactive state, besides the PPI and QoS information associated to the QoS Flows as indicated in clause 5.4.3.

CN assisted RAN paging information may be derived by the AMF per UE and/or per PDU Session based on collection of UE behaviour statistics, Expected UE Behaviour and/or other available information about the UE (such as subscribed DNN, SUPI ranges, Multimedia priority service), and/or information received from other network functions when downlink signalling or user plane traffic is triggered.

The CN assisted RAN paging information consists of a service priority (values 1 to 256) which provides AN with a way to understand how important the downlink signalling is. The AMF derives this service priority based on available information as described above. The method to derive the service priority is implementation depended and can be controlled by operator.

The Core Network may provide the CN assisted RAN paging information to RAN in different occasions, e.g. during downlink N1 and N2 message delivery, etc.

The AMF shall set a “Disable UE’s temporary ID based PEI” indication in the CN assisted RAN paging information to indicate the RAN to stop using the UE’s temporary ID based PEI as specified in clause 5.4.12 if there is an active emergency PDU Session or an emergency PDU Session has been released but the PDU Session status has not been synchronized with UE. After the emergency PDU Session released and the PDU Session status synchronized with UE, the AMF shall set a “Re-enable UE’s temporary ID based PEI” indication in the CN assisted RAN paging information to indicate the RAN to reuse the UE’s temporary ID based PEI.

****************** next CHANGE ***************

### 5.4.12	Paging Early Indication with Paging Subgrouping Assistance

#### 5.4.12.1	General

The RAN and UE may use a Paging Early Indication with Paging Subgrouping (PEIPS) to reduce the UE's power consumption in RRC_IDLE and RRC_INACTIVE over NR (see TS 38.304 [50]).

The paging subgrouping can be based on either the UE's temporary ID or a Paging Subgroup ID allocated by the AMF. Paging subgrouping based on the UE's temporary ID is implemented within the NG-RAN. For paging subgrouping based on UE's temporary ID, the UE's support is included in the UE Radio Capability for Paging, either derived by the NG-RAN from UE provided UE radio capability (see clause 5.4.4) or based on UE Radio Capability ID if UE radio capability signalling optimisation is used (see clause 5.4.4.1a).

The RAN shall disable/re-enable the UE’s temporary ID based PEI based on the indication “Disable/Re-enable UE’s temporary ID based PEI” from AMF as specified in clause 5.4.3. For the UE, it shall disable the UE’s temporary ID based PEI if there is an active emergency PDU Session or an emergency PDU Session has been released but the PDU Session status has not been synchronized with the Core Network. After the emergency PDU Session released and the PDU Session status synchronized with the core network, the UE re-enable the UE’s temporary ID based PEI.

The AMF, when determining its paging strategy (see clause 5.4.3), should take into consideration whether a gNB is using Paging subgrouping based on the UE's temporary ID.

NOTE:	Paging messages sent to that gNB can increase UE power consumption for other UEs that support Paging Subgrouping based on the UE's temporary ID.

If paging subgroups are being allocated by the AMF, then all AMFs connected to one gNB (including the AMFs in different PLMNs using 5G MOCN network sharing) shall use a consistent policy in allocating UEs to the paging subgroups. The AMF may configure up to 8 different Paging Subgroup IDs.

NOTE:	Because the UE uses the AMF allocated paging subgroup across all cells in its TAI-list, and, different, overlapping TAI lists can be allocated to different UEs, then to avoid UE power consumption increasing, it is likely to be necessary that all AMFs with an overlapping coverage area use a consistent policy in allocating UEs to the paging subgroups.

The NG-RAN node and the UE determine per cell which paging subgrouping method to use as defined in TS 38.304 [50] and TS 38.331 [28].

****************** END CHANGES ***************