3GPP TSG-WG SA2 Meeting #157 	S2-2307171

Berlin, Germany, 22-26 May 2023

# Title:	[Draft] Reply LS to 3GPP SA2 on 3GPP and non-3GPP interworking information in VoWiFi

# Response to:	LS to 3GPP SA2 on 3GPP and non-3GPP interworking information in VoWiFi (S2-2306286)

# Release:	Release 18

# Work Item:	N/A

Source:	SA2

To:	GSMA TSGUEWIFI

Cc:	CT1, GSMA NG

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 would like to thank GSMA for the LS on 3GPP and non-3GPP interworking information in VoWiFi.

SA2 would like to inform GSMA that handover from 5GS or EPS and WLAN is controlled via two UE configuration parameters called “Allow_handover_PDN_connection_WLAN_and_EPS” and " Allow_Handover_PDU_session_non-3GPP_and_NG-RAN" in IMS MO TS 24.167 which enable preventing/allowing handover of the PDN connection providing access to the IMS from WLAN to EPS and 5GS.

These UE configuration parameters prevent or allow handover of the PDN connection providing access to the IMS from WLAN to EPS or 5GS regardless of the value of the IMS voice over PS session indicator sent by the VPLMN.If the UE is not attached in EPS or 5GS prior to the handover, the UE will not know the value of the IMS voice over PS session indicator until the handover completes since this indicator is sent in the Attach or Registration Accept messages. If the UE is attached/registered in EPS or 5GS prior to the handover, the UE will know the value of the IMS voice over PS session indicator, before initiating the handover. In TS 24.229 subclauses L.2.2.1 and W.2.2.1 the text: " if not prevented by other rules or policies" implies that the UE should take the IMS voice over PS session indicator into account to decide whether to initiate the handover.

TS 23.502 defines in clause "4.13.6.3	Transfer of PDU session used for IMS voice from non-3GPP access to 5GS" the procedures for transfer of PDU session from non-3GPP access (WLAN) 5GS or EPS using EPS fallback. In order to address the case that handover is possible sometimes directly in EPS to avoid EPS fallback procedure the following NOTE is documented:

NOTE 1:	If the UE is aware (e.g. implementation-dependent mechanisms) that voice over NR may not be natively supported in the current Registration area, the UE can attempt to move to E-UTRA to initiate a handover of the IMS PDU Session to EPC or 5GC to continue the IMS voice session. The remaining steps are not executed.

SA2 does not see the need for any extra functionality to be specified in rel.18.

2. Actions:

To GSMA TSGUEWIFI:

ACTION: 	SA2 respectfully request GSMA to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #158	21-25 August 2023		Gothenburg, Sweden

TSG-SA2 Meeting #159	9-13 October 2023		Xiamen, P.R. China