3GPP TSG-WG SA2 Meeting #157	S2-2307253

# Berlin, Germany, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0847 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Location Accuracy Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections are needed on Location Accuracy Analytics: - “LOS” and “NLOS” abbreviations are not defined (and NLOS may either mean “non line of sight” or “near line of sight”). - “Measurement method” is unclear: measurement of what? - Terminology “below cell level” was agreed to be changed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add abbreviations - Clarify that the measurement method is for accuracy measurement. - Modify “below cell level” to be “longitude and latitude level” based on S2-2306105 as agreed in SA2#156e - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.17.1, 6.17.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes in this CR complement and do not clash with those in CR#0743R1. |  |  |  |  |  |  |  |  |
| This ’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

LOS	Line Of Sight

NLOS	Non Line Of Sight

# * * * * Next change * * * *

## 6.17	Location Accuracy Analytics

### 6.17.1	General

This clause specifies how an NWDAF can provide Location Accuracy Analytics which provides analytics for LCS QoS accuracy to an analytics consumer such as LMF.

The Location Accuracy Analytics provides one or a combination of the following information:

-	location accuracy (i.e. Horizontal Accuracy (see clause 4.3.1 of TS 22.071 [35]) and optinally Vertical Accuracy (see clause 4.3.2 of TS 22.071 [35]);

-	Indication on whether a UE is indoor or outdoor when a location measurement is made (optional);

-	Indication on the ratio of LOS/NLOS measurements;

-	Applicable area for location accuracy prediction (optional);

-	requested Confidence of location accuracy prediction (optional).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Location Accuracy";

-	Target of Analytics Reporting: "any UE";

Editor´s note:	Other target of analytics reporting is FFS.

-	Location (horizontal and optional vertical) or area of interest;

-	Accuracy measurement method;

-	Analytics Filter Information as defined in table 6.17.1-1;

-	optionally, a preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset (see clause 6.17.3);

-	optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics subsets (see clause 6.17.3) in order to be notified by the NWDAF;

-	optionally, maximum number of objects; and

-	an Analytics target period indicates the time period over which the statistics or predictions are requested.

Table 6.17.1-1: Analytics Filter Information related to Location Accuracy Analytics

| Information | Description |
| --- | --- |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. |
| List of analytics subsets | List of analytics subsets that are requested among those specified in clause 6.17.3. |
| NOTE: All parameters are optional. |  |

# * * * * Next change * * * *

### 6.17.3	Output Analytics

The Location Accuracy analytics are shown in table 6.17.3-1.

Table 6.17.3-1: Location Accuracy statistics

| Information | Description |
| --- | --- |
| List of Accuracy sustainability Analytics (1..max) |  |
| >Applicable Area | A list of TAIs or Cell IDs or longitude and latitude level location information that the analytics apply to. |
| >Applicable Time Period | The time period within the Analytics target period that the prediction applies to. |
| > Positioning Method | The positioning method used to measure location. |
| > Analytics accuracy (horizontal and optionally vertical) | predictions about the horizontal and optionally vertical location accuracy of applicable positioning method at the indicated location or location area. |
| > Percentage of UE in indoor/outdoor in target area (optional) | Provides a predicted ratio of UEs that are indoor/outdoor in the target area. |
| > Percentage of NLOS/LOS measurements in target area | Provides predicted ratio of LOS to NLOS measurements of UEs in target area. |
| > Location | TA, or cell, or other location information as defined in TS22.271 |
| >Applicable Time Period | The time period within the Analytics target period that the prediction applies to. |
| > Positioning Method | The positioning method used to measure location. |
| > Analytics accuracy (horizontal and optionally vertical) | predictions about the horizontal and optionally vertical location accuracy of applicable positioning method at the indicated location or location area. |
| > Indication whether location is indoor/outdoor (optional) | If a single location for a UE is provided, indicates whether this location is predicted to be indoor or outdoor. |
| > Indication whether the location is measured NLOS/LOS | If a single location for a UE is provided, indicates whether this location is measured with NLOS or LOS. |
| >Confidence | Confidence of the prediction. |

# * * * * End of changes * * * *