3GPP TSG-WG SA2 Meeting #157	S2-2307321

# Berlin, Germany, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0848 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ratio of moving users as analytics subset of UE flow analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE flow analytics as specified in CR#0662 allows the NWDAF to indicate the ratio of UEs that are moving in the area of interest, by using a “Speed threshold” entry set to 0. However, if the analytics consumer is only interested in knowing the ratio of UEs that are moving (and does not need numbers/ratios for other speed thresholds), there is no way to indicate this in the analytics request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds “Ratio of moving users” as an explicit analytics subset. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Excessive computation and signalling when the analytics consumer is only interested in knowing the number or ratio of UEs that are moving. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR uses the text from CR#0662R5 as baseline because the text proposed to be modified is not yet in the TS. |  |  |  |  |  |  |  |  |
| This ’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (baseline text is from CR#0662R5) * * * *

### 6.x.3	Output analytics

The output analytics of UE flow provided by NWDAF is defined in Table 6.x.3-1 and Table 6.x.3-2.

Table 6.x.3-1: UE Flow statistics

| Information | Description |
| --- | --- |
| Applicable Area | A geograohical area as defined in TS 23.273 [39] that the analytics applies to. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time during the beginning and the end of UE location information collection |
| > Total number of users (NOTE) | Total number of users in the area of interest |
| > Ratio of moving users (NOTE) | Ratio of moving users in the area of interest. |
| > Average speed (NOTE) | Average speed of all users in the area of interest. |
| > Speed threshold (1...max) (NOTE) | Threshold utilized to filter the users |
| >> Number of users | The number of users whose speed is faster than the speed threshold |
| >> Ratio | Percentage of users whose speed is faster than the speed threshold |
| > Direction (1…max) (NOTE) | Heading directions of the UE flow in the target area, such as north, south, east, west etc. |
| >> Number of users | The number of users in the specific direction. |
| >> Average speed | Average speed of users in the specific direction. |
| >> Ratio | Percentage of users. |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

Table 6.x.3-2: UE Flow predictions

| Information | Description |
| --- | --- |
| Applicable Area | A geograohical area as defined in TS 23.273 [39] that the analytics applies to. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time during the beginning and the end of UE location information collection |
| > Total number of users (NOTE) | Total number of users in the area of interest |
| > Ratio of moving users (NOTE) | Ratio of moving users in the area of interest. |
| > Average speed (NOTE) | Average speed of all users in the area of interest. |
| > Speed threshold (1...max) (NOTE) | Threshold utilized to fiter the users |
| >> Number of users | The number of users whose speed is faster than the speed threshold |
| >> Ratio | Percentage of users whose speed is faster than the speed threshold |
| > Direction (1..max) (NOTE) | Heading directions of the UE flow in the target area, such as north, south, east, west etc. |
| >> Number of users | The number of users in the specific direction. |
| >> Average speed | Average speed of users in the specific direction. |
| >> Ratio | Percentage of users. |
| Confidence | Confidence of the prediction |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

# * * * * End of changes * * * *