SA WG2 Meeting #157	S2-2307322

# 22-26 May 2023, Berlin, Germany	(was S2-230xxxx)

Title:	[DRAFT] Reply LS on addressing packet loss during multicast MBS delivery

Response to:	S6-231018 / S2-2306284

Release:	Release 17

Work Item:	5MBS, MCOver5MBS

Source:	SA2

To:	SA6

Cc:	CT3

Contact Person:

#### Name:	Haris Zisimopoulos

E-mail Address:	harisz@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.247 Rel-18 CR0274

1. Overall Description:

SA2 thanks SA6 for their LS on addressing packet loss during multicast MBS delivery (S6-231018 / S2-2306284).

Regarding SA6 concerns:

“During the silent period without DL media, the multicast MBS session used for this group communication might become inactive and the joined UEs become IDLE. The new DL packets trigger the multicast MBS session being activated and the IDLE UEs being paged. However, not all the UEs become CONNECTED at the same time. The late CONNECTED UEs will lose the packet as the NG-RAN will start to transmit the packet when the first UE becomes CONNECTED which triggers the shared delivery restoring again.”

SA2 would like to clarify that:

- When the multicast MBS session becomes inactive, during reception of MBS DL data for that session, MBS session activation procedure as specified in TS 23.247 clause 7.2.5.2 is triggered by the MB-UPF.

- During that procedure, the MB-UPF is buffering the MBS downlink data. This buffering needs to consider that UEs are being paged via possibly multiple AMFs (and with multiple SMFs involved), and this can be configured in a way that under normal circumstances all UEs within coverage will receive the paging message. Note that it is impossible to guarantee that all UEs will receive and respond to paging, some might not even be in coverage or have turned off without deregistration.

- Even though the NG-RAN starts transmissions of received MBS downlink packets when the first UE becomes RRC Connected, this is not an issue, since the MBS downlink data is being buffered not in the NG-RAN but in the MB-UPF while the MBS session activation is being performed.

SA2 has agreed to the attached CR clarifying the MB-UPF buffering of MBS data during MBS session activation procedure.

2. Actions:

To SA6:

ACTION: 	SA2 kindly asks SA6 to take the clarifications and feedback above into account.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #158	21-25 August 2023		Gothenburg, Sweden

TSG-SA2 Meeting #159	9-13 October 2023		Xiamen, P.R. China