3GPP TSG SA WG-2 Meeting #157 	S2-2307325

Berlin, Germany, May 22 – 26, 2023

# Source:	Qualcomm Incorporated

# Title:	New SID on Architecture Enhancements for Vehicle Mounted Relays (VMR) Phase 2

# Document for:	Approval

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Architecture Enhancements for Vehicle Mounted Relays Phase 2

Acronym:	VMR_Ph2

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 930021 | Stage 1 of Vehicle-Mounted Relays | Stage 1 for VMR in Rel-18 |
| 980019 | Architecture Enhancements for Vehicle Mounted Relays | Stage 2 for VMR in Rel-18 |
| 850009 | Architecture enhancements for the support of Integrated access and backhaul (IAB) | Baseline IAB architecture support. |
| 941009 | Mobile IAB (Integrated Access and Backhaul) for NR | RAN aspects of the VMR feature in Rel-18. |

# 3	Justification

In Rel-18, SA2 conducted a study followed by a work item on VMR (Vehicle-Mounted Relays). During the study, it was agreed to limit the scope of the work to the IAB (Integrated Access and Backhaul) architecture, whereby the IAB-node consist of an IAB-MT and an IAB-DU, with the IAB-DU establishing an F1 interface with a donor CU over a wireless link. There are however other architecture options to achieve the functionality of vehicle-mounted relay, for instance the so-called “Velcro” solution whereby the relay node consists of a UE co-located with a full gNB, with the gNB in the relay establishing N2 and N3 interface to an AMF residing in the 5GC over a PDU session. These architecture options which have not yet been studied by SA2 may be better suited to certain deployment scenarios for vehicle-mounted relays, for instance using the VMR where IAB is not widely supported, or the scenario in which the relay process local traffic in the vehicle to provide onboard services with low latency.

Additionally, the following other functionalities were not included in the Rel-18 work:

- NTN backhauling, to provide coverage e.g on vessels, aircrafts and in other areas without TN coverage

- Extension of UAV functionality to relay to provide service to non-UAV-capable UEs in helicopter or drones

It is therefore proposed to have a follow-up study to cover the above-mentioned items which were not specified in Rel-18.

# 4	Objective

The aim of this study work is to investigate and identify potential architecture and system level enhancements to further evolve the functionalities of base station relays mounted on vehicles. Specifically, the objectives include:

- WT#1: Identify gaps for supporting the architecture with a full gNB/CU onboard of a relay and the use of a PDU session for the wireless backhauling of the N2/N3 interfaces.

- WT#2: Enable mobility for a full gNB/CU onboard of a relay using a PDU session for the wireless backhauling of the N2/N3 interfaces.

- WT#3: Specify architecture enhancements for the selected option(s) to enable authorization of relay

- WT#4: Specify architecture enhancements for the selected option(s) to enable QoS support over the backhaul

- WT#5: Specify architecture enhancements for the selected option(s) to enable Cell ID/TAC management

- WT#6: Specify architecture enhancements for the selected option(s) to enable support for UE location services and emergency services

- WT#7: Specify architecture enhancements for the selected option(s) to enable NTN backhaul for VMR

- WT#8: Specify architecture enhancements for the access to onboard services/edge computing

- WT#9: Specify architecture enhancements for the selected option(s) to extend UAV functionality to VMR

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 1 | 1 | Yes | - |
| WT#2 | 2 | 1 | Yes |  |
| WT#3 | 1 | 0.5 | No | Depends on WT#1 and #2 |
| WT#4 | 0.5 | 0.5 | No | Depends on WT#1 and #2 |
| WT#5 | 0.5 | 0.5 | Maybe | Depends on WT#1 and #2 |
| WT#6 | 1 | 0.5 | Maybe | Depends on WT#1 and #2 |
| WT#7 | 1 | 1 | Maybe | Depends on WT#1 and #2 |
| WT#8 | 0.5 | 0.5 | Maybe | Depends on WT#1 and #2 |
| WT#9 | 0.5 | 0.5 | Maybe | Depends on WT#1 and #2 |

Total TU estimates for the study phase: 8

Total TU estimates for the normative phase: 6

Total TU estimates: N + M = 14

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on architecture enhancements for vehicle-mounted relays Phase 2 | TSG#103 Mar. 2024 | TSG#104 Jun. 2024 | Cheng, Hong, Qualcomm Incorporated, hongc@qti.qualcomm.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Cheng, Hong, Qualcomm Incorporated,

hongc@qti.qualcomm.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

-     Security aspects

-     Charging aspects

-     RAN aspects

# 9	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated

Hyundai Motor Company

KT Corp.