3GPP TSG-SA WG2#157	S2-2307384

Berlin, Germany, 22 – 26 May 2023	(revision of xx-yyxxxx)

# Source:	Samsung

# Title:	New SID: Study on Satellite access Phase 3

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Satellite access Phase 3

Acronym:	FS_5GSAT_Ph3

Unique identifier:	{A number to be provided by MCC at the plenary}

Potential target Release:	{Rel-19}

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  |  |
| Don't know |  |  | X |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

# 3	Justification

Justification for objective 1:

- Emergency services are most important services in 3GPP standard, different regulatory bodies have strict requirement to establish the emergency call quickly. The most important aspect to achieve this for UE in limited service is to identify accurately the operator supporting emergency service in UEs location. But in the area where UE is not allowed to operate (i.e. UE has received #78), at that location though the NG-RAN node broadcast emergency services as supported, it is not guaranteed that UE can still get the emergency service. On attempt of emergency service, the UE may be allowed or not allowed for emergency service even if broadcast information on emergency service indicates it is supported. This is a false information for the UE which can lead for delay in establishing the emergency call, because UE will have to undergo trial and error mechanism on the available networks to identify which operator supports emergency service. Thus, we propose to study mechanism to bring this feature at par with terrestrial network where UE can identify accurately whether emergency services are supported over each PLMN at a given location.

- MINT feature in release-17 supported disaster roaming for the forbidden PLMNs, in the context of PLMNs supporting satellite NG-RAN which are not allowed to operate at the UE location (i.e. one which have given #78 to the UE) can also be extended to provide disaster roaming services to the UE. This option is especially useful if all the RAN nodes of terrestrial network are down due to some kind of disaster e.g. earthquake etc.

Justification for objective 2:

- SA1 has studied the requirements when feeder link is unavailable. We propose to study mobility enhancement, power saving and store & forward mechanism in the 5G system for this deployment choice.

Justification for objective 3:

- For terrestrial networks, there is a mechanism available for network to redirect the UE to E-UTRA from N1 mode, instruct the UE to disable N1 mode(and force UE to remain in EPS), or disable E-UTRA(and force UE to remain in N1 mode) etc but for NTN RATs e.g. GEO, MEO and LEO though the RAT restrictions are defined at UDM they cannot be enforced on the UE, for example the solutions available for TN are not available. Thus, we propose to study mechanisms to support enforcement of RAT restrictions between different RAT types of satellite accesses.

Justification for objective 3 & 4: (If not concluded in Rel-18)

Rel-18 features are dependent on UE and AMF determining the unavailability duration for the UE based on which different power saving features are defined. But if there is no standard mechanism to provide SCAI information to the UE/AMF then:

- This information available between UE and AMF may not be in sync,

- In roaming areas configuring the SCAI information into the UE will be near to impossible because of practical challenges involved in interworking with different operators without standard mechanisms.

- AMF may remain dependent on the UE indicated information but it may not be trust worthy.

Thus we propose to study standard mechanism to provide SCAI information to the UE and AMF, identify the content of SCAI information which is necessary for UE and AMF. Identify further enhancements on how unavailability period can be accurately determined for the UE.

# 4	Objective

Following objectives are expected to be studied:

- Study mechanisms to support disaster roaming and improve emergency service using satellite access

- Study mechanisms to support store and forward mechanism in 5G system, mobility enhancements and power saving, when feeder link is unavailable.

- Study mechanisms to support enforcement of mobility restrictions between different RAT types of satellite accesses.

- Study standard mechanism to provide satellite coverage availability information (SCAI) to the AMF.

- Study standard mechanism to provide satellite coverage availability information (SCAI) to the UE and determine further enhancements to accurately determine unavailability period for the UE.

Plan: Estimate 7 TUs (5 TUs for the study and 2 TUs for the normative work).

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.7xy | Study on Satellite access Phase 3 | TBD | TBD | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Samsung

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

RAN2, TBD

# 9	Supporting Individual Members

Supporting IM name

Samsung