3GPP TSG SA WG2 Meeting #157	S2-2307599

# May 22 – 26, 2023, Berlin, Germany

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4685 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discontinuous coverage reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | We propose that reporting of unavailability period or indication of entering loss of coverage due to enterying DC is exempted from NAS level congesetion control otherwise unnecessarily UE context will be deleted from the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that NAS level congestion control is exempted for UE to report unavailability period/indication of loss of coverage.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | During MM back off timer if UE does not report unavailability period this can result in UEs context loss at the network.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.4.13.1	Mobility Management and Power Saving Optimization

For NR satellite access that provides discontinuous network coverage, a UE out-of-coverage period may be determined, which indicates the timing information for when a UE is expected to be out of coverage, and when the UE is expected to re-gain coverage again. The UE out-of-coverage period may consider current and expected future locations of the UE.

Tracking Area or RAT specific configuration in the AMF may be used to set timer values based on typical coverage periods of a satellite system.

NOTE 1:	For example, if a satellite system only provides coverage to a UE for 20 minutes when a satellite passes, and the maximum time before a satellite passes any point on the earth is 10 hours, the AMF could configure the periodic registration timer and Mobile Reachable timer to be just greater than 20 minutes and the Implicit Deregistration timer to be greater than 10 hours to avoid unintended implicit detach due to coverage gap. Such configuration does not require AMF to be aware of detailed coverage times for each UE or for different locations.

"Discontinuous coverage" includes UE notification to the network when the UE is about to leave from or return to coverage (as described below) and overload control for discontinuous coverage as defined in clause 5.4.13.5. During Registration procedure, a UE supporting Discontinuous coverage functionalities described in this clause and clause 5.4.13.5 provides "discontinuous coverage support" indication as part of 5GMM Core Network Capability in the Registration Request message for initial registration and for every mobility registration. The AMF indicates whether Discontinuous coverage functionalities described in this clause and clause 5.4.13.5 are supported by providing the "discontinuous coverage support" indication in Registration Accept message.

Editor's note:	Capability negotiation between UE and network for the features introduced for discontinuous network coverage for satellite access is FFS.

If the UE is able to determine its own UE out-of-coverage period, and decides to remain in no service the following applies:

-	the UE triggers the Mobility Registration Update procedure to inform the network of its UE out-of-coverage period, as described in clause 5.4.1.4

Editor's note:	It is FFS whether the UE can also indicate how long it will be available before the unavailability period starts.

Editor's note:	Whether Support of Unavailability Period (see clause 5.4.1.4) can be used directly without updates is to be determined.

Editor's note:	How the procedure of UE sending out-of-coverage period will be affected if the Satellite coverage availability information that is sent to the UE is not standardised is FFS.

If the UE is not able to determine its own UE out-of-coverage period but can determine that it is about to lose coverage, the following applies:

-	The UE triggers the Mobility Registration Update procedure and indicates that the UE is about to lose coverage due to discontinuous coverage but does not provide a UE out-of-coverage period.

-	In this case, if the AMF is able to determine a UE out-of-coverage period based on satellite coverage availability information, as described below, the AMF provides an expected unavailability duration to the UE in the Registration Accept. The AMF may take the out of coverage period into account when determining Periodic Registration Update timer value for the UE. The AMF stores the information that the UE is unavailable in UE context, and considers the UE is unreachable (i.e. clear the PPF in AMF) until the UE enters CM-CONNECTED state.

Editor's note:	It is FFS whether the AMF can provide an expected unavailability duration in the Registration Accept also if the UE provided an out-of-coverage time the Registration Request.

-	The UE triggers a Registration Update procedure when it returns to coverage using any access type.

-	The AMF may indicate to the UE that the UE is required to perform this procedure when leaving and returning to coverage. If the AMF does not indicate to the UE that the UE is required to perform this procedure, the usage is UE implementation dependent.

To avoid signalling overload, the Mobility Registration Update procedure as described above shall be triggered during a time range before the UE loses coverage, as many UEs may lose coverage at the same time. The UE should trigger the Mobility Registration Update procedure early enough such that the procedure, under normal conditions, is able to complete before the start of the unreachability period.

NOTE 2:	A UE based on implementation can combine successive periods of no satellite coverage into a single continuous period that is notified to the network if the UE does not require network access during this period.

NOTE 3:	UE informing the network of coverage gaps would increase signalling and UE power consumption if coverage gaps are more frequent than UE's communication period.

The AMF may be aware of the satellite coverage availability information and determine UE out-of-coverage period. In this case the following applies:

-	In addition to criteria described in other applicable clauses, the AMF may take UE out-of-coverage period into account when determining e.g. Periodic Registration Update timer, extended DRX in CM-IDLE (see clause 5.31.7.2), MICO mode configuration (see clause 5.4.1.3) and Estimated Maximum Wait Time.

-	The AMF uses existing procedures to provide the UE with timers (e.g. Periodic Registration Update timer), extended DRX in CM-IDLE (see clause 5.31.7.2), and MICO mode configuration (see clause 5.4.1.3), and to the NG-RAN Extended Connected Time (see clause 5.31.7.3). This is to keep UE in power saving mode and avoid the network attempting to page the UE if it is out of satellite network coverage.

The AMF should adjust the mobile reachable timer or Implicit Deregistration timer or both such that the AMF does not implicitly deregister the UE while the UE is out of coverage, see clause 5.4.1. Features described for High latency communication in clause 5.31.8 may be used to handle mobile terminated (MT) communication with UEs being unreachable due to discontinuous coverage.

The UE may send Mobility Registration Update procedure to inform the network of its UE unavailability period (see clause 5.4.1.4) even if Mobility Management back-off timer is running.

# >>>>END OF CHANGES<<<<