3GPP TSG-WG SA2 Meeting #157	S2-2308041

# Berlin, Germany, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0149 | rev | 2 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving ENs on multiple SMF coordination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The common EAS discovery procedure needs to be updated with coordination among SMFs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update on Common EAS discovery procedure with coordination among multiple SMFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Coordination among SMFs procedure is imcomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501 CR 4638 TS23.502 CR 4208 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.3.2.5	Common EAS discovery for a set of UEs

The following is the procedure for discovery common EAS for set of UEs accessing the same application. Different UEs can be served by different SMFs.

The common EAS IP address for the set of UEs can be provided by AF or determined by 5GC. AF can provide the common EAS IP via AF Traffic influence procedure as defined in clause 4.3.6.2 of TS 23.502 [3], AF may determine the common EAS IP address based on candidate DNAI(s) reported by SMF as described in clause 4.3.6.3 of TS 23.502 [3]. Also, the common EAS IP address can be determined by SMF using EAS discovery procedure, and SMF stores the common EAS IP address in UDR as part of AF traffic influence request information.

Figure 6.2.3.2.5-1: Common EAS discovery for a set of UEs

1.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [3] is used to request selecting the common EAS for a set of UEs accessing the application as identified in the AF Request.

AF may use External/Internal Group ID(s) or a list of UEs or any UE as Target UE Identifier(s) and additionally Spatial Validity Condition to identify the set of UEs for correlated selection of common EAS.

The following information may be included in AF request as defined in clause 5.6.7.1 of TS 23.501 [2]:

-	An EAS Correlation indication can be provided for selecting the same EAS for set of UEs accessing the same application.

-	A Traffic Correlation ID may be provided for identification of the set of UEs accessing the application identified by the Traffic Description in AF request.

-	A Common EAS to be accessed by the set of UEs may be included in AF request, if it is determined by AF.

-	FQDN(s) may be included which is corresponding to the application traffic identified by Traffic Description in AF request.

-	Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or a UE list or group ID can be provided for defining set of UEs accessing the same EAS.

In step 3a of figure 4.3.6.2-1 in TS 23.502 [3], NEF updates the AF influence data related to the traffic correlation ID in the UDR with a Notification Endpoint to subscribe to be notified with information related to SMF’s involvement for UE members of the set of UEs.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [3], PCF determines the UEs influenced by the AF Request, and for each UE, based on AF request, PCF creates PCC rule with Traffic Correlation ID, EAS Correlation indication,  Common EAS, FQDN(s) and NEF subscription to SMF due to step 3a  and sends the PCC rule to the SMF.

2.	Based on steps 1-19 in figure 6.2.3.2.2-1, with the following updates:

In step 9:

If FQDN in Neasdf_DNSContext_Notify Request is corresponding to the application indicated in PCC rule, e.g. the FQDN is included in the FQDN(s) in the PCC rule and if EAS Correlation indication is set, SMF determines the UE belongs to set of UEs identified by Traffic Correlation ID and accessing the application and determines the UE connects to the common EAS for the set of UEs. If FQDN(s) is included in PCC rule, the SMF can use the FQDN(s) in PCC rule and the FQDN in Neasdf_DNSContext_Notify Request to match the FQDN with the PCC rule, i.e. the matched PCC rule includes the FQDN(s) containing the FQDN in Neasdf_DNSContext_Notify Request.

If the common EAS is not available or SMF decides to selecte a new common EAS for the set of UEs:

Steps 10-15 are used for discovering of common EAS. After  step 15, coordination procedure among SMFs is operated as clause 6.2.3.2.7 for common EAS IP coordination.

Else if the common EAS is available and to be used for the set of UEs:

Steps 10-15 are skipped.

In step 16:

SMF may determine the DNAI based on the common EAS.

In step 17:

SMF sends DNS message handling rule including IP address for the common EAS and the Forwarding Action "Respond directly to the DNS request" for instructing EASDF to return the Common EAS IP address in a DNS response to UE directly.

In step 19:

If received IP address of the common EAS and instructed to respond directly in step 17, EASDF sends DNS response with the IP address of the common EAS to UE.

# * * * * End of changes * * * *