SA WG2 Meeting #S2-158	S2-2308293

21 - 25 August, 2023, Goteborg, Sweden

3GPP TSG-CT WG1 Meeting #142	C1-234361

# Bratislava, 22– 26 May 2023

Title:	Reply LS on Overload Control in Support of Discontinuous Network Coverage

Response to:	LS S2-2305576 on Overload Control in Support of Discontinuous Network Coverage

Release:	Release 18

Work Item:	5GSAT_Ph2

Source:	CT1

To:	SA2

Cc:

Contact person:

Name:	Chen Xu

E-mail Address:	chenxu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 24.501CR5240

# 1	Overall description

CT1 thanks SA2 for their LS to ask CT1 to decide whether the maximum wait time will require a new IE or reuse existing IE used for MINT.

CT1 would like to reply to SA2 that CT1 has agreed to use a new IE for discontinuous coverage maximum waiting time in TS 24.501CR5240 for the following reasons:

-	the IE for MINT includes a range i.e. both minimum and maximum time, whereas the overload control for discontinuous coverage only needs the maximum time;

-	A UE supporting discontinuous coverage for satellite access and MINT could receive different maximum wait time for both features at the same time.

# 2	Actions

To SA2

ACTION: 	CT1 kindly asks SA2 to take the above reply into account.

# 3	Date of next CT1 meetings

CT1#143	21st  – 25th August 2023 		Goteborg

CT1#144	9th   – 13th October 2023 		China (TBC)

CT1#145	13th   – 17th November 2023 		Chicago