3GPP TSG SA WG2 Meeting SA2#158		S2-2308483

August 21 – 25, 2023, Gothenburg, Sweden

Title:	[draft]Reply LS on UE Policy Control with PCF re-selection during AMF relocation

Response to:	LS S2-2308295/S2-2210170 on LS on UE Policy Control with PCF re-selection during AMF

Release:	Release 17, Release 18

Work Item:	5GS_Ph1, TEI17, eUEPO

Source:	Ericsson [to be: SA2]

To:	CT3

Cc:	CT1

Contact person:	Belen Pancorbo

belen.pancorbo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR 4267 TS 23.502 and CR 4268 (TS 23.502)

# 1	Overall description

SA2 has discussed the question raised by CT3 in LS S2-2210170 that was responded as follows:

Question 1 (raised by CT3): At the reception of UE Policy Association creation request, when no indication about the list of stored PSIs, ANDSP support, and UE OSId is received, shall the PCF interpret that the UE is triggering initial registration and the UE does not have UE Policies stored? If not, how does the PCF determine it is a different scenario?

Answer 1:  SA2 could not reach an agreement on whether the PCF needs to differentiate the 2 scenarios or whether the existing procedure is sufficient. SA2 is further seeking feedback from CT1 (via LS S2-2211347) on UE behaviour when UE PSIs are delivered to the UE in order to decide the way forward. SA2 would provide definitive feedback to CT3 after consulting with CT1.

A CT1 LS reply is received in S2-2308295/C1-234389 that provides the following feedback:

SA2 asked:

--------------

Question 1: How will the UE react if the PCF provides a Policy Section that contains only the PSI and there is no Policy Section stored in the UE with that PSI?

Question 2: Will the same UE behaviour be expected for a Rel-15, Rel-16 and Rel-17 UE?

--------------

There are different possible interpretations from CT1 specification within CT1 WG regarding what the UE behavior is.

When the PCF provides a Policy Section that contains only the PSI and there is no Policy Section stored in the UE with that PSI, and the UE is compliant to TS 24.501 Rel-15 only, Rel-16 only, Rel-17 only, or present Rel-18 version, depending on interpretation of the specification, the UE can:

- send a MANAGE UE POLICY COMMAND REJECT message; or

- ignore the instruction (i.e. the Policy Section that contains only the PSI), not diagnose an error, and continue processing MANAGE UE POLICY COMMAND message.

SA2 has discussed CT1 feedback then provide a response to Question 1 in CT3 LS S2-2210170:

Question 1 (raised by CT3): At the reception of UE Policy Association creation request, when no indication about the list of stored PSIs, ANDSP support, and UE OSId is received, shall the PCF interpret that the UE is triggering initial registration and the UE does not have UE Policies stored? If not, how does the PCF determine it is a different scenario?

Answer to Question 1: the reception of the UE Policy Association creation request with no UE Policy Container can either indicates that a) the UE does not have any stored PSIs or any UE capability such as ANDSP support or UE OSId to report or that b) the UE has stored PSIs, but moved to a new AMF that established a new UE Policy Association with a new PCF, given that the UE is not aware of this, the list of stored PSIs are not reported to the PCF at UE Policy Association Establishment. To differentiate these two scenarios, a new indication of AMF relocation is included in the UE Policy Association Establishment as described in attached Rel-17 CR 4267 to 23.502 and its Rel18 mirror.

# 2	Actions

To CT3

ACTION: 	SA2 kindly asks CT3 to take the above replies into account.

# 3	Dates of next TSG SA WG2 meetings

SA2#159	9th   – 13th October 2023 		Xiamen (China)

SA2#160	13th   – 17th November 2023 		Chicago