3GPP TSG-WG SA2 Meeting #S2-158	S2-2308522

# 21 - 25 August, 2023, Goteborg, Sweden		(revision of xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User Plane Stack for Ranging/SL positioning service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Inspur |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - It is concluded that PC5-U for V2X or ProSe can be used for the transmissiong of RSPP message. But the stack is not reflected in the specification. - 5G V2X user plane include LTE PC5-U and NR PC5-U. It is not clear that whether or not |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the specificaction of stack used to transmit RSPP messages. - Add a EN indicating it is up to RAN2 to decide if or not LTE PC5-U can be used to transmit RSPP messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The stack for Ranging/SL positioning service is missing. - It is not clear RSPP messages can be transmitted via LTE PC5-U. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1	User plane for PC5 supporting Ranging/SL positioning service

Figure 6.1-1 depicts a user plane stack for NR PC5 reference point supporting Ranging/SL positioning service.

# Figure 6.1-1: User Plane for NR PC5

The user plane for NR PC5 supporting Ranging/SL positioning service can be established via:

-	5G ProSe Direct Communication via PC5, specified in clause 5.3, TS 23.304 [7];

-	5G V2X communication via NR PC5, specified in clause 6.3, TS 23.287 [6].

Editor's note:	Whether or not LTE PC5-U can be used to transmit RSPP messages is subject to RAN2’s conclusion.

# * * * * End of changes * * * *