3GPP TSG-WG SA2 Meeting #158	S2-2308588

# Goteborg, Sweden, Aug 21 – 25, 2023	(was S2-230XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4739 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for UE power saving management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Protocol Description can be used for PDU Set information marking or End of data burst indication marking. - In the scenario of PDU Set based control, the Protocol Description is provided within the PDU Set Control Information in PCC rule can provide PDU Set QoS parameters and Protocol Description. However, in the scenario of End of data burst indication marking, there is no statement saying about what information the PCC rule and/or AF can provide to request the SMF to trigger UPF mark the end of data burst. This should be clarified. In our perspective, Protocol Description and End of Data Burst Marking Indication are needed as the Data Burst Based Handling Information in the PCC rule. Base on that or local configuration, the SMF can trigger End of Data Burst Marking. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Data Burst Based Handling Information in PCC Rules based on the information provided by the AF and/or the local operator policies. - Editorial changes to the End of Data Burst Marking Indication and N6 Traffic Parameter Measurement Information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The defintion for UE power saving management is not complete and sufficient correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.5.4, 5.37.8.2, 5.37.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

5.8.5.4	QoS Enforcement Rule

The following table describes the QoS Enforcement Rule (QER) that defines how a packet shall be treated in terms of bit rate limitation and packet marking for QoS purposes. All Packet Detection Rules that refer to the same QER share the same QoS resources, e.g. MFBR.

Table 5.8.5.4-1: Attributes within QoS Enforcement Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this QER |  |
| Rule ID | Unique identifier to identify this information. |  |
| QoS Enforcement Rule correlation ID (NOTE 1) | An identity allowing the UP function to correlate multiple Sessions for the same UE and APN. | Is used to correlate QoS Enforcement Rules for APN-AMBR enforcement. |
| Gate status UL/DL | Instructs the UP function to let the flow pass or to block the flow. | Values are: open, close, close after measurement report (for termination action "discard"). |
| Maximum bitrate | The uplink/downlink maximum bitrate to be enforced for the packets. | This field may e.g. contain any one of: - APN-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of all PDN Connections to an APN) (NOTE 1). - Session-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of the PDU Session) - QoS Flow MBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - SDF MBR (for a QER that is referenced by the uplink/downlink Packet Detection Rule of a SDF) - Bearer MBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Guaranteed bitrate | The uplink/downlink guaranteed bitrate authorized for the packets. | This field contains: - QoS Flow GBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - Bearer GBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Averaging window | The time duration over which the Maximum and Guaranteed bitrate shall be calculated. | This is for counting the packets received during the time duration. |
| Down-link flow level marking | Flow level packet marking in the downlink. | For UPF, this is for controlling the setting of the RQI in the encapsulation header as described in clause 5.7.5.3. |
| QoS Flow ID | QoS Flow ID to be inserted by the UPF. | The UPF inserts the QFI value in the tunnel header of outgoing packets. |
| Paging Policy Indicator | Indicates the PPI value the UPF is required to insert in outgoing packets (see clause 5.4.3.2). | PPI applies only for DL traffic. The UPF inserts the PPI in the outer header of outgoing PDU. |
| Packet rate (NOTE 1) | Number of packets per time interval to be enforced. | This field contains any one of: - downlink packet rate for Serving PLMN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections using CIoT EPS Optimisations as described in TS 23.401 [26]). - uplink/downlink packet rate for APN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections to the same APN using CIoT EPS Optimisations as described in TS 23.401 [26]). |
| End of Data Burst Marking Indication | Indicates to the UPF to detect the last PDU of the data burst based on the End Indication in the protocol header identified by the Protocol Description and insert an End of Data Burst indication of the last PDU of a Data burst to the NG-RAN over GTP-U | NG-RAN can configure UE power management schemes like connected mode DRX when UPF provides an indication of the End of Data Burst, see clause 5.37.8.3. |
| PDU Set Information marking Indicator | Indicates the UPF to insert PDU Set Information related to packets belonging to a PDU Set into GTP-U header. | UPF identifies PDU Sets in DL traffic and forwards PDU Set related information of each PDU to the NG-RAN over GTP-U, as described in clause 5.37.5. |
| NOTE 1: This parameter is only used for interworking with EPC. |  |  |

# * * * * Next change * * * *

5.37.8.1	General

The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

-	Indication of End of Data Burst.

The UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity are provided by the CN to NG RAN via TSCAI.

5.37.8.2	Periodicity and N6 Jitter Information associated with Periodicity

In the procedures for setting up or updating an AF session with QoS, the 5G System may be provided with Periodicity information for NG-RAN to configure UE power management for connected mode DRX. The Periodicity information is provided by the AF to the PCF via NEF or directly to the PCF when the AF is trusted.

If Periodicity information received from the AF/NEF the PCF sends the Periodicity information r to the SMF in Traffic Parameter Information in the PCC rule.  Based on the Periodicity and in accordance with the operator's local policies, or the AF request N6 Traffic Parameters measurement. PCF includes an indication for SMF to request the UPF to perform N6 Traffic Parameter(s) measurement (i.e. DL periodicity associated N6 jitter range and if not provided by the AF, UL/DL periodicity) within the PCC Rules.

Upon reception of a PCC rule with Traffic Parameter Information (i.e. Periodicity information), the SMF includes the Periodicity information in the TSCAI and forwards it to the NG-RAN. If the PCC rule indicates to perform N6 Traffic Parameter measurements, the SMF shall request the UPF to monitor and periodically report the N6 Traffic Parameters (i.e. the N6 jitter range associated with the DL Periodicity and, if not provided by the AF, UL/DL periodicity) using the N4 Session Modification procedure, see clause 5.8.5.11. If the measurement of N6 jitter range associated with the DL Periodicity is required and the DL Periodicity is available at the SMF, the SMF also sends the DL Periodicity to the UPF. The UPF reports the measured N6 Traffic Parameters to SMF via N4 interface.

NOTE 1:	How the UPF derives the N6 jitter and periodicity (i.e. when periodicity is not provided by the AF) is implementation dependent.

At reception of measured N6 Traffic Parameter(s) from the UPF in the N4 Session Level Report, the SMF includes the N6 jitter range, DL periodicity and/or the UL periodicity in the TSCAI and forwards it to the NG-RAN in an NGAP message, see clause 5.27.2. The Periodicity information may be provided by the AF or measured by the PSA UPF.

NOTE 2:	In order to prevent frequent updates from the UPF, the UPF sends the N6 Jitter Measurement Report periodically or only when the N6 jitter is larger than a threshold.

The DL periodicity associated N6 jitter indicates the positive or negative deviation of the arrival time of first arrived packet of a Data Burst compared to the ideal Data Burst start time which is be determined based on the DL periodicity.

5.37.8.3	End of Data Burst Indication

An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g. to configure UE power management schemes like connected mode DRX.

The PCF may provision Data Burst Based Handling Information (i.e. End of Data Burst Marking Indication and the Protocol Description) within PCC Rules based on the information provided by the AF and/or the local operator policies.

SMF should request the UPF to detect the last PDU of the data burst and mark the End of Data burst in the GTP-U header of the last PDU in downlink and provides the Protocol Description to the UPF, according to the PCC rule and/or the local operator policies.

According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the Data Burst End indication in the protocol header as defined in Protocol Description (see TS 26.522 [179]) or UPF implementation and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst.

NOTE:	There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.

# * * * * End of changes * * * *