3GPP TSG-WG SA2 Meeting # 158	S2-2308670

# August 21 - 25, 2023, Goteborg, Sweden										revision of S2-230xxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4765 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove EN from Annex N.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the specification has an Editor’s note as below in clause N.6: Editor's note: The exchange between the Localised Service Provider or the SNPN providing access to localized service and the HPLMN or the subscribed SNPN acting as Credentials Holder is FFS. To enable a UE to select an SNPN providing access to Localised services it is specified that the “Credentials Holder controlled prioritized lists of preferred SNPNs and GINs” will contain validity conditions. There are 2 different ways the “Credentials Holder controlled prioritized lists of preferred SNPNs and GINs” information is received by the UDM and provided to the UE. - The information may be stored in “Steering of Roaming” field of the AM subscription data, which the UDM then retrieves from UDR and delivers to the UE during registration procedure. This is done when SNPN or HPLMN policy for the SOR-AF invocation is absent (see Annex C.5 of TS 23.122). The information in “Steering of Roaming” field of the AM subscription data is stored via O&M. External paramter provisioning cannot be used to update this data. - The SoR AF may provide the information using Nudm_ParameterProvision service as specified in Annex C.5 and C.6 of TS 23.122. In this case the UDM does not store the information provided by the SoR AF in AM subscription data of the UE. Instead, it only secures the information and transparently delivers to the UE. So, currently we do not have any mechanisms to allow an external application to update the “Steering of Roaming” field of the AM subscription data. Also, the SoR AF is not in scope of SA2 and we do not specify how the SoR AF gets the information to generate the “Credentials Holder controlled prioritized lists of preferred SNPNs and GINs”. It is, therefore, proposed that the Localised Service Provider or the SNPN providing access to localized services can use O&M based mechanism to deliver the necessary information to the HPLMN or the subscribed SNPN acting as Credentials Holder and then the HPLMN or the CH can deliver the “Credentials Holder controlled prioritized lists of preferred SNPNs and GINs” to the UE using either of the above 2 mechanisms. It is proposed to remove the EN from Annex N.6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN from N.6 without any further action |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | N.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Begin of Changes * * *

# N.6	Configuration of Credentials Holder for determining SNPN selection information

To enable the HPLMN or the subscribed SNPN acting as Credentials Holder to generate and provision UEs with SNPN selection information for discovery and selection of SNPNs providing localised services, based on service agreement between the Localised Service Provider or the SNPN providing localized services and the HPLMN or the subscribed SNPN acting as Credentials Holder, the Localised Service Provider or the SNPN providing access to localized services can provide configuration information for SNPN selection to the HPLMN or the subscribed SNPN acting as Credentials Holder. The configuration information for SNPN selection may contain at least one of the following parameters:

a.	Identifier of the SNPN providing access to one or more localized services;

b.	Identification of each localized service;

c.	validity information for each localized service, e.g. the validity time information or/and location assistance information; and/or

d.	List of UE IDs (e.g. GPSIs or External Group ID) identifying the UEs subscribed with a localized service.

The operator of the HPLMN or the subscribed SNPN acting as Credentials Holder then may use the information received from the SNPN providing localised services and/or Localized Service Provider to create or update the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs for accessing Localized Services and provision the UEs using the Steering of Roaming procedure as defined in TS 23.122 [17].

* * * End of Changes * * *