SA WG2 Meeting #158	S2-2308698

# Gothenburg, Sweden, 2023-08-21 – 2023-08-25

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0168 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Home Routed-Session Breakout (HR-SBO) - resolving Overlapping UE IP EN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | @SA2#157 it was agreed to support both N6 tunneling and UE IP address translation on N6 to support overlapping UE IP addresses. The related EN was hovewer not removed and some contradictory text remain in 23.548. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the EN and revise ambigous text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HR-SBO is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.2, 6.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

#### 6.7.2.2	PDU Session establishment for supporting HR-SBO in VPLMN

Figure 6.7.2.2-1: Procedure for PDU Session establishment supporting HR-SBO in VPLMN

1.	During the Registration procedure, the AMF receives the HR-SBO allowed indication per DNN/S-NSNAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

2.	During the PDU Session Establishment procedure for Home-routed roaming as in clause 4.3.2.2.2 of TS 23.502 [3], if the AMF had received in SMF Selection Subscription data from UDM the HR-SBO allowed indication for the DNN/S-NSSAI in the step 1, the AMF selects a V-SMF supporting HR-SBO and sends an HR-SBO allowed indication to the V-SMF in the step 2 and the step3a of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

If the V-SMF supporting the HR-SBO receives the HR-SBO allowed indication from AMF, the V-SMF may:

-	select UL CL/BP UPF and L-PSA UPF based on UE location information and this indication in the step 4 of the Figure 4.3.2.2.2-1 of TS 23.502 [3].

NOTE 1:	The UL CL/BP UPF and L-PSA UPF can be co-located in the single V-UPF.

-	The PSA UPF selected for the UE DNS traffic towards the V-EASDF is instructed by the V-SMF to change the UE IP address to a unique IP address towards the V-EASDF.

NOTE 2: 	The changed IP address does not need to be a public IP address as the traffic is targeting the V-EASDF within the VPLMN domain.

-	select a V-EASDF;

-	obtain the V-EASDF IP address based on local configuration, or invoke Neasdf_DNSContext_Create Request including the DNN, S-NSSAI, HPLMN ID and the translated unique IP address that is configured in the PSA UPF for the DNS traffic of this UE, to obtain the V-EASDF IP address; and

-	send the request for the establishment of the PDU Session supporting HR-SBO in VPLMN and optionally send the V-EASDF IP address to the H-SMF in the Nsmf_PDUSession_Create Request in the step 6 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

The H-SMF authorizes the request for HR-SBO based on SM subscription data (i.e. HR-SBO authorization indication) in the step 7 of the procedure in the clause 4.3.2.2.2-1 of TS 23.502 [3].

Once the HR-SBO is authorized, the H-SMF requests and retrieves the optional VPLMN Specific Offloading Policy from H-PCF. The H-SMF generates VPLMN Specific Offloading Information (i.e. IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN in VPLMN, and/or authorized Session AMBR for Offloading) based on the VPLMN Specific Offloading Policy.

If HR-SBO is authorized for the PDU session, the H-SMF provides in the Nsmf_PDUSession_Create Response in the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] with the following information:

-	optionally, VPLMN Specific Offloading Information that may include FQDN range, IP range, session AMBR for the local part of DN and charging policy. The VPLMN specific offloading policy may refer to either allowed or not allowed traffic for HR-SBO (the latter is being used when the HPLMN would like to ensure that certain traffic are not allowed for HR-SBO). The VPLMN specific offloading policy for the allowed and not allowed traffic should be mutually exclusive, i.e., either a list of allowed or a list of not allowed traffic descriptors should be sent, but not both. The VPLMN specific offloading policy can be configured in the H-SMF per EC application and tagged with Offload Identifiers, and which VPLMN specific offloading policy to be sent to V-SMF can be indicated by these Offload Identifiers received from UDM/H-PCF. Similarly, H-SMF also sends the Offload Identifiers as labels to the list of HR-SBO related information to be sent to V-SMF for each application. If the given V-SMF has already received the VPLMN specific offloading policy for certain Offload Identifiers, this could be indicated to the H-SMF in any subsequent request to another HR-PDU Session from the same V-SMF, and the H-SMF will in this case send only the Offload Identifier as a response, if the policies for the given Offload Identifier have not changed (if available in HPLMN based on the SLA between HPLMN and VPLMN);

-	the V-EASDF IP address (corresponding to clause 6.7.2.3) or DNS server IP address of HPLMN (corresponding to clause 6.7.2.5) as DNS server address to be sent to the UE via PCO; and

-	optionally, the DNS server address provided by HPLMN to be used for DNS requests related with traffic not to be subject to HR-SBO, including to configure V-EASDF corresponding to clause 6.7.2.3, or configure the UPF in VPLMN to perform IP replacement as described in clause 6.7.2.5;

NOTE 3:	In this Release, only public IP address can be used as the DNS server address provided by HPLMN.

-	optionally, the HPLMN address information (e.g. H-UPF IP address on N6) to be used by V-EASDF to build EDNS Client Subnet option for target FQDN of the DNS query which is not authorized for HR-SBO as described in clause 6.7.2.3;

-	the HR-SBO authorization result (i.e. whether HR-SBO request is authorized or not).

The H-SMF may indicate to the UE either that for the PDU Session the use of the EDC functionality is allowed or that for the PDU Session the use of the EDC functionality is required.

If the request for HR-SBO is not authorized and DNS context has been created, the V-SMF delete the DNS context from the selected V-EASDF, and the subsequent steps related to the EASDF in this procedure are skipped.

The detailed information of VPLMN Specific Offloading Policy is described in clause 6.4 of TS 23.503 [4].

NOTE 4:	The VPLMN Specific Offloading Policy can be prior configured in HPLMN based on the service level agreement between the VPLMN and HPLMN.

3.	The V-SMF configures the V-EASDF with the DNS handling rules using the received VPLMN Specific Offloading Information.

The V-SMF optionally configures the V-EASDF with the DNS server address provided by the HPLMN as default DNS server (corresponding to clause 6.7.2.3), after the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] if they are received from H-SMF in the step 2. If V-SMF has not received the DNS server address provided by HPLMN from H-SMF in step 2, a default DNS server may be configured to V-EASDF.

If HPLMN address information is received, the V-SMF may also configure the V-EASDF to build EDNS Client Subnet option based on this HPLMN address information for target FQDN of DNS query which is not authorized for HR-SBO.

If the V-SMF has interacted with the V-EASDF in step 2, then the V-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the V-EASDF.

The V-SMF configures the UL-CL UPF and PSA UPF selected in the step 2 to forward DNS messages to V-EASDF.

4A.	EAS Discovery procedure with V-EASDF is performed as described in clause 6.7.2.3.

4B.	EAS Discovery procedure with Local DNS Server/Resolver is performed as described in clause 6.7.2.4.

4C.	EAS discovery procedure with V-EASDF using IP replacement mechanism as described in clause 6.7.2.5.

#### 6.7.2.3	EAS Discovery Procedure with V-EASDF for HR-SBO

Figure 6.7.2.3-1: Procedure for EAS Discovery with V-EASDF for HR-SBO roaming scenario

1.	The DNS query sent by the UE reaches the V-EASDF via the UL-CL UPF and PSA UPF in VPLMN selected in step 2 of Figure 6.7.2.2-1.

NOTE 1:	The network needs to ensure that EASDF can disambiguate the DNS traffic of different UEs that would be allocated with same private UE IP address. This can be done by implementation and/or deployment specific means, e.g. tunnelling on N6 or source IP address mapping.

If the target FQDN of the DNS query is not part of the FQDN authorized by the H-SMF in step 2 of Figure 6.7.2.2-1, the following a) or b) may be performed:

a)	The V-EASDF proceeds to step 12 of Figure 6.2.3.2.2-1 where it sends the DNS query which may include the HPLMN address information as the EDNS Client Subnet option. The DNS query is sent to the DNS server address according to the DNS message handling rules provided by the V-SMF or to the default DNS server configured in the V-EASDF. Upon receiving the DNS response, the procedure proceeds immediately to step 5.

NOTE 2:	If HPLMN DNS or the default DNS server does not support the ECS option, it cannot be ensured that an AS close to H-UPF will be resolved.

b)	The UL CL/BP UPF sends the DNS request to the DNS server address provided by HPLMN via V-UPF (if exists) and H-UPF (through N9), by modifying the packet's destination IP address (corresponding to V-EASDF) to the DNS server address provided by HPLMN on UL-CL or H-UPF. For the corresponding DNS response received by H-UPF, the H-UPF or ULCL modifies the packets' destination IP address to that of the V-EASDF.

This assumes that the UL-CL is able to detect FQDN(s) in traffic sent to the IP address of the EASDF. It is thus incompatible with usage of DoT (DNS over TLS) or DoH to protect the DNS traffic exchanged between the UE and the PLMN.

The rest of the procedure assumes the target FQDN of the DNS query is part of the FQDN authorized by the H-SMF in step 2 of Figure 6.7.2.2-1.

2.	The step 8 to 15 of the procedure in the Figure 6.2.3.2.2-1 by replacing SMF and EASDF with V-SMF and V-EASDF respectively.

3.	The V-SMF selects UL CL/BP and local PSA in VPLMN based on the V-EASDF notification, EAS Deployment Information in the VPLMN and UE location. The V-SMF may perform insertion or change of UL CL/BP and local PSA in VPLMN .

The V-SMF configures the UL-CL/BP and local PSA for the traffic to be offloaded to the local part of DN based on the VPLMN Specific Offloading Information received from H-SMF.

In the case of UL-CL, the V-SMF configures the traffic detection rules and traffic routing rules on the UL-CL UPF based on the EAS Deployment Information and the EAS addresses included in VPLMN Specific Offloading Information.

If there is no other V-UPF between the selected UL CL/BP in this step and H-UPF, the V-SMF sets up user plane between this UL CL/BP and H-UPF via the interaction with H-SMF. Otherwise, the V-SMF sets up user plane between this ULCL/BP and the existing V-UPF.

The V-SMF sets up user plane between the selected UL CL/BP in this step and RAN (if no other V-UPF exists between RAN and this UL CL/BP) or the V-UPF (if exists between this UL CL/BP and RAN).

NOTE 3:	If the selected UL-CL/BP and local PSA in this step is the UL-CL/BP and PSA selected by V-SMF in the step 2 of Figure 6.7.2.2-1, the insertion of UL-CL/BP and local PSA in VPLMN will not be performed in this step.

NOTE 4:	In the home routed roaming scenario, the V-UPF selected during PDU Session establishment procedure can be deployed at a central area within VPLMN. In this case, the V-UPF is located in the user plane path between UL-CL/BP UPF in VPLMN and PSA-UPF in HPLMN. In some deployments, the UL-CL/BP UPF can be collocated with the V-UPF.

4.	The steps 17 to 18 of the procedure in clause 6.2.3.2.2 by replacing SMF and EASDF with V-SMF and V-EASDF respectively.

5.	V-EASDF sends the DNS Response to the UE.

## END of CHANGES