3GPP TSG-SA WG2 Meeting #158	S2-2308763

21 - 25 August, 2023, Gothenburg, Sweden   	(Rev of)

# Title:	(DRAFT) Reply LS on Preparation of NG-RAN resources to serve the alternative S-NSSAI

# Response to:	(S2-2308322/R3-233432) Response to Reply LS on Support of network slices which have area of service not matching deployed tracking areas

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:

Name:	Peter Hedman

Tel. Number:

E-mail Address:	peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

SA2 thanks RAN3 for the Response to Reply LS on Support of network slices which have area of service not matching deployed tracking areas. Note, title of this reply LS is changed to better reflect the content.

RAN3 asked for feedback on 3 approaches to prepare NG-RAN resources to serve the alternative S-NSSAI and SA2 would like to provide the following feedback:

Approach 1: The resources associated to the RRMPolicyRatio(s) (See TS28.541) of which the alternative S-NSSAI is member are adjusted (by OAM) in order to take into account the additional load potentially generated by remapping one or more slices into the alternative S-NSSAI.

Approach 2: By retrieving data from the OAM and making them available to the AMF, the AMF can determine the RRMPolicyRatio utilisation for the resources of potential Alternative S-NSSAIs. In this way, an appropriate Alternative S-NSSAIs can be selected, whose RRMPolicyRatio(s) has free resources. The AMF/SMF is expected to select different Alternative S-NSSAIs on a per UE basis to achieve load distribution.

Approach 3: Coordination is taken to ensure that both the original slice and the one or more potential remapped slices are member of the same RRMPolicyRatio(s) (See TS28.541). This ensures that the resources that would have been used at RAN for the original slice may be used instead for the alternative slice.

SA2 answer:

-	SA2 agrees that Approach 1 is a possible deployment option.

-	SA2 agrees that the entity deciding the Alternative S-NSSAI (OAM, NSSF or PCF) could be made aware of information enabling the entity deciding a load distribution between S-NSSAIs and to distribute the UEs between S-NSSAIs, when possible and applicable. However, that would require multiple S-NSSAIs to be configured for an S-NSSAI. It would also require logic  to enable OAM, NSSF and PCF to distribute the load between these. SA2 has not yet agreed any CR to support approach 2.

-	SA2 agrees that Approach 3 is a possible deployment option.

2. Actions:

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to take the above into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #159   October 9 - 13, 2023 	Xiamen, CN

TSG-SA2 Meeting #160   November 13 – 17, 2023 	Chicago, US