3GPP TSG-WG SA2 Meeting #158	S2-2308894

# Goteborg, Sweden, Aug 21 – 25, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4356 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the time window(s) for selecting the candidate UE(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The time window(s) for selecting the candidate UE(s) included in the Nnef_MemberUESelectionAssistance_Subscribe is intended to determine the candidate UE(s) in different time window that can fulfil the Member UE filtering criteria. However, only NWDAF can provide the statistics or pridictions analytics during a time window, so it should be clarified that the time window(s) for selecting the candidate UE(s) can only be used by the NEF when subscribing/requesting to NWDAF. The NEF can map the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations. In addition, it’s unclear what the recommended time window(s) means. Since the NEF derives recommended time window(s) based on the validity period(s) of the NWDAF analytics and the Member UE filtering criteria, so the recommended time window(s) should mean the time window(s) during which the list(s) of candidate UE(s) can fulfil the Member UE filtering criteria. Furthermore, it is possible that the list of candidate UE(s) which fulfil the Member UE filtering criteria are different in different recommended time windows. The description of a list of target UE(s) should be aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following NOTE is added: NOTE: The time window(s) for selecting the candidate UE(s) can only be used by the NEF when subscribing/requesting to NWDAF. The NEF maps the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations. Clarify the meaning of recommended time window(s). Update the “Target of Member UE Selection Assistance Reporting” to “a list of target UE(s)”. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The meaning and usage of time window(s) for selecting the candidate UE(s) are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.1, 5.2.6.32.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.15.13.1	Member UE selection general information flow

This clause describes the procedures that are generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistance to Member UE selection and update

1.	AF subscribes the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including a list of target UE(s), one or more UE member filtering criteria listed in the Table 4.15.13.2-1 and optionally, time window(s) for selecting the candidate UE(s). Subsequently, the AF may update the filtering criteria of the subscription by invoking Nnef_UEMemberSelectionAssistance_subscribe and providing a Subscription Correlation ID.

NOTE:	The time window(s) for selecting the candidate UE(s) can only be used by the NEF when subscribing/requesting to NWDAF. The NEF maps the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations.

2a.	[OPTIONAL] NEF verifies the authorization of the AF Request and identifies which information needs to be collected for each UE in the list of target UE(s) and executes the corresponding service operation based on the Member UE filtering criteria provided by the AF, e.g. events, analytics and/or notifications.

2b.	[OPTIONAL] The NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF uses the list of target UE(s) received in step 1 for the Member UE update using the updated filtering criteria.

3.	NEF interacts with different 5GC network functions to collect the required information for each UE in the list of target UE(s). The set of interactions between NEF and among 5GC NFs are dependent on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list(s) of candidate UE(s) which fulfil the Member UE filtering criteria in the AF request. Considering the validity period(s) of the analytics used for Member UE selection criteria, the NEF may derive recommended time window(s) during which the list(s) of candidate UE(s) can fulfil the Member UE filtering criteria, and the recommended time window(s) are a subset of the time window(s) for selecting the candidate UE(s) received from the AF. In different recommended time windows, the list of candidate UE(s) which fulfil the Member UE filtering criteria may be different.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and possibly additional information. See clause 5.2.6.32.4 for details.

# * * * * Next change * * * *

5.2.6.32.2	Nnef_MemberUESelectionAssistance_Subscribe service operation

Service operation name: Nnef_MemberUESelectionAssistance_Subscribe

Description: The NF consumer subscribes to receive the Member UE selection assistance information, or the subscription is updated if the same subscription is already defined in NEF.

Inputs, Required: Notification Target Address (+ Notification Correlation ID), at least one filtering criteria shown in Table 4.15.13.2-1.

Inputs, Conditional Required:

If no Subscription Correlation ID is provided in the subscription, a list of target UE(s) (GPSI or a list of GPSIs) is required.

Inputs, Optional: Application ID, Subscription Correlation ID (in the case of modification of the existing subscription), Expiry time, a set of Member UE filtering criteria shown in the Table 4.15.13.2-1, time window(s) for selecting the candidate UE(s), specific parameters depending on the Member UE filtering criteria, Periodicity (the periodicity of member update), maximum number of UEs (indicates the maximum number of candidate UE(s) that need to be fed back).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

# * * * * End of changes * * * *