3GPP TSG-WG SA2 Meeting #158	S2-2308897

# Goteborg, Sweden, Aug 21 – 25, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0889 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the analytics exposure in roaming case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the Consumer NF is aware that if the UE(s) indicated in Target of Analytics Reporting is/are roaming UE(s), the Consumer NF can send an Analytics request/subscribe to the RE-NWDAF. However, the RE-NWDAF does not know if the the UE(s) indicated in Target of Analytics Reporting is/are roaming UE(s), so it needs to be re-checked. To avoid repeated checks by RE-NWDAF, the Consumer NF could include a Roaming UE indication which can be used to identify the roaming UE(s) in the Target of Analytics Reporting when requesting/subscribing to the RE-NWDAF. If the Roaming UE indication is received, the RE-NWDAF can determine the roaming UE(s) based on the Roaming UE indication; otherwise, the RE-NWDAF needs to re-check if the UE(s) indicated in Target of Analytics Reporting is/are roaming UE(s). If the Consumer NF is unaware that if the UE(s) indicated in Target of Analytics Reporting includes roaming UE(s), the Target of Analytics Reporting may include both roaming UE(s) and non-roaming UE(s), so the description that the Target of Analytics Reporting is/are roaming in step 1 is not very proper. The H-NWDAF/V-NWDAF which does not support roaming exchange capability can not directly reject the Analytics request/subscribe before determining that the Target of Analytics Reporting includes roaming UE(s). Therefore, the corresponding description in step 1 is updated. Since whether the RE-NWDAF performs analytics aggregation for roaming scenario are up to implementation, when the RE-NWDAF does not perform analytics aggregation and sends the HPLMN analytics and VPLMN analytics to the consumer, the HPLMN ID/VPLMN ID could be included in the Analytics response/notification to indicate if the analytics is for roaming UE(s). The scenario if the Consumer NF in VPLMN can be aware that the Target of Analytics Reporting includes roaming UE(s) should also be considered. The “roaming entry” in the figure is not aligned with the agreed term “roaming exhcange” in the last meeting. It should clarified in Nnwdaf_RoamingAnalytics_Subscribe and Nnwdaf_RoamingAnalytics_Request service operations that the Target of Analytics Reporting sent by the V-RE-NWDAF to the H-RE-NWDAF only contains the inbound roaming UE(s), and the Target of Analytics Reporting sent by the H-RE-NWDAF to the V-RE-NWDAF only contains the outbound roaming UE(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Roaming UE indication that can be included in the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation, and clarify that the RE-NWDAF can determine the roaming UE(s) based on the Roaming UE indication. Change “Target of Analytics Reporting is/are roaming” to “Target of Analytics Reporting includes roaming UE(s)”. Update the description in step 1, so that the H-NWDAF/V-NWDAF first determines that the Target of Analytics Reporting includes roaming UE(s) before the H-NWDAF/V-NWDAF may reject the Analytics request/subscribe. Clarify that the RE-NWDAF may include the HPLMN ID/VPLMN ID in the Analytics response/notification to indicate if the analytics is for roaming UE(s). Add corresponding description about if the Consumer NF in VPLMN can be aware that the Target of Analytics Reporting includes roaming UE(s) to clause 6.1.5.2. Change the “V-NWADF (roaming entry)” and “H-NWADF (roaming entry)” to “V-RE-NWDAF” and “H-RE-NWDAF” in the figures, respectively. Add the following NOTE to clause 7.7.2 and clause 7.7.5. NOTE 2: Target of Analytics Reporting sent by the V-RE-NWDAF to the H-RE-NWDAF only contains the inbound roaming UE(s), and the Target of Analytics Reporting sent by the H-RE-NWDAF to the V-RE-NWDAF only contains the outbound roaming UE(s). Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some descriptions in the procedures of analytics exposure in roaming case is not very reasonable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.2, 6.1.5.3, 6.1.5.4, 7.7.2, 7.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.5.2	Analytics Exposure from HPLMN to VPLMN

Figure 6.1.5.2-1 shows the procedure that the V-RE-NWDAF requests network analytics (i.e. slice load level analytics, NF load analytics, service experience analytics, etc.) in HPLMN from a H-RE-NWDAF, upon receiving an analytics information/subscription request from the NF (e.g. AMF) in VPLMN.

NF in VPLMN

V-RE-NWDAF

Other H-NWDAF(s)

1. Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsSubscription_Subscribe

5. V-RE-NWDAF collects data in VPLMN and derives analytics

H-RE-NWDAF

2. Nnwdaf_RoamingAnalytics_Request / Nnwdaf_RoamingAnalytics_Subscribe

3a. H-RE-NWDAF collects data in HPLMN and derives analytics

3b. Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsSubscription_Subscribe

3c. Nnwdaf_AnalyticsInfo_Request Response / Nnwdaf_AnalyticsSubscription_Notify

4. Nnwdaf_RoamingAnalytics_Request Response / Nnwdaf_RoamingAnalytics_Notify

6. Nnwdaf_AnalyticsInfo_Request Response / Nnwdaf_AnalyticsSubscription_Notify

Figure 6.1.5.2-1: Procedure for analytics exposure from HPLMN to VPLMN

1.	If the Consumer NF in VPLMN (e.g. AMF) is aware that the UE(s) indicated in Target of Analytics Reporting is/are inbound roaming user(s), the Consumer NF discovers a V-RE-NWDAF as described in clause 5.2 and sends an Analytics request/subscribe (Analytics ID, Analytics Filter Information, Target of Analytics Reporting) to V-RE-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe. V-RE-NWDAF determines that the UE(s) indicated in Target of Analytics Reporting is/are inbound roaming user(s) based on the Roaming UE indication (if available). For the inbound roaming user(s) indicated in the Target of Analytics Reporting, V-RE-NWDAF determines based on operator configuration and the requested analytics whether analytics or input data from the HPLMN are required, or the analytics can be derived locally. The subsequent steps only apply if analytics from the HPLMN are required. If input data from the HPLMN are required, the procedures in clause 6.2.11 apply.

If the Consumer NF is unaware if the Target of Analytics Reporting includes roaming UE(s) and sends the Analytics request/subscribe to a V-NWDAF which does not support roaming exchange capability, the V-NWDAF may determine that the Target of Analytics Reporting includes roaming UE(s), e.g. by inquiring the AMF(s) serving the UE(s) at the UDM with the Nudm_UECM service, then the V-NWDAF may either:

-	forward the Analytics request/subscribe to a V-RE-NWDAF after discovering a V-RE-NWDAF as described in clause 5.2. The V-NWDAF may include the HPLMN ID in the Analytics request/subscribe. The V-NWDAF may also include the Roaming UE indication to indicate the roaming UE(s) in the Target of Analytics Reporting; or

-	reject the Analytics request/subscribe with a proper cause value. And the following steps will not be performed.

NOTE 1:	It is possible that the Target of Analytics Reporting sent by the Consumer NF to the V-RE-NWDAF includes both inbound roaming user(s) and non-roaming user(s).

NOTE 2:	The V-NWDAF is not depicted in the flow.

2.	V-RE-NWDAF checks the roaming agreements related to analytics from HPLMN to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the request is rejected, the following steps are skipped.

V-RE-NWDAF discovers a H-RE-NWDAF as described in clause 5.2. V-RE-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Analytics Filter Information, Target of Analytics Reporting) to H-RE-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in VPLMN. The Target of Analytics Reporting sent by the V-RE-NWDAF to the H-RE-NWDAF only contains the inbound roaming user(s).

NOTE 3:	The inbound roaming user(s) are distinguished by the V-RE-NWDAF according to UE ID(s) (i.e. SUPI(s)).

3a.	H-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN, and user consent for analytics as defined in clause 6.2.9 if needed, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is rejected, the following steps are skipped.

If H-RE-NWDAF supports to generate the requested analytics, it collects data from the NF(s) and/or OAM in HPLMN and derives the requested analytics; otherwise steps 3b and 3c are executed.

3b-3c.	[Optional] If the H-RE-NWDAF does not support to generate the requested analytics, it may request/subscribe to other NWDAF(s) in the HPLMN (if available) for the analytics and get corresponding response/notification.

Editor's note:	The user consent check procedure and security aspects will align with SA WG3's conclusion.

4.	H-RE-NWDAF sends the HPLMN analytics information to the V-RE-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The H-RE-NWDAF may restrict the exposed analytics information based on HPLMN operator polices.

5.	[Optional] If the Consumer NF also indicates request or subscription of analytics information available in the VPLMN (e.g. via Target of Analytics Reporting) in step 1, V-RE-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the requested analytics. These steps can be executed in parallel with steps 3-4. The V-RE-NWDAF may perform analytics aggregation with the analytics information received from the H-RE-NWDAF and analytics information generated by itself, based on the analytics request or subscription.

6.	V-RE-NWDAF sends the HPLMN analytics information received in step 4, or the aggregated analytics information if step 5 are performed, to the Consumer NF in VPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. When sending the HPLMN analytics information to the Consumer NF in VPLMN, the V-RE-NWDAF may also include the HPLMN ID in the Analytics response/notification to indicate that the analytics is for inbound roaming user(s).

If the Analytics request/subscribe is forwarded to the V-RE-NWDAF by a V-NWDAF as described in step 1, the V-NWDAF forwards the analytics information received from the V-RE-NWDAF to the Consumer NF using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE 4:	The present document describes that the RE-NWDAF may perform analytics aggregation for roaming scenario, but whether and how the RE-NWDAF performs analytics aggregation for roaming scenario are up to implementation.

# * * * * Next change * * * *

6.1.5.3	Analytics Exposure from VPLMN to HPLMN

Figure 6.1.5.3-1 shows the procedure that the H-RE-NWDAF requests network analytics (i.e. service experience analytics, slice load level analytics, etc.) in VPLMN from the V-RE-NWDAF, upon receiving an analytics information/subscription request from the NF (e.g. PCF) in HPLMN.

NF in HPLMN

H-RE-NWDAF

Other V-NWDAF(s)

1. Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsSubscription_Subscribe

5. H-RE-NWDAF collects data in HPLMN and derives analytics

V-RE-NWDAF

2. Nnwdaf_RoamingAnalytics_Request / Nnwdaf_RoamingAnalytics_Subscribe

3a. V-RE-NWDAF collects data in VPLMN and derives analytics

3b. Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsSubscription_Subscribe

3c. Nnwdaf_AnalyticsInfo_Request Response / Nnwdaf_AnalyticsSubscription_Notify

4. Nnwdaf_RoamingAnalytics_Request Response / Nnwdaf_RoamingAnalytics_Notify

6. Nnwdaf_AnalyticsInfo_Request Response / Nnwdaf_AnalyticsSubscription_Notify

Figure 6.1.5.3-1: Procedure for analytics exposure from VPLMN to HPLMN

1.	If the Consumer NF in HPLMN (e.g. H-PCF) is aware that the UE(s) indicated in Target of Analytics Reporting is/are outbound roaming user(s), the Consumer NF discovers a H-RE-NWDAF as described in clause 5.2 and sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to H-RE-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe. H-RE-NWDAF determines that the UE(s) indicated in Target of Analytics Reporting is/are outbound roaming user(s) based on the Roaming UE indication (if available). For the outbound roaming user(s) indicated in the Target of Analytics Reporting, H-RE-NWDAF determines based on operator configuration and the requested analytics whether analytics or input data from the VPLMN are required, or the analytics can be derived locally. The subsequent steps only apply if analytics from the VPLMN are required. If input data from the VPLMN are required, the procedures in clause 6.2.10 apply.

If the Consumer NF is unaware if the Target of Analytics Reporting includes roaming UE(s) and sends the Analytics request/subscribe to a H-NWDAF which does not support roaming exchange capability, the H-NWDAF may determine that the Target of Analytics Reporting includes roaming UE(s), e.g. by inquiring the AMF(s) serving the UE(s) at the UDM with the Nudm_UECM service, then the H-NWDAF may either:

-	forward the Analytics request/subscribe to a H-RE-NWDAF after discovering a H-RE-NWDAF as described in clause 5.2. The H-NWDAF may include the VPLMN ID in the Analytics request/subscribe. The H-NWDAF may also include the Roaming UE indication to indicate the roaming UE(s) in the Target of Analytics Reporting; or

-	reject the Analytics request/subscribe with a proper cause value. And the following steps will not be performed.

NOTE 1:	It is possible that the Target of Analytics Reporting sent by the Consumer NF to the H-RE-NWDAF includes both outbound roaming user(s) and non-roaming user(s).

NOTE 2:	The H-NWDAF is not depicted in the flow.

2.	If PLMN ID of the VPLMN is not included in the Analytics request/subscribe, the H-NWDAF inquires it at the UDM for the UEs indicated as Target of Analytics Reporting. The H-RE-NWDAF checks user consent for analytics as defined in clause 6.2.9. H-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN to determine if the roaming analytics request/subscription can be accepted or must be rejected. If the request is rejected, the following steps are skipped.

Editor's note:	The user consent check procedure and security aspects will align with SA WG3's conclusion.

The H-RE-NWDAF discovers the V-RE-NWDAF as described in clause 5.2.

H-RE-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Analytics Filter Information, Target of Analytics Reporting, [NF ID(s)]) to V-RE-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in HPLMN. The Target of Analytics Reporting sent by the H-RE-NWDAF to the V-RE-NWDAF only contains the outbound roaming user(s). The H-RE-NWDAF may obtain NF ID(s) of the NF(s) serving the roaming UE(s) in the VPLMN, e.g. AMF ID(s), SMF ID(s), from the UDM and include the NF ID(s) in the analytics request/subscribe.

3a.	V-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is rejected, the following steps are skipped.

If V-RE-NWDAF supports to generate the requested analytics, it collects data from the NF(s) serving the roaming UE(s) and/or OAM in VPLMN and derives the analytics; otherwise steps 3b and 3c are executed. The NF(s) serving the roaming UE(s), e.g. AMF(s) or SMF(s), if indicated in step 2, can be used as data source.

3b-3c.	[Optional] If the V-RE-NWDAF does not support to generate the requested analytics, it may request/subscribe to other NWDAF(s) in the VPLMN (if available) for the analytics and get corresponding response/notification(s).

4.	V-RE-NWDAF sends the VPLMN analytics information to the H-RE-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The V-RE-NWDAF may restrict the exposed analytics information based on VPLMN operator polices.

5.	[Optional] If the Consumer NF also indicates request or subscription of analytics information available in the HPLMN (e.g. via Target of Analytics Reporting) in step 1, H-RE-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the analytics. These steps can be executed in parallel with steps 3-4. The H-RE-NWDAF may perform analytics aggregation with the analytics information received from the V-RE-NWDAF and analytics information generated by itself, based on the analytics request or subscription.

6.	H-RE-NWDAF sends the VPLMN analytics information received in step 4, or the aggregated analytics information if step 5 are performed, to the Consumer NF in HPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. When sending the VPLMN analytics information to the Consumer NF in HPLMN, the H-RE-NWDAF may also include the VPLMN ID in the Analytics response/notification to indicate that the analytics is for outbound roaming user(s).

If the Analytics request/subscribe is forwarded to the H-RE-NWDAF by a H-NWDAF as described in step 1, the H-NWDAF forwards the analytics information received from the H-RE-NWDAF to the Consumer NF using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE 3:	The present document describes that the RE-NWDAF may perform analytics aggregation for roaming scenario, but whether and how the RE-NWDAF performs analytics aggregation for roaming scenario are up to implementation.

# * * * * Next change * * * *

6.1.5.4	Contents of Analytics Exposure in roaming case

When requesting or subscribing to analytics involving one or more roaming UEs, the Consumer NF shall send the request or subscription to an RE-NWDAF belonging to the same PLMN as the Consumer NF. The Consumer NF may indicate the following parameters in the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation:

-	The parameters as defined in clause 6.1.3, with the following differences:

-	Parameters related to analytics aggregation, analytics transfer or ML model selection should not be included;

-	if Target of Analytics Reporting indicates specific UE(s) or a group of UEs and the Consumer NF is aware that the UE(s) indicated in Target of Analytics Reporting is/are roaming user(s), the UE(s) shall belong to the same HPLMN (if HPLMN analytics are subscribed/requested) or be served by the same VPLMN (if VPLMN analytics are subscribed/requested)

-	Roaming UE indication: identifies the roaming UE(s) in the Target of Analytics Reporting.

-	Additionally, the following parameters:

-	If the NWDAF service consumer in a VPLMN requests/subscribes to HPLMN analytics, in Analytics Filter Information:

-	[OPTIONAL] PLMN ID of the HPLMN;

NOTE 1:	If PLMN ID of the HPLMN is not provided by the NWDAF service consumer, the V-RE-NWDAF derives it from the SUPIs of UEs indicated as Target of Analytics Reporting.

-	[OPTIONAL] mapped S-NSSAI of the HPLMN.

-	[OPTIONAL] AOI (in the form of geographical area) in the HPLMN

-	If the NWDAF service consumer in a HPLMN requests/subscribes to VPLMN analytics, in Analytics Filter Information:

-	[OPTIONAL] PLMN ID of the VPLMN

-	[OPTIONAL] AOI (in the form of geographical area) in the VPLMN

NOTE 2:	If PLMN ID of the VPLMN is not provided by the NWDAF service consumer, the H-RE-NWDAF inquires it at the UDM for the UEs indicated as Target of Analytics Reporting.

NOTE 3:	In this release of the specification, only one PLMN ID (of HPLMN or VPLMN) is supported in the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation.

Based on the analytics request or subscription from the NWDAF service consumer in the HPLMN, and local configuration, the H-RE-NWDAF may indicate the following parameters in the Nnwdaf_RoamingAnalytics_Request or Nnwdaf_RoamingAnalytics_Subscribe service operation to the V-RE-NWDAF, for requesting/subscribing to analytics in the VPLMN:

-	Analytics ID;

-	PLMN ID of the HPLMN;

NOTE 4:	It is up to SA WG3 to decide if a special protection of the PLMN ID of the HPLMN is required.

-	Analytics Filter Information:

-	[OPTIONAL] HPLMN S-NSSAI;

NOTE 5:	V-NWDAF maps that S-NSSAI to an S-NSSAI of the VPLMN which will be used in the Analytics Filter Information.

-	other Analytics Filter Information (e.g. AOI in the form of geographical area in the VPLMN) as provided by NWDAF service consumer in the HPLMN, if applicable in the VPLMN.

-	[OPTIONAL] NF ID(s) of the NF(s) (e.g. AMF(s), SMF(s)) serving the roaming UE(s) in the VPLMN;

-	other parameters as provided by NWDAF service consumer in the HPLMN, if applicable in the VPLMN.

Based on the analytics request or subscription from the NWDAF service consumer in the VPLMN, and local configuration, the V-RE-NWDAF may indicate the following parameters in the Nnwdaf_RoamingAnalytics_Request or Nnwdaf_RoamingAnalytics_Subscribe service operation to the H-RE-NWDAF, for requesting/subscribing to analytics in the HPLMN:

-	Analytics ID;

-	PLMN ID of the VPLMN;

NOTE 6:	It is up to SA WG3 to decide if a special protection of the PLMN ID of the VPLMN is required.

-	Analytics Filter Information:

[OPTIONAL] HPLMN S-NSSAI;

NOTE 7:	If an S-NSSAI but no mapped S-NSSAI is provided by the NWDAF service consumer in the Analytics Filter Information, the V-NWDAF maps the S-NSSAI to the S-NSSAI of the HPLMN and provides that mapped S-NSSAI of the HPLM as the Analytics Filter Information.

-	other Analytics Filter Information (e.g. AOI in the form of geographical area in the HPLMN) as provided by NWDAF service consumer in the VPLMN, if applicable in the HPLMN.

-	other parameters as provided by NWDAF service consumer in the VPLMN, if applicable in the HPLMN.

The RE-NWDAF provides the following output information to the consumer RE-NWDAF of the Nnwdaf_RoamingAnalytics_Request or Nnwdaf_RoamingAnalytics_Notify service operation:

-	The output information as defined in clause 6.1.3, with the following differences:

-	Information related to analytics aggregation (i.e. analytics metadata information) should not be included.

NOTE 8:	The output of the analytics depends on the roaming agreements.

NOTE 9:	UE specific data and analytics exchange between HPLMN and VPLMN and the possible storage is agreed between the operators bilaterally via roaming agreements (e.g. SLA) and takes into consideration all relevant regulatory requirements.

# * * * * Next change * * * *

7.7.2	Nnwdaf_RoamingAnalytics_Subscribe service operation

Service operation name: Nnwdaf_RoamingAnalytics_Subscribe

Description: Subscribes to NWDAF analytics related to roaming UE(s).

Inputs, Required:

-	(Set of) Analytics ID(s).

-	PLMN ID of the consumer NWDAF.

-	Target of Analytics Reporting.

-	Notification Target Address (+ Notification Correlation ID).

-	Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

NOTE 2:	Target of Analytics Reporting sent by the V-RE-NWDAF to the H-RE-NWDAF only contains the inbound roaming UE(s), and the Target of Analytics Reporting sent by the H-RE-NWDAF to the V-RE-NWDAF only contains the outbound roaming UE(s).

Inputs, Optional:

-	Analytics Filter Information.

-	Time window for historical analytics.

-	Subscription Correlation ID (in the case of modification of the analytics subscription).

-	preferred level of accuracy of the analytics.

-	preferred level of accuracy per analytics subset.

-	Reporting Thresholds.

-	maximum number of objects requested (max).

-	preferred order of results, maximum number of SUPIs requested (SUPImax).

-	time when analytics information is needed.

-	Output strategy.

-	NF (or NF-Set) ID(s) of NFs in VPLMN serving the UE(s) in Target of Analytics Reporting.

NOTE 3:	Analytics Filter Information, Reporting Thresholds, maximum number of objects requested (max), maximum number of SUPIs requested (SUPImax) and time when analytics information is needed can be provided per individual Analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

# * * * * Next change * * * *

7.7.5	Nnwdaf_RoamingAnalytics_Request service operation

Service operation name: Nnwdaf_RoamingAnalytics_Request

Description: The consumer requests analytics related to roaming UE(s).

Inputs, Required:

-	(Set of) Analytics ID(s).

-	PLMN ID of the consumer NWDAF.

-	Target of Analytics Reporting.

-	Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

NOTE 2:	Target of Analytics Reporting sent by the V-RE-NWDAF to the H-RE-NWDAF only contains the inbound roaming UE(s), and the Target of Analytics Reporting sent by the H-RE-NWDAF to the V-RE-NWDAF only contains the outbound roaming UE(s).

Inputs, Optional:

-	Analytics Filter Information.

-	Time window for historical analytics.

-	preferred level of accuracy of the analytics.

-	preferred level of accuracy per analytics subset.

-	Reporting Thresholds.

-	maximum number of objects requested (max).

-	preferred order of results, maximum number of SUPIs requested (SUPImax).

-	time when analytics information is needed.

-	Output strategy.

-	NF (or NF-Set) ID(s) of NFs in VPLMN serving the UE(s) in Target of Analytics Reporting.

NOTE 3:	Analytics Filter Information, Reporting Thresholds, maximum number of objects requested (max), maximum number of SUPIs requested (SUPImax) and time when analytics information is needed can be provided per individual Analytics ID.

Outputs, Required: If the request is accepted, then set of the tuple (Analytics ID, Analytics specific parameters). When the request is not accepted, an error response.

Outputs, Optional: Timestamp of analytics generation, validity period, confidence, revised waiting time.

# * * * * End of changes * * * *