SA WG2 Meeting #S2-158	S2-2308941

21 - 25 August, 2023, Goteborg, Sweden	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0300 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC INACTIVE UE Mobility within the same RNA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the RAN2#120 and #121bis meeting, it has agreed, - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB - When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving. It means that RRC INACTIVE UE can obtain the PTM configuration via multicast MCCH of current cell. If no configuration found by RRC INACTIVE UE moved from other cell, the UE resumes RRC connection. So it proposed to remove the EN on the RRC_INACTIVE UE mobility within RNA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN on the RRC_INACTIVE UE mobility within RNA. Clarify the UE can keep RRC INACTIVE if it can obtain the PTM configuration or resume if it cannot obtain the PTM configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN on the RRC_INACTIVE UE mobility within RNA is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 7.2.3.8.2	Mobility of UE in RRC_INACTIVE state receiving MBS data within RNA

When a UE in RRC_INACTIVE state is receiving ongoing MBS session data and moves to a new cell within the RNA, the UE may obtain the multicast MBS session configuration as specified in the TS 38.413 [15] to receive the MBS session data. If the UE in RRC_INACTIVE state does not obtain valid multicast MBS session configuration, the UE resumes the connection in the same way as specified in clause 7.2.3.7.

END OF CHANGES