3GPP TSG-SA WG2 Meeting #158	S2-2309047

# Goteborg, Sweden, August 21 - 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4835 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Considerationon PDU Set handling during handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When we use PDU Set QoS parameter like PSDB, during handover if some part of PDU Set is deliverd in the source RAN then the remaining part of PDU Sets and buffered PDU sets should be forwarded into and served in the target RAN. Then the original PSDB would not be appropriate for the forwarded data so that some compensation is required e.g. via interaction b/w serving RAN and target RAN, which is more serious than PDU based handling, which is single PDU-based scheduling based on e.g. PDB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During handover, PSDB value for the forwarded PDU Sets should be compensated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDU Sets may be lost during handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

5.37.5.2	PDU Set Information and Identification

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

NOTE 1:	In addition to considering the PDU Set Importance within a QoS Flow, NG-RAN could also consider the relative PDU Set Importance across QoS Flows of the same Priority Level when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator.

NOTE 2:	The PDU Set Information can be different for different PDU Sets within a QoS Flow.

The SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the Protocol Description indicating the header, extension header (e.g. RTP/SRTP) and payload type (e.g. H.264) used by the service data flow. The Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1.

PSA UPF can identify the PDU Set Information using the Protocol Description and the received RTP/SRTP headers or using implementation specific means. The details of the RTP/SRTP headers, header extensions and/or payloads used to identify PDU Set Information are defined in TS 26.522 [179].

For each DL PDU received on N6 for which PDU Set based QoS handling is indicated from the SMF, the PSA UPF applies the rules for PDU Set identification and provides PDU Set Information which is available to the RAN in the GTP-U header.

During handover if a part of a PDU Set or PDU Set(s) in source NG-RAN should be forwarded directly or indirectly to a target NG-RAN, they should be handled by the target NG-RAN in compensated manner.

NOTE yy:	Whether or which additional information are delivered to target NG-RAN for compensated PDU Set based QoS parameter value(e.g. PSDB) is decided by RAN WGs.

********** End of Changes **********