3GPP TSG-SA WG2 Meeting #158	S2-2309056

# August 21 – 25, 2023, Goteborg, Sweden							 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0423 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to MO and MT LR for SL positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Previous meetings have captured 5GC-MT-LR and 5GC-MO-LR Procedure for SL positioning in clause 6.20.2 and 6.20.5 to obtain the Target UE’s absolut location. It needs the LMF to determine of using SL positioning on top of the current 5GC-MT-LR and 5GC-MO-LR Procedure. To support these kind of enhancements, it means the following preconditions should be satisfied first: - The selected LMF shall support SL positioning function - LMF should be aware of the Target UE’s SL positioning capabilities to determine to use the SL-MT-positioning. However it seems that the above preconditions are missing to be capatured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the 5GC-MT-LR and 5GC-MO-LR Procedure for SL positioning, it is proposed to point out that: - the AMF selects the LMF that supports the SL positioning function if the Target UE has indicated that supports SL positioning. - The AMF includes the UE’s SL positioning capabilities for LMF to determine the subsequent procedures of using SL positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC-MT-LR and 5GC-MO-LR Procedure for SL positioning can not be performed due to the select LMF without SL positioning capability and LMF has no knowledge of whether UE supports SL positioning or not to initiate the SL-MT-LR procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.20.2, 6.20.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 6.20.2	5GC-MO-LR Procedure using SL positioning

This procedure is used to estimate the location of a UE based on the location of one or more Located UEs and the distance and/or direction between the UE and the Located UE(s). 5GC-MO-LR Procedure as defined in clause 6.2 applies with the following differences:

-	In step3, the AMF shall select the LMF that supports SL positioning if the UE has the authorized SL positioning capability.

-	In Step4, the AMF includes the UE’s SL positioning capabilities for LMF to determine the subsequent procedures of using SL positioning.

-	Step 5 is replaced by step 10-19 of clause 6.20.1 with the following adaptations:

-	UE1 is the target UE, and UE2/.../UEn is the Located UE.

-	Step 14: LMF determines whether to obtain location of the UE with the assistance of one or more Located UE (s). LMF triggers the UE to perform Ranging/Sidelink Positioning with the Located UE.

* * * * Start of 1st Change * * * *

6.20.5	5GC-MT-LR Procedure using SL positioning

The procedure is used to estimate the location of a UE by using the location of one or more Located UEs and the distance and/or direction between the UE and the Located UE(s).

Procedure of Figure 6.1.2-1 can be reused, with following adaptations:

-	In step10 of Figure 6.1.2-1, the AMF selects the LMF that supports SL positioning if the UE has the authorized SL positioning capability.

-	In setp11 of Figure 6.1.2-1, the AMF includes the UE’s SL positioning capabilities for LMF to determine the subsequent procedures of using SL positioning.

-	Step 12 of Figure 6.1.2-1 replaced by the step 10-16 of Figure 6.20.3-1 with the following adaptations:

-	UE1 is the target UE, and UEs 2 to n are Located UEs.

-	In step 10, the types of required location results is absolute solution, and the other UEs 2 to n are the candidate Located UE(s) if included. SL-MT-LR request also includes the indication of Target UE/LMF selecting Located UE.

-	In step 14, if UE1 receives the indication of LMF selecting Located UE in step 11, step 14 includes the obtained information of all the discovered Located UEs. If UE1 receives the indication of Target UE selecting Located UE in SL-MT-LR request, UE1 performs the Located UE selection, and SL-MT-LR response includes the selected Located UEs ID.

* * * * End of Change * * * *