3GPP TSG-WG SA2 Meeting #158	S2-2309163

# Goteborg, Sweden, August 21 – 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0907 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to data and analytics exchange in roaming scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The following EN for Roaming architecture can be removed based on the reply LSs from GSMA S2-2306287 and S2-2306288: Editor's note: Data and analytics exchange between HPLMN and VPLMN require updates based on feedback from GSMA. 2. "roaming entry capability" should be "roaming exchange capability", and "Roaming Entry NWDAF" should be "Roaming Exchange NWDAF", as agreed in SA2#157 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the EN regarding GSMA feedback for Roaming architecture. - Change "roaming entry" to "roaming exchange". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary EN remains. Terminology not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 6.2.10, 6.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

4.3	Roaming architecture

Based on operator's policy and local regulations (e.g. privacy), data or analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN). In a PLMN, an NWDAF is used as entry point to exchange analytics and to collect input data for analytics with other PLMNs. The NWDAF with roaming exchange capability is called Roaming Exchange NWDAF (RE-NWDAF).

Figure 4.3-1: Roaming Architecture to exchange Input Data or Data Analytics between V-PLMN and H-PLMN

Using the architecture shown in Figure 4.3-1:

-	For outbound roaming users, the NF consumer in the HPLMN can retrieve analytics related to the VPLMN via the H-RE-NWDAF in HPLMN and V-RE-NWDAF in VPLMN.

NOTE 1:	The analytics from the VPLMN may be generated by the V-RE-NWDAF in the VPLMN or by other NWDAFs in the VPLMN.

-	For outbound roaming users, the H-RE-NWDAF in HPLMN can collect data from the VPLMN via V-RE-NWDAF in VPLMN.

-	For inbound roaming users, the NF consumer in the VPLMN can retrieve analytics from the HPLMN via V-RE-NWDAF in VPLMN and H-RE-NWDAF in HPLMN.

NOTE 2:	The analytics from the HPLMN may be generated by H-RE-NWDAF in the HPLMN or other NWDAFs in the HPLMN.

-	For inbound roaming users, the V-RE-NWDAF can collect data in the HPLMN from the H-RE-NWDAF.

NOTE 3:	Both local breakout and home routed roaming architecture support the data or analytics exchanging between PLMNs.

NOTE 4:	Interactions between RE-NWDAFs of different PLMNs may be via SEPPs, which are not depicted in the architecture for the sake of clarify.

* * * Next Change * * * *

6.2.10	Data collection by H-RE-NWDAF from V-RE-NWDAF for outbound roaming users

This procedure may be used by the RE-NWDAF in the HPLMN as service consumer to subscribe/unsubscribe to notifications about input data from the VPLMN for outbound roaming users (from the HPLMN perspective). H-RE-NWDAF and V-RE-NWDAF in the procedure are NWDAFs with roaming exchange capability.

Figure 6.2.10-1: data collection by H-RE-NWDAF from V-RE-NWDAF for outbound roaming users

1.	For subscription to collected data related to the UE(s), the H-RE-NWDAF checks the user consent of related users.

NOTE:	How the user consent is checked needs to be aligned with SA WG3.

2.	The H-RE.NWDAF of HPLMN discovers V-RE.NWDAF of VPLMN that supports the Nwdaf_RoamingData service using the NRF as specified in Clause 5.2.

NOTE:	The access to the Nnf_EventExposure services is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy.

3.	The H-RE-NWDAF subscribes/unsubscribes to notifications about input data by invoking the Nnwdaf_RoamingData_Subscribe / Nnwdaf_RoamingData_Unsubscribe service operation. It optionally may indicate the IDs of AMFs and for local breakout also SMFs in the VPLMN handling related UEs, as obtained from the UDM.

4.	RE-V-NWDAF checks if the HPLMN is authorised to request the input data based on VPLMN operator polices (that may depend on the HPLMN and may indicate permissible or restricted input data and related parameters).

5.	The V-RE-NWDAF triggers new data collection from NF(s ) (as indicated via the AMF ID(s) or SMF ID(s)) if needed and monitors the requested input data using procedures as described in clauses 6.2.1 to 6.2.8.

6.	The V-RE-NWDAF may restrict the exposed input data based on VPLMN operator polices (that may depend on the HPLMN) and may store them for auditing.

7.	The V-RE-NWDAF notifies input data to the H-RE-NWDAF.

6.2.11	Data collection by V-RE-NWDAF from H-RE-NWDAF for inbound roaming users

This procedure may be used by an RE-NWDAF in the VPLMN to subscribe/unsubscribe to notifications about input data from the HPLMN for inbound roaming users (from the VPLMN perspective). H-RE-NWDAF and V-RE-NWDAF in the procedure are NWDAFs with roaming exchange capability.

Figure 6.2.11-1: Data Collection by V-NWDAF from H-NWDAF for inbound roaming users

1.	The V-RE-NWDAF of VPLMN discovers H-RE-NWDAF of HPLMN that supports the Nnwdaf_RoamingData service using the NRF as specified in clause 5.2.

NOTE:	The access to the Nnf_EventExposure services is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

2.	The V-RE-NWDAF subscribes/unsubscribes to input data information by invoking Nnwdaf_RoamingData_Subscribe / Nnwdaf_RoamingData_Unsubscribe service operation.

3.	H-RE-NWDAF checks if the VPLMN is authorised to subscribe to the indicated input data based on HPLMN operator polices (that may depend on the VPLMN and may indicate permissible or restricted input data and related parameters) and user consent of related users.

NOTE:	User consent check needs to be aligned with SA WG3.

4.	The H-RE-NWDAF triggers new data collection if needed and monitors the requested input data, using procedures as described in clauses 6.2.1 to 6.2.8.

5.	The H-RE-NWDAF may restrict the exposed input data based on HPLMN operator polices (that may depend on the VPLMN) and may store them for auditing.

6.	The H-RE-NWDAF notifies input data to the V-RE-NWDAF.

* * * End of Changes * * * *