3GPP TSG-WG SA2 Meeting #158	S2-2309182

Goteborg, Sweden, Aug 21 – 25, 2023	(revision of S2-230xxxx)

# Title:	[DRAFT] LS Reply on issues on QoS monitoring for satellite backhaul

# Response to:	LS (S2-2308298/C3-232469) on issues on QoS monitoring for satellite backhaul

# Release:	Rel-18

# Work Item:	5GSATB

Source:	[Huawei to be] SA2

To:	CT3

Cc:

Contact Person:

Name:	Hui Ni

Tel. Number:

E-mail Address:	hui DOT ni AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2309183

1. Overall Description:

SA2 thanks the LS IN (S2-2308298/C3-232469) from CT3. SA2 replies the questions as below:

Question 1:	Please clarify whether the PCF may determine to request QoS monitoring to get the report(s) for the packet delay based on the request received from the AF or based on local policy/configuration at the PCF (e.g., on the satellite backhaul category reported from the SMF)?

Answer:	SA2 agreed that PCF may determine to request QoS monitoring based on either 1) request received from the AF (as described in clause 6.1.3.21 and 6.1.3.22 of TS 23.503), or 2) local policy/configuration at the PCF.

SA2 agreed S2-2309183 as attached to clarify the above PCF behaviours.

Question 2:	If the PCF triggers packet delay reporting for dynamic satellite backhaul categories based on local policy/configuration, does the report influence any policy decisions at the PCF?

Answer:	PCF may decide to update policy considering QoS monitoring report, which is up to PCF decision.

2. Actions:

To CT3 group.

ACTION: 	SA2 kindly asks CT3 group to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #159		October 09 – 13, 2023			Xiamen, CN

TSG-SA2 Meeting #160		November 13 – 17, 2023			Chicago, USA