SA WG2 Meeting #158		S2-2309213

August 21-25, 2023, Gothenburg, Sweden

Source:	InterDigital Inc, Samsung

Title:	Unavailability period support with non-3GPP Access

Document for:	Discussion

Agenda Item:	9.37

Work Item / Release:	SUECR / Rel-18

Abstract of the contribution: This paper discusses how to support the Unavailability Period feature over non-3GPP access.

# 1.	 Discussion

In S2-2308290 / C1-233533, CT1 asked SA2 “CT1 requests clarification on whether SUECR feature is applicable on non-3GPP access, i.e., can UE indicate unavailability period duration over non-3GPP access?”.

Observation 1: The Unavailability Period feature allows the UE and network to store its context while an event that makes the UE unavailable takes place. When the event is over, the UE and network do not need to rebuild the context.

Observation 2: It is likely that a UE that is using non-3GPP access may need to execute an event that makes the UE unavailable (e.g., restarting after a software updates). For example if a software upgrade is done, the whole of the UE will restart thus it will not be able to selectively become unavailable only for 3GPP access.

Observation 3: Use of the Unavailability Period feature relies on the network configuring “a Periodic Registration Update time longer than or equal to the combination of the Unavailability Period Duration and Start of Unavailability Period to avoid interfering with the UE dealing with the event that causes the unavailability” and to avoid losing the UE context (i.e., implicitly deregistering).

Observation 4: There is no Periodic Registration Update timer associated with non-3GPP access. However, the non-3GPP Deregistration timer does apply over non-3GPP access.  If the non-3GPP Deregistration timer expires while the UE is unavailable, the UE may be implicitly deregistered.

Proposal 1: It is proposed to support the Unavailability Period feature over non-3GPP access by specifying that the AMF may take the Unavailability Period Duration (if available) and Start of Unavailability Period into account when determining non-3GPP Deregistration timer value.

A corresponding TS 23.501 CR is submitted in S2-239214 and a corresponding LS Response is submitted in S2-239215.